Installation, Upgrade, and Release Notes
Windows NT 3.51 for Digital Alpha Systems

# Copyright and Trademark Information

Restricted Rights: Use, duplication, or disclosure by the U.S. Government is subject to restrictions as set forth in subparagraph (c) (1) (ii) of the Rights in Technical Data and Computer Software clause at DFARS 252.227-7013.

© Copyright Digital Equipment Corporation 1995

All rights reserved.

The information in this document is subject to change without notice and should not be construed as a commitment by Digital Equipment Corporation. Digital Equipment Corporation assumes no responsibility for any errors that may appear in this document.

The software described in this document is furnished under a license and may be used or copied only in accordance with the terms of such license.

No responsibility is assumed for the use or reliability of software on equipment that is not supplied by Digital or its affiliated companies.

The following are trademarks of Digital Equipment Corporation:

Alpha AXP, AlphaServer, AlphaStation, AXP, DEC, Digital, StorageWorks, and the DIGITAL logo.

CompuServe is a registered trademark of CompuServe Inc. Microsoft, Windows NT, Windows NT Server, and Windows NT Workstation are trademarks and Windows is a registered trademark of Microsoft Corporation. OSF/1 is a trademark of the Open Software Foundation Inc. UNIX is a registered trademark in the United States and other countries licensed exclusively through X/Open Company Ltd.

All other trademarks and registered trademarks are the property of their respective holders.

Revision/Update Information: This revised manual supersedes the Windows NT 3.51 Release Notes version 2.2

Date: November 1995

# Release Information

Microsoft Windows NT™ Server™ and Workstation™ version 3.51 for Digital™ AlphaServer™ and AlphaStation™ Systems

Version 3.0
July 2026

This document provides the information you need to successfully prepare your system and install Microsoft Windows NT Server or Workstation 3.51 on your Digital Alpha system. The instructions contained in these release notes take you step-by-step through the system setup process. Different parts of these instructions apply depending on whether you want to preserve an existing configuration or set up your system from scratch and perform a fresh installation of Windows NT 3.51.

These release notes cover all system models in the following lines:

§	DECpc AXP™

§	AlphaStation 200, 250, and 400

§	AlphaStation 500 and 600

§	AlphaServer 1000

§	AlphaServer 2000, 2100, and 2200 (referred to as 2x00 within these notes)

§	AlphaServer 2100A

Note that these instructions do not apply to systems with AlphaBIOS firmware.

This document also provides general information and workarounds for known issues related to Alpha systems and their peripheral devices when used in conjunction with Microsoft Windows NT 3.51. This information is located in the System Issues section of these release notes. Before performing the procedures contained in this document, be sure to read each group of steps thoroughly to be sure they apply to you. Please retain these instructions for future use. This document is intended as additional information over and above that presented in the Microsoft Windows NT documentation.

# How to Use These Release Notes

These release notes are intended to be read, and the steps performed, in sequential order.

# Document Conventions

| Convention | Description |
| --- | --- |
| Bold monospace text | Bold, monospace text indicates text that should be typed by the user exactly as shown. |
| Bold italic monospace text | Bold, monospace, italic text indicates variable information that is typed by the user. |
| Italic text | Italic text indicates emphasized words. |
| Monospace text | Monospace text indicates system output. |
| {bmct light.bmp} | A light bulb symbol indicates a useful tip or shortcut. |
| Text between lines | Text between lines indicates an important tip, note, or caution. |

# System Configuration Overview

There are several steps that must be performed in the correct order to prepare your system for installation of Microsoft Windows NT 3.51. The following procedure provides an overview of the configuration process. You can click on the links in each step to get more details about that step. As you will read in each of these sections, some of the steps may not apply to you; therefore, it is important that you read each section and any procedure in it before performing the procedures.

#### To set up your system:

1.	Gather the information and materials you will need.

2.	Read the System Issues section of this document to learn about any special considerations for your system model.

3.	Set the operating system type to Windows NT if you are moving to Windows NT from Digital UNIX (formerly DEC OSF/1™) or OpenVMS™.

4.	Update your system firmware if necessary.

5.	Set the default system configuration if your system has never been configured, or if you are moving to Windows NT from Digital UNIX or OpenVMS.

6.	Run the EISA configuration Utility (ECU) if your system has an EISA bus and you have added, removed, or moved any EISA cards in your system. The ECU must be also be run if you are moving to Windows NT from Digital UNIX or OpenVMS.

7.	Partition and format your hard disk for use with Windows NT if this has not already been done.

8.	Install a fresh copy of Windows NT 3.51, or upgrade from Windows NT 3.1 or 3.5 to Windows NT 3.51.

9.	Install additional video support if your system has a TGA video adapter.

Note	Some of the screen illustrations in these release notes might vary slightly from the screens you see on your system. This is because of slight differences between system models. These differences in no way reduce the accuracy of the instructions presented here.

# Preparing to Set Up Your System

Before setting up your system, gather the information and materials listed here.

### Information Needed

Contact the administrator of your local area network to obtain the network information you will need. This information will vary depending on the type of network hardware and software installed at your location.

Typically, the network information you need includes:

§	The networking protocols used at your site

§	The default networking protocol to select

§	The network domain to which you belong

§	Any special access or password information for your domain

§	The brand and model of network card in your system

§	Any special network card settings to be taken into account during installation of Windows NT

§	If you are using TCP/IP, your IP address, DNS search order, gateway address, and LMHOSTS information

In addition, you should know the brand and model of display adapter and SCSI adapter installed in your system.

### Materials Needed

Before beginning installation of Microsoft Windows NT, make sure that you have the following materials:

§	Microsoft Windows NT ARC firmware

| System Series | Minimum ARC Firmware Version Required |
| --- | --- |
| AlphaServer 2100A 5/xxx | 4.43 |
| AlphaServer 2100A 4/xxx | 4.43 |
| AlphaServer 2x00 5/xxx | 4.29 |
| AlphaServer 2x00 4/xxx | 3.5-31 |
| AlphaServer 1000 | 4.26 |
| AlphaStation 500 | 4.44 |
| AlphaStation 600 | 4.24 |
| AlphaStation 250 | 3.5-31 |
| AlphaStation 200, & 400 | 3.5-31 |
| Alpha AXP™ 150 | 3.54 |

§	For systems with an EISA bus, the ECU diskette.

| System Series | ECU Version Required |
| --- | --- |
| AlphaServer 2100A 5/xxx | 1.8 |
| AlphaServer 2100A 4/xxx | 1.8 |
| AlphaServer 2x00 5/xxx | 1.8 |
| AlphaServer 2x00 4/xxx | 1.8 |
| AlphaServer 1000 | 1.8 |
| AlphaStation 600 | 1.8 |
| AlphaStation 250 | 1.8 |
| AlphaStation 200 & 400 | 1.6 |
| DECpc AXP 150 | 1.6 |

§	Microsoft Windows NT Server or Workstation 3.51 CD-ROM.

§	A partitioned and formatted hard disk drive installed in your computer. If your drive has not yet been prepared, or if you wish to configure your hard drives from scratch, follow the Hard Disk Preparation instructions.

§	Hardware support diskette or vendor-supplied driver diskettes; for example, video drivers, SCSI drivers, and network adapter drivers.

# Obtaining Firmware Updates

Firmware updates can be obtained in several ways:

§	From CompuServe® or the Internet. These always have the very latest released firmware.

§	From Digital Customer Service, by calling 1-800-354-9000 in the United States, and asking for the latest firmware diskette for your system model.

§	The latest Digital Alpha System Firmware Console CD-ROM. This has the latest firmware as of the CD-ROM date.

If you are unsure about your particular upgrade requirements, call the Digital Equipment Corporation Technical Support line at 1-800-354-9000 for further assistance in the United States. For locations outside the United States, contact your local authorized Digital partner for assistance.

Note	Each system line has its own independent firmware numbering scheme. Even though two different system lines might have firmware with the same revision number, they are not the same. For example, firmware revision 4.43 from the AlphaServer 2100A 5/xxx line will not work on systems from the AlphaServer 2100A 4/xxx line. When you update your system’s firmware, make sure to obtain the firmware for your specific system line.

#### To obtain firmware from CompuServe:

1.	Log on to CompuServe and go to the DEC4WNT forum in the Library Section [4], Hardware Support.

2.	Download the README.TXT file. This file contains a complete list of the firmware revisions needed to run Microsoft Windows NT on Digital Alpha systems. This document also provides specific instructions on what files to download to obtain the latest firmware for your system.

3.	Once you have determined which files you need, download those files and follow the instructions included to create a firmware update diskette.

#### To obtain firmware via the Internet using FTP:

This procedure assumes you are using a command-line FTP utility.

1.	Start an FTP session.

2.	Type open gatekeeper.dec.com and press Enter.

3.	At the User prompt, type anonymous and press Enter.

4.	At the Password prompt, type your Internet e-mail name¾for example, joe@company.com. If you do not have an Internet e-mail name, just type your first and last name.

5.	At the FTP prompt, type cd /private/fwaxp and press Enter.

6.	Type get readme.txt and press Enter. This file contains a complete list of the firmware and ECU revisions needed to run Microsoft Windows NT 3.51 on Digital Alpha systems. This document also provides specific instructions on what files to download to obtain the latest firmware and ECU configuration files for your system.

7.	Once you have determined which files you need, return to the FTP session, type bin, and press Enter. This command makes it possible to receive binary files.

8.	Using the following format, type the command to receive a file, where filename is the file you want to receive: get filename. Then press Enter. The file is transferred to the directory from which you started the FTP session. Repeat this process for each file you want to receive.

9.	When you are done receiving the file(s) you need, type bye and press Enter. This will end the FTP session.

#### To obtain firmware via the Internet using a browser:

§	Connect to URL http://www.windowsnt.digital.com.

#### To obtain the Digital Alpha System Firmware Console CD-ROM

You can obtain Alpha system firmware updates on CD-ROM by calling 1-800-DIGITAL in the United States or your local authorized Digital partner, and ordering one of two part numbers.

§	To obtain just the latest firmware CD-ROM, order part number QZ–003AA–E8.

§	To obtain a 1- year subscription of quarterly CD-ROM updates, order part number QY–003AA–A8.

# General System Issues

The following system issues detail items to be aware of when configuring your system. Depending on the features of your system model, some of the items listed might not apply to you. If you have an AlphaServer 2100A system, also see the AlphaServer 2100A Specific System Issues section.

#### Digital TGA Graphics Adapter

If your system has a TGA graphics adapter installed, Windows NT Setup automatically installs the 2D graphics driver. The OpenGL 3D graphics accelerator libraries, however, are not installed as part of Windows NT setup. If you want full support for OpenGL 3D graphics, you must install the TGA driver after Microsoft Windows NT setup is completed. For more information, see Installing OpenGL 3D Support.

#### RAID Controller

If you have the Digital SWXCR RAID controller installed, you must use version 3.11 or greater of the RAID Configuration utility with Windows NT 3.51.

Disks connected to the StorageWorksÔ RAID Array 200 Subsystem (the RAID Array Subsystem includes the SWXCR EISA backplane RAID controller) must be spun up before they can be accessed. While running the ECU, verify that the controller is set to spin up four disks every 6 seconds. This is the default setting if the default configuration files for the SWXCR controller are being used.

Make sure to reset the computer after changing the RAID configuration. This reset is required to make your changes take effect.

There is an anomaly in Windows NT that can cause the system to crash if the SWXCR utilities and the Windows NT Disk Administrator program are both run at the same time. Do not run the SWXCR utilities and Disk Administrator utility concurrently.

#### Network Cards

Windows NT 3.51 includes an updated network adapter driver for Digital Equipment Corporation network adapters based on the DC21040 chip. Currently, this includes the DE425 and DE435 cards as well as built-in adapters. This new driver also supports many new Digital network cards, and has been significantly enhanced since the previous version.

The existing DC21040 driver is not automatically updated when upgrading to Windows NT 3.51 from Windows NT 3.5 or 3.1. If you are performing a fresh install of Windows NT 3.51, however, the latest driver is automatically installed onto your system. Refer to Updating the DECÔ DC21040 Adapter Driver for step-by-step instructions on correctly updating the driver.

#### EISA Bus Configuration

When updating the EISA configuration, make sure to use the Microsoft Windows NT ECU diskette. Using the ECU diskette for OpenVMS or Digital UNIX may result in an inoperable system. If you make a backup of the ECU diskette, ensure that the volume label is SYSTEMCFG.

The ECU does not automatically configure ISA cards. For example, if you add an Adaptec AHA154x to the bus, you will need to choose the Add a Board option within the ECU and select !ISA0000.CFG for the ISA slot. You can then manually configure the AHA154x, which will force the ECU to resolve ISA-EISA resource conflicts.

Alternatively, you can examine the resources used by the EISA cards from within the ECU under Step 3: View or edit details, and move those that conflict with the ISA cards.

If your computer has more than 24 logical or physical disks, the ECU may fail. If this occurs, remove the extra disks and run the ECU. Then, reinsert the disks.

If you run the ECU under version 4.38 of the ARC firmware, be sure to save your EISA configuration when exiting the ECU, even if you have not made any changes to your setup. If you do not save your configuration each time, it will become corrupted and the ECU will have to be run again. This is an anomaly that applies only to version 4.38 of the ARC firmware.

#### PCI Parity Checking

Your computer is shipped from the factory with PCI parity checking disabled. This setting accommodates third-party adapter cards in your computer that either do not fully comply with the PCI specification or do not generate PCI parity. It is important to note, however, that parity checking is used to ensure data integrity, and leaving parity checking off increases the risk of generating data errors.

#### To turn on parity checking:

1.	From the Boot menu, select Supplementary Menu.

2.	At the Supplementary menu, select Set Up the System.

3.	At the Setup menu, select Machine Specific Setup.

4.	Use the arrow keys to highlight PCI Parity Checking Enabled, then press Enter.

5.	When asked to confirm your choice, select Yes and press Enter.

6.	Press the Esc key three times to return to the Boot menu.

Note	If turning PCI parity checking on generates parity errors, disable PCI parity checking by following the same procedure, but select PCI Parity Checking Disabled instead.

#### Upgrading Firmware from CD-ROM

If you have version 4.24 or 4.38 of ARC firmware on your system, you cannot upgrade your firmware from CD-ROM. In these cases, copy the firmware upgrade files for your system to a diskette, and upgrade from there. This issue has been corrected in versions after 4.38.

# AlphaServer 2100A - Specific System Issues

In addition to the general system issues discussed in the General System Issues section, these system issues apply to all models of the AlphaServer 2100A.

#### S3-Based Video Adapters

Caution	If you have a PCI video adapter with an S3 controller chip, the following information can prevent serious system damage.

There is a problem in the S3 controller that causes it to respond to PCI bus transactions that it should not respond to. As a result, this can cause excessive current along the bus, causing physical damage to the PCI-to-EISA bridge chip.

If you have an S3-based PCI video adapter, this problem can be avoided by placing the PCI S3 adapter in the bottommost PCI slot.

#### SCSI Adapter

The KZPSA SCSI controller is only supported in the lower four PCI slots.

# Setting the Operating System Type

If your system has never been set up for Windows NT, or if you are moving from another operating system to Windows NT, it is necessary to set the operating system type to Windows NT. Doing this causes the ARC, rather than the SRM, firmware to load when the system is initialized.

#### To set the operating system type:

1.	Turn on the power to your computer. After a short wait, a blue screen appears.

2.	Press Enter. A >>> or P000>>> prompt appears. At this point, you have booted into the SRM Console firmware.

If the computer booted directly into the ARC Boot menu and you did not see the >>> prompt, the operating system type has already been properly set for use with Microsoft Windows NT. Skip to the next section, on updating the firmware.

3.	Type the following two commands. Press Enter after each command.

>>> set console graphics

>>> set os_type nt

4.	Turn off the power to your computer. Wait at least 5 seconds, then turn the power back on.

The system performs several diagnostics. You can ignore any error messages because the remainder of the configuration procedure corrects any errors appearing at this point.

# Updating the Firmware (All Systems Except DECpc AXP 150)

To learn about updating your firmware, read this section; however, if you have a DECpc AXP 150 system, view the following sections, which has information specific to your system.

For Windows NT 3.51 to operate properly on your Alpha system, the minimum required revision level of the ARC firmware must be installed.

If you have the required version, a firmware update is not required. However, updating to the latest version of the firmware is recommended to incorporate the latest firmware enhancements into your system.

Caution	The following set of instructions will correctly update both the SRM Console firmware if applicable, and the Windows NT ARC firmware in your computer. While it is possible to update the firmware images independently, this is not advised because the firmware images in the firmware update kit are a matched set. When performing a firmware update:

§	Update both the SRM and ARC firmware at the same time.

§	Make sure that the SRM and ARC firmware come from the same update kit.

#### To update the firmware:

1.	When the ARC Boot Menu appears, note the version at the top of the screen. Compare this version to the required version listed earlier and determine if an update is required. If no update is required, you can skip to the section Setting Default Environment Variables, or update the firmware anyway to take advantage of new features and bug fixes.

2.	If you are updating the firmware, select Supplementary Menu and press Enter

3.	Insert a diskette or CD-ROM containing the updated firmware for your system model into the diskette or CD-ROM drive.

4.	From the Supplementary menu, select Install New Firmware and press Enter. The update utility begins loading.

5.	After a few moments, the Firmware Update Utility screen is displayed. Depending on your system model, either a UPD> or Apu-> prompt is displayed. At the prompt, type update and press Enter.

6.	When asked to confirm the update, type y and press Enter.

As the SRM and ARC firmware images are verified, a “PASSED!” or “Programming Completed” message is displayed (the exact message depends on your system model). If you do not see the message for both firmware images, call the Digital Technical Support line at 1-800-354-9000 in the United States for further assistance. For locations outside the United States, contact your local authorized Digital partner.

Note	On the AlphaStation 200 and 400, if you get a message that a jumper may not be set correctly, check the following jumper settings and correct them as necessary.

§	AlphaStation 200 - J6, located on the right side of the memory simms, should have pins 1-2 jumpered.

§	AlphaStation 400 - J4, located on the daughter card, should have pins 1-2 jumpered.

7.	Once the update is complete, type exit at the prompt, then press Enter. Then reset your system.

When your system restarts, you are prompted to select a default language. For information on selecting a default language, see Selecting a Default Language.

# Updating the Firmware (DECpc AXP 150 Only, with Windows NT Already Installed)

If you have a DECpc AXP 150 system with Windows NT already installed, read this section to learn about updating your firmware; however, if Windows NT is not yet installed in your DECpc AXP 150 system, use the right Browse button to view the following section. To learn about firmware updates for other systems, use the left Browse button to view the previous section.

For Windows NT 3.51 to operate properly on your Alpha system, the minimum required revision level of the ARC firmware must be installed.

If you have the required version, a firmware update is not required. However, updating to the latest version of the firmware is recommended to incorporate the latest firmware enhancements into your system.

Caution	The following set of instructions will correctly update both the SRM Console firmware if applicable, and the Windows NT ARC firmware in your computer. While it is possible to update the firmware images independently, this is not advised because the firmware images in the firmware update kit are a matched set. When performing a firmware update:

§	Update both the SRM and ARC firmware at the same time.

§	Make sure that the SRM and ARC firmware come from the same update kit.

#### To update the firmware:

1.	When the ARC Boot Menu appears, note the version at the top of the screen. Compare this version to the minimum required revision and determine if an update is required. If no update is required, skip to the section Setting Default Environment Variables, or update the firmware anyway to take advantage of new features and bug fixes.

2.	Insert a diskette containing the DECpc AXP 150 firmware into the diskette drive. Verify that the write-protect tab is in the write-enabled position first.

Note	If you received the firmware on a CD-ROM, copy the firmware, along with the Save Environment (SAVEENV.EXE) and Restore Environment (RESTENV.EXE) utilities, to a diskette before continuing, and use the diskette when performing the firmware update.

3.	Select Run a Program and press Enter. You are asked for the program to run.

4.	Type a:\saveenv and press Enter to save environment variables.

If a “Pathname is not defined” message appears, repeat step 3 and type eisa()disk()fdisk()saveenv instead. Then press Enter.

5.	When asked whether you want to continue, type y. After your environment variables and boot selections have been saved to the diskette in drive A, an “Environment successfully saved message” is displayed.

If a version mismatch error appears, you can ignore the warning as the mismatch will be automatically corrected during the update process. If any other messages appear, do not continue with the update process and contact your local authorized Digital partner.

6.	Press any key to return to the ARC Boot menu.

7.	Select Supplementary Menu and press Enter.

8.	From the Supplementary menu, select Install New Firmware and press Enter. The FlashFile Update utility is started.

9.	When asked for confirmation, type y to continue with the update.

10.	When asked to select the location of the update file, verify that Floppy #0\jensfw.bin is selected and press Enter.

If the firmware update image is located in a different location, select Other Location and type the correct information.

11.	When asked for confirmation, type y twice to continue with the update. The update process begins.

If you see the message  “The update has succeeded,”  the update is successful. If prompted, press a key to continue.

If you do not see this message, call the Digital Technical Support line at 1-800-354-9000 in the United States for further assistance. For locations outside the United States, contact your local authorized Digital partner.

12.	Turn off the power to your computer. Wait at least 5 seconds, then turn the power back on.

If you see error messages indicating that the computer's NVRAM is incorrect or that the machine requires configuration, you can ignore them, as the following steps will correct any errors.

13.	At the ARC Boot menu, select Run a Program and press Enter. You are asked for the program to run.

14.	Type eisa()disk()fdisk()restenv and press Enter to restore environment variables.

15.	When asked whether you want to continue, type y.

After your environment variables and boot selections have been restored, a message appears stating that environment variables have been restored. If any other message appears, contact your Digital service representative.

16.	Turn off the power to your computer. Wait at least 5 seconds, then turn the power back on. Any error messages appearing at this point will be corrected by the remainder of the configuration process.

# Updating the Firmware (DECpc AXP 150 Only, with Windows NT Not Yet Installed)

For Windows NT 3.51 to operate properly on your Alpha system, the minimum required revision level of the ARC firmware must be installed.

If you have the required version, a firmware update is not required. However, updating to the latest version of the firmware is recommended to incorporate the latest firmware enhancements into your system.

Caution	The following set of instructions will correctly update both the SRM Console firmware if applicable, and the Windows NT ARC firmware in your computer. While it is possible to update the firmware images independently, this is not advised because the firmware images in the firmware update kit are a matched set. When performing a firmware update:

§	Update both the SRM and ARC firmware at the same time.

§	Make sure that the SRM and ARC firmware come from the same update kit.

#### To update the firmware:

1.	When the ARC Boot Menu appears, note the version at the top of the screen. Compare this version to the minimum required revision and determine if an update is required. If no update is required, skip to the next section, Setting Default Environment Variables, or update the firmware anyway to take advantage of new features and bug fixes.

2.	Select Supplementary Menu and press Enter.

3.	Insert a diskette containing the DECpc AXP 150 firmware into the diskette drive.

4.	From the Supplementary menu, select Install New Firmware and press Enter. The FlashFile Update utility is started.

5.	When asked for confirmation, type y to continue with the update.

6.	When asked to select the location of the update file, verify that Floppy #0\jensfw.bin is selected and press Enter.

If the firmware update image is located in a different location, select Other Location and type the correct information.

7.	When asked for confirmation, type y twice to continue with the update. The update process begins.

If you see the message “The update has succeeded,” the update is successful. If prompted, press a key to continue.

If you do not see this message, call the Digital Technical Support line at 1-800-354-9000 in the United States for further assistance. For locations outside the United States, contact your local authorized Digital partner.

8.	Turn off the power to your computer. Wait at least 5 seconds, then turn the power back on. Any error messages appearing at this point will be corrected by the remainder of the configuration process.

# Selecting a Default Language

The first time you power up after updating your workstation’s firmware, you are prompted to select a default language in which to display the firmware screens.

#### To select a default language:

1.	When asked to select a default language, select either English or Other Language as appropriate and press Enter.

If you selected English, your workstation continues to the Boot menu. You can skip to Setting Default Environment Variables.

If you selected Other Language, you are prompted for the location of the message and font files. Continue on to step 2.

2.	Insert the diskette or CD-ROM containing the firmware message and font files into the appropriate drive. If the files are located on your hard disk, skip to step 3.

3.	When asked to select the location of the message and font files, select either Floppy Disk, CD-ROM, or Hard Disk as appropriate, and press Enter.

The available languages are displayed on the screen.

4.	At the Language menu, select the language of your choice and press Enter.

5.	The screen clears and you are asked if you would like to save the language files to hard disk. Select Yes and press Enter.

Note	If your hard disk has not yet been formatted, attempting to save the language files to the hard disk results in an error. For information on setting up your hard disk, see Hard Disk Preparation.

6.	You are asked to choose a hard disk location for the language files. A default location will be selected. If you wish to use this location, press Enter.

Otherwise, select Alternate Hard Disk Location and press Enter. You are then asked to edit the partition and the directory location of the language files.

The language files are copied onto your hard disk, and the ARC Boot menu appears in the language you selected.

Note	You can change the default language at any time by selecting Set Up the System from the Boot menu, then selecting Set Default Language and following the prompts.

# Setting Default Environment Variables

These steps are necessary only if:

§	Your system arrived from the factory without first being configured.

§	You are switching from Digital UNIX or OpenVMS to Windows NT.

You must know your disk configuration before performing these steps. If you are unsure about your disk configuration, go to step 1 but select Display Hardware Configuration from the Supplementary menu instead of Set Up the System to display a summary of the hardware in your computer. Then return to the Supplementary menu and choose Set Up the System and follow the remaining steps.

#### To set default environment variables:

1.	When the ARC Boot menu appears, select Supplementary Menu and press Enter. At the Supplementary menu, select Set Up the System and press Enter.

2.	At the Setup menu, select Set Default Environment Variables and press Enter.

3.	When prompted to enter the system partition location, select SCSI Hard Disk and press Enter.

4.	When prompted to enter SCSI bus and SCSI ID numbers, enter the SCSI bus and ID for the location of the system partition. These values are used to set an environment variable, which Windows NT uses to determine the correct drive and partition to boot from.

If you are unsure about these values, accept the default values of 0. If these values are incorrect for your particular system (as is the case if your hard drives are on a secondary controller on SCSI bus 1), Windows NT Setup will prompt you to run the ARC Installation (ARCINST.EXE) utility. This utility will then provide valid values to choose from to set this environment variable correctly.

5.	When prompted to enter the partition number for the disk, type 2 and press Enter.

6.	Select Set Default Configuration and choose the floppy and keyboard settings corresponding to your system.

# Setting the EISA Configuration (Systems with an EISA Bus)

You must run the ECU if your system has an EISA bus and you have added, removed, or moved any EISA cards in your system. The ECU must be also be run if you are moving to Windows NT from Digital Unix or OpenVMS.

However, if you don’t have an EISA bus, you can skip this procedure and proceed with Hard Disk Preparation.

#### To set your EISA configuration:

1.	Insert the ECU diskette for Microsoft Windows NT into the disk drive.

2.	At the Boot menu, select Set Up the System and press Enter.

3.	At the Setup menu, select Run EISA Configuration Utility from Floppy and press Enter. After a few moments, the EISA Configuration Utility window appears. Press Enter to continue.

If any of the EISA boards in your computer have not yet been identified, a Caution message box appears. You can press Enter and ignore this message because your boards will be identified in the following steps.

4.	Press Enter to acknowledge any errors that are displayed. The Steps in Configuring Your Computer dialog box appears.

5.	Select Step 5: Save and Exit and press Enter. At the confirmation dialog box, select Save the Configuration and press Enter. When a message box appears notifying you that the configuration has been saved, press Enter. The computer is restarted with the updated information.

# Disk-Partitioning Information

The goal of the hard disk preparation instructions is to guide you through a fail-safe set of steps for creating two partitions of a particular size, in a particular order. By following these steps you will have the basic, recommended Windows NT partition arrangement.

Although you can create as many or as few partitions as you prefer, and create them in any order, there is an underlying reasoning for the recommended partitioning arrangement.

#### Size and Order of Partitions

By following the instructions in the Hard Disk Preparation section, you create two partitions. The first (large) partition is 6 megabytes less than the total size of the drive, and the second (small) partition consists of the remaining 6 megabytes. The large partition will hold the operating system and application/data files. The small partition, which must be formatted with the FAT format, holds the operating system loader and the few files necessary for your computer to boot. The small partition is called the boot partition.

This arrangement provides two benefits:

§	Windows NT and the ARC standard require that a boot partition be formatted with the FAT file system. However, the Windows NT file system provides advantages over FAT, such as additional security and more efficient use of disk space. By keeping the FAT boot partition as small as possible, the maximum amount of space is left available for use as an NTFS partition.

§	Most applications attempt to install themselves onto drive C by default. Although the program installation drive is usually configurable by the user, many people accept the default of C. By making the first partition large, drive C becomes the larger drive. This arrangement makes program installation easier and provides disk space where you are likely to need it.

# Hard Disk Preparation

Microsoft Windows NT cannot read or write to disks formatted for the Digital UNIX or OpenVMS operating system. If your hard disks are formatted for either Digital UNIX or OpenVMS, you must partition and format all of your hard disks to either the FAT file system, NT file system (NTFS), or a combination of both file systems before installing Windows NT.

If your hard disks are already partitioned and formatted with either the FAT or NT file system, you can skip this group of steps and begin installing the Microsoft Windows NT software.

Caution	If the hard disk you are going to partition currently contains any information you need, back that information up to another medium before proceeding.

Skip this group of steps if you want to maintain your current partitions.

#### To partition your hard disk:

1.	At the Boot menu, select Set Up the System and press Enter.

2.	From the Setup menu, choose Manage Boot Selections and delete all existing boot selections.

3.	Insert the Microsoft Windows NT CD-ROM into your computer’s CD-ROM drive.

4.	At the Boot menu, select Run a Program and press Enter. You are asked for the program to run.

5.	Type cd:\alpha\arcinst and press Enter. The ARC Installation Program screen appears.

6.	Select Configure Partitions and press Enter. The available disk-partitioning options appear.

7.	Select Delete Partition and press Enter. The list of available disks appears.

If your computer has only one hard disk, press Enter. If your computer has more than one hard disk, select the disk to be prepared and press Enter.

Depending on the previous disk configuration, you might be notified that no partitions exist, or you may see a list of one or more partitions. Follow the on-screen prompts to delete all partitions on the disk.

8.	Select Create System Partition and press Enter.

When the list of available disks appears, select the disk to be prepared and press Enter. You are prompted for the size of the partition.

9.	Type a number that is 6 megabytes less than the maximum size displayed, then press Enter For example, if the largest possible value is 500, type 494. A partition of that many megabytes is created.

When asked if you want to make this the system partition, type n and press Enter. The partition is formatted.

Once the formatting is completed, press Enter.

10.	Select Create System Partition and press Enter. This next series of steps creates the second partition.

When the list of available disks appears, select the same disk and press Enter. You are prompted for the size of the partition.

11.	Type the larger of the two values displayed (it will be close to 6), then press Enter. The second partition is created.

12.	After the partition is created, press Enter when prompted. The partition is formatted.

Once the formatting is completed, press Enter.

If you see a “System Partition has already been defined message,” it means that Windows NT has been previously installed. Type y to delete the existing system partition definition.

13.	When the partitioning menu appears, select Exit and press Enter. When the main Arcinst menu appears, select Exit again and press Enter. The ARC firmware Boot menu appears.

Note	Steps 13 through 18 address an anomaly in the updated version of the ARC Installation (ARCINST.EXE) utility supplied by Microsoft on all Windows NT 3.51 CDs. This updated version can only create system partitions. Therefore, in order to arrive at the Digital-recommended partition arrangement, it is necessary to first create two system partitions using ARCINST.EXE, and then edit system environment variables to remove the system flag from one of the partitions.

14.	Select Supplementary Menu and press Enter.

15.	Select Set Up the System and press Enter.

16.	From the Setup menu, select Edit Environment Variables and press Enter.

17.	Next to Name:, type systempartition and press Enter. The current system partition definition is displayed on the Value: line.

18.	Press the Home key to move the cursor to the beginning of the system partition definition, and press the Delete key to delete the text up to and including the semicolon (;). Once the text is deleted, press Enter.

Caution	Do not to delete the text after the semicolon. If you accidentally delete any of this text, press the Esc key twice, then return to step 15.

19.	Press the Esc key to return to the Setup menu. Then select Supplementary menu, save changes, and press Enter.

Your computer is now prepared for installation of Windows NT.

# Installing Windows NT

Follow the steps in the procedure below to install Windows NT for the first time or to upgrade from version 3.1 or 3.5 to version 3.51.

§	If you are upgrading Windows NT from Windows NT 3.1 or 3.5 to 3.51 and you have a network adapter based on the DEC DC21040 chip, you must manually upgrade the adapter driver. After upgrading Windows NT, see Updating the DEC DC21040 Adapter Driver for more information. Note that this additional procedure is unnecessary if you are installing Windows NT for the first time.

§	If your system has a TGA graphics adapter installed, Windows NT Setup automatically installs the 2D graphics driver. The OpenGL 3D graphics accelerator libraries, however, are not installed as part of Windows NT setup. If you want full support for OpenGL 3D graphics, you must install the TGA driver after Microsoft Windows NT setup is completed. For more information, see Installing OpenGL 3D Support.

During Microsoft Windows NT installation, you are prompted for the location to install the Windows NT files. If you prepared your hard disk by following the instructions in Hard Disk Preparation section, install into the larger of the two partitions you just created and, when prompted, format that partition with the NT file system. If you are installing Windows NT onto a disk that was previously set up, install Windows NT into the same directory as the previous installation.

Because Windows NT searches the network for domain and other network information during installation, verify that your network controller is properly terminated and that the network cable is attached to the network controller before beginning to install Windows NT.

#### To install Windows NT:

1.	Insert the Microsoft Windows NT CD-ROM into your CD-ROM drive.

2.	From the ARC Boot menu, select Supplementary Menu and press Enter. When the Supplementary menu appears, select Install Windows NT from CD-ROM and press Enter. Windows NT Setup begins.

3.	If a Hardware Support Disk (sometimes called an OEM Installation Disk) was included with your system, make sure to choose Custom Setup when prompted.

When the screen listing the recognized Alpha systems is displayed, choose Other and follow the prompts to select your system model.

4.	Follow the prompts to complete setup of Windows NT.

# Updating the DEC DC21040 Adapter Driver

If you have a network adapter based on the DEC DC21040 chip, the existing DC21040 driver is not automatically updated when upgrading to Windows NT 3.51. The updated driver must be installed manually after upgrading to Windows NT 3.51. Follow these steps to update Windows NT to version 3.51 and update the network driver.

#### To update the DEC DC21040 network adapter driver:

1.	Start Windows NT and double-click the Network icon in the Control Panel. The Network Settings dialog box is displayed.

2.	If you are running TCP/IP protocol, double-click TCP/IP Protocol in the Installed Network Software box. Write down all of your TCP/IP configuration information displayed. This information must be entered after the driver update. Choose Cancel when finished.

3.	In the Installed Adapter Cards box, select the card(s) for which the driver is to be updated, then choose Remove. Choose Yes in the confirmation message box. However, do not choose OK to exit the Network Settings dialog box at this point.

4.	Open a Command Prompt window and type the following commands:

cd /d %windir%\system32
			dir oemnad?.inf
			ren oemnad?.inf oemnad?.inx

5.	Return to the Network Settings dialog box and choose the Add Adapter button.

6.	Select your card(s) from the list. Follow the prompts to install the card.

7.	If you have TCP/IP running, enter the TCP/IP configuration information you noted earlier.

8.	Shut down and restart your system.

# Installing OpenGL 3D Support (Systems with TGA Graphics)

This information is relevant if your system uses TGA graphics.

After Windows NT is completely installed, you can install the OpenGL libraries. You must install the OpenGL libraries if you are using any software that takes advantage of the OpenGL 3D application programming interface (API) functions and you have a ZLXp-Ex video adapter.

#### To install the OpenGL libraries for the ZLXp-Ex video adapter:

1.	Insert the ZLXp-E (TGA) installation disk into the diskette drive.

2.	Within the Main program group of Windows NT, double-click the Control Panel icon.

3.	Double-click the Display icon.

4.	Choose the Change Display Type button.

5.	In the Display Type dialog box, choose the Change button.

6.	In the Select Device dialog box, choose the Other button.

7.	When prompted for the location of the driver files, verify that drive A is selected and choose OK. Digital ZLXp-E will appear in the list of devices.

8.	Choose the Install button to install the driver.

9.	When prompted, choose Yes to continue the installation.

10.	When prompted whether to replace the existing driver, choose New to use the version of the driver located on the diskette.

11.	Choose Continue, then choose OK twice.

12.	When prompted, choose the Restart Now button.

13.	After Windows NT restarts, the display resolution will be reset to 640 x 480. Use the Display applet within the Control Panel to change the resolution if you prefer.