Network Working Group                                     P. Prindeville

Request for Comments: 1048                             McGill University

February 1988

BOOTP Vendor Information Extensions

Status of this Memo

This memo proposes an addition to the Bootstrap Protocol (BOOTP).

Comments and suggestions for improvements are sought.  Distribution

of this memo is unlimited.

Introduction

As workstations and personal computers proliferate on the Internet,

the administrative complexity of maintaining a network is increased

by an order of magnitude.  The assignment of local network resources

to each client represents one such difficulty.  In most environments,

delegating such responsibility to the user is not plausible and,

indeed, the solution is to define the resources in uniform terms, and

to automate their assignment.

The basic Bootstrap Protocol [RFC-951] dealt with the issue of

assigning an internet address to a client, as well as a few other

resources.  The protocol included provisions for vendor-defined

resource information.

This memo defines a (potentially) vendor-independent interpretation

of this resource information.

Overview of BOOTP

While the Reverse Address Resolution (RARP) Protocol [RFC-903] may be

used to assign an IP address to a local network hardware address, it

provides only part of the functionality needed.  Though this protocol

can be used in conjunction with other supplemental protocols (the

Resource Location Protocol [RFC-887], the Domain Name System [RFC-

883]), a more integrated solution may be desirable.

Bootstrap Protocol (BOOTP) is a UDP/IP-based protocol that allows a

booting host to configure itself dynamically, and more significantly,

without user supervision.  It provides a means to assign a host its

IP address, a file from which to download a boot program from some

server, that server's address, and (if present) the address of an

Internet gateway.

Prindeville                                                     [Page 1]

RFC 1048                    BOOTP Extensions               February 1988

One obvious advantage of this procedure is the centralized management

of network addresses, which eliminates the need for per-host unique

configuration files.  In an environment with several hundred hosts,

maintaining local configuration information and operating system

versions specific to each host might otherwise become chaotic.  By

categorizing hosts into classes and maintaining configuration

information and boot programs for each class, the complexity of this

chore may be reduced in magnitude.

BOOTP Vendor Information Format

The full description of the BOOTP request/reply packet format may be

found in [RFC-951].  The rest of this document will concern itself

with the last field of the packet, a 64 octet area reserved for

vendor information, to be used in a hitherto unspecified fashion.  A

generalized use of this area for giving information useful to a wide

class of machines, operating systems, and configurations follows.  In

situations where a single BOOTP server is to be used among

heterogeneous clients in a single site, a generic class of data may

be used.

Vendor Information "Magic Cookie"

As suggested in [RFC-951], the first four bytes of this field have

been assigned to the magic cookie, which identifies the mode in

which the succeeding data is to be interpreted.  The value of the

magic cookie is the 4 octet dotted decimal 99.130.83.99 (or

hexadecimal number 63.82.53.63) in network byte order.

Format of Individual Fields

The vendor information field has been implemented as a free

format, with extendable tagged sub-fields.  These sub-fields are

length tagged (with exceptions; see below), allowing clients not

implementing certain types to correctly skip fields they cannot

interpret.  Lengths are exclusive of the tag and length octets;

all multi-byte quantities are in network byte-order.

Fixed Length Data

The fixed length data are comprised of two formats.  Those that

have no data consist of a single tag octet and are implicitly

of one-octet length, while those that contain data consist of

one tag octet, one length octet, and length octets of data.

Pad Field (Tag: 0, Data: None)

May be used to align subsequent fields to word boundaries

Prindeville                                                     [Page 2]

RFC 1048                    BOOTP Extensions               February 1988

required by the target machine (i.e., 32-bit quantities such

as IP addresses on 32-bit boundaries).

Subnet Mask Field (Tag: 1, Data: 4 subnet mask bytes)

Specifies the net and local subnet mask as per the standard

on subnetting [RFC-950].  For convenience, this field must

precede the GATEWAY field (below), if present.

Time Offset Field (Tag: 2, Data: 4 time offset bytes)

Specifies the time offset of the local subnet in seconds

from Coordinated Universal Time (UTC); signed 32-bit

integer.

End Field (Tag: 255, Data: None)

Specifies end of usable data in the vendor information area.

The rest of this field should be filled with PAD zero)

octets.

Variable Length Data

The variable length data has a single format; it consists of

one tag octet, one length octet, and length octets of data.

Gateway Field (Tag: 3, Data: N address bytes)

Specifies the IP addresses of N/4 gateways for this subnet.

If one of many gateways is preferred, that should be first.

Time Server Field (Tag: 4, Data: N address bytes)

Specifies the IP addresses of N/4 time servers [RFC-868].

IEN-116 Name Server Field (Tag: 5, Data: N address bytes)

Specifies the IP addresses of N/4 name servers [IEN-116].

Domain Name Server Field (Tag: 6, Data: N address bytes)

Specifies the IP addresses of N/4 domain name servers RFC-

883].

Log Server Field (Tag: 7, Data: N address bytes)

Specifies the IP addresses of N/4 MIT-LCS UDP log server

[LOGGING].

Prindeville                                                     [Page 3]

RFC 1048                    BOOTP Extensions               February 1988

Cookie/Quote Server Field (Tag: 8, Data: N address bytes)

Specifies the IP addresses of N/4 Quote of the Day servers

[RFC-865].

LPR Server Field (Tag: 9, Data: N address bytes)

Specifies the IP addresses of N/4 Berkeley 4BSD printer

servers [LPD].

Impress Server Field (Tag: 10, Data: N address bytes)

Specifies the IP addresses of N/4 Impress network image

servers [IMAGEN].

RLP Server Field (Tag: 11, Data: N address bytes)

Specifies the IP addresses of N/4 Resource Location Protocol

(RLP) servers [RFC-887].

Hostname (Tag: 12, Data: N bytes of hostname)

Specifies the name of the client. The name may or may not

domain qualified: this is a site-specific issue.

Reserved Fields (Tag: 128-254, Data: N bytes of undefined

content)

Specifies additional site-specific information, to be

interpreted on an implementation-specific basis.  This

should follow all data with the preceding generic tags 0-

127).

Extensions

Additional generic data fields may be registered by contacting:

Joyce K. Reynolds

USC - Information Sciences Institute

4676 Admiralty Way

Marina del Rey, California  90292-6695

or by E-mail as: JKREYNOLDS@ISI.EDU

(nic handle JKR1).

Implementation specific use of undefined generic types (those in the

range 12-127) may conflict with other implementations, and

registration is required.

Prindeville                                                     [Page 4]

RFC 1048                    BOOTP Extensions               February 1988

When selecting information to put into the vendor specific area, care

should be taken to not exceed the 64 byte length restriction.

Nonessential information (such as host name and quote of the day

server) may be excluded, which may later be located with a more

appropriate service protocol, such as RLP or the WKS resource-type of

the domain name system.  Indeed, even RLP servers may be discovered

using a broadcast request to locate a local RLP server.

Comparison to Alternative Approaches

Extending BOOTP to provide more configuration information than the

minimum required by boot PROMs may not be necessary.  Rather than

having each module in a host (e.g., the time module, the print

spooler, the domain name resolver) broadcast to the BOOTP server to

obtain the addresses of required servers, it would be better for each

of them to multicast directly to the particular server group of

interest, possibly using "expanding ring" multicasts.

The multicast approach has the following advantages over the BOOTP

approach:

- It eliminates dependency on a third party (the BOOTP server) that

may be temporarily unavailable or whose database may be incorrect or

incomplete.  Multicasting directly to the desired services will

locate those servers that are currently available, and only those.

- It reduces the administrative chore of keeping the (probably

replicated) BOOTP database up-to-date and consistent.  This is

especially important in an environment with a growing number of

services and an evolving population of servers.

- In some cases, it reduces the amount of packet traffic and/or the

delay required to get the desired information.  For example, the

current time can be obtained by a single multicast to a time server

group which evokes replies from those time servers that are

currently up.  The BOOTP approach would require a broadcast to the

BOOTP server, a reply from the BOOTP server, one or more unicasts to

time servers (perhaps waiting for long timeouts if the initially

chosen server(s) are down), and finally a reply from a server.

One apparent advantage of the proposed BOOTP extensions is that they

provide a uniform way to locate servers.  However, the multicast

approach could also be implemented in a consistent way across

multiple services.  The V System naming protocol is a good example of

this; character string pathnames are used to name any number of

resources (i.e., not just files) and a standard subroutine library

looks after multicasting to locate the resources, caching the

discovered locations, and detecting stale cache data.

Prindeville                                                     [Page 5]

RFC 1048                    BOOTP Extensions               February 1988

Another apparent advantage of the BOOTP approach is that it allows an

administrator to easily control which hosts use which servers.  The

multicast approach favors more distributed control over resource

allocation, where each server decides which hosts it will serve,

using whatever level of authentication is appropriate for the

particular service.  For example, time servers usually don't care who

they serve (i.e., administrative control via the BOOTP database is

unnecessary), whereas file servers usually require strong

authentication (i.e., administrative control via the BOOTP database

is insufficient).

The main drawback of the multicast approach, of course, is that IP

multicasting is not widely implemented, and there is a need to locate

existing services which do not understand IP multicasts.

The BOOTP approach may be most efficient in the case that all the

information needed by the client host is returned by a single BOOTP

reply and each program module simply reads the information it needs

from a local table filled in by the BOOTP reply.

Acknowledgments

I would like to thank the following persons for their helpful

comments and insights into this memo: Drew Perkins, of Carnagie

Mellon University, Bill Croft, of Stanford University, and co-author

of BOOTP, and Steve Deering, also of Stanford University, for

contributing the "Comparison to Alternative Approaches" section.

References

[RFC-951]   Croft, B., and J. Gilmore, "Bootstrap Protocol", Network

Information Center, SRI International, Menlo Park,

California, September 1985.

[RFC-903]   Finlayson, R., T. Mann, J. Mogul, and M. Theimer, "A

Reverse Address Resolution Protocol", Network Information

Center, SRI International, Menlo Park, California, June

1984.

[RFC-887]   Accetta, M., "Resource Location Protocol", Network

Information Center, SRI International, Menlo Park,

California, December 1983.

[RFC-883]   Mockapetris, P., "Domain Name - Implementation and

Specification", Network Information Center, SRI

International, Menlo Park, California, November 1983.

[RFC-950]   Mogul, J., "Internet Standard Subnetting Procedure",

Prindeville                                                     [Page 6]

RFC 1048                    BOOTP Extensions               February 1988

Network Information Center, SRI International, Menlo

Park, California, August 1985.

[RFC-868]   Postel, J., "Time Protocol", Network Information Center,

SRI International, Menlo Park, California, May 1983.

[IEN-116]   Postel, J., "Internet Name Server", Network Information

Center, SRI International, Menlo Park, California, August

1979.

[LOGGING]   Clark, D., Logging and Status Protocol", Massachusetts

Institute of Technology Laboratory for Computer Science,

Cambridge, Massachusetts, 1981.

[RFC-865]   Postel, J., "Quote of the Day Protocol", Network

Information Center, SRI International, Menlo Park,

California, May 1983.

[LPD]       Campbell, R., "4.2BSD Line Printer Spooler Manual", UNIX

Programmer's Manual, Vol II,  University of California at

Berkeley, Computer Science Division, July 1983.

[IMAGEN]    "Image Server XT Programmer's Guide", Imagen Corporation,

Santa Clara, California, August 1986.

Prindeville                                                     [Page 7]