Note: Any other hidden text in this Word document represents Digital’s working notes, is not part of the issued document and shall be ignored.  Digital recommends hidden text display is turned off when reading or printing this document.

Customer Specification Document

Prepared for BT by

Digital Equipment Co. Ltd.

Document Number: 14 Unnamed Copy

Prepared By:	Mike McCabe

Version:		1.0

Date:			2 October 1996

DOCUMENT CONTROL

| Document Name | Customer Specification Document |
| --- | --- |
| Project Document Number | BTTELE01 |
| Author(s) | Mike McCabe |
| Maintainer | Digital |
| Owner | Digital |
| Version | 1.0 |
| Issue Date | 2 October 1996 |
| Maintenance File Location | CHEFS cluster |
| Maintenance File Name | BTTELCSD.DOC |
| File Format | Word for Windows Version 6.0 |

VERSION HISTORY

| Version | Date | Description of Change & Reason for Change |
| --- | --- | --- |
| AX01 | 23 July 1996 | Internal Digital draft |
| AX02 | 29 July 1996 | Customer draft review |
| AX02a | 5 August | GeoTel review copy |
| AX03 | 9 August | 2nd Customer review |
| AX04 | 11 September | Incorporate BT comments |
| AX04B | 27 September | GeoTel/Digital Comments |
| 1.0 | 2 October | Main Release |

Document Roles and Responsibilities

The following people have been involved in the preparation of this document.

| Name (Company) | Role | Responsibility |
| --- | --- | --- |
| Mike McCabe (Digital) | Author | Project Manager |
| John Abram (Digital) | Contributor | Solution Architect |
| Graham Cobb (Digital) | Contributor | Solution Architect |
| Jerry Gechter (GeoTel) | Contributor | Technical Reviewer |
| Various (BT) | Reviewers | Various |

Copyright Digital Equipment Corporation and British Telecom October 1996.  All rights reserved.

This document is the copyright of, and contains proprietary information of, Digital Equipment Co. Ltd. and British Telecommunications plc, and as such shall not be reproduced or disclosed to a third party without the prior written consent of both Digital and BT.

The DIGITAL logo is a trademark of Digital Equipment Corporation.

The following are trademarks of Digital Equipment Corporation: DECbrouter, DEChub, DECNIS, RouteAbout.

Intelligent CallRouter, CallRouter, Post-Routing and Pre-Routing are registered trademarks and Longest Available Agent, Network Routing Operating System and Skills Based Routing are trademarks of  GeoTel Communications Corporation.

Microsoft, MS and MS-DOS, NT, Windows, Windows NT are all registered trademarks of Microsoft Corporation.

All other trademarks are acknowledged.

Table of Contents

PREFACE

1. INTRODUCTION

1.1 Overview

1.2 Objective of the CSD

1.3 CSD Scope

1.4 Methodology

2. DESIGN REQUIREMENTS

2.1 Deployment

2.2 Service Design

2.3 Physical Design

2.3.1 Central Sites

2.3.2 Operational Sites

2.3.3 Customer Sites

2.3.4 Demonstration Model

2.3.5 Integration, Verification, Validation and Test Model

2.3.6 Operational Support model

2.4 Pre-staging

2.5 Packaging

2.6 OSS Integration

2.7 Prompt and Collect

3. APPLICATION REQUIREMENTS

3.1 Capabilities

3.2 Documentation and Help Screens

3.3 Supported ACD/PBX Equipment

3.4 Customer Database Access

3.5 Calling Line Identity (CLI)

3.6 C7

3.7 First Party Clear

3.8 Labels

3.9 Year 2000

3.10 Dial Plan

3.11 Basic Translations

3.12 Bespoke Development

4. COMMUNICATIONS

4.1 Overall Architecture

4.1.1 Central Sites

4.1.2 Customer Sites

4.2 Roles and Responsibilities

4.3 Volumes

4.3.1 ICR WAN (I1)

4.3.2 ICR to PG and ICR to Customer AWS (I2)

4.3.3 ICR to BT AWS (I3)

4.3.4 PG to Remote ACD

4.3.5 C7 Links

4.4 Resilience

4.5 IP Routing

4.6 Backups

4.7 Security

4.8 Network Management

4.8.1 Surveillance

4.8.2 Customer Network

4.9 Remote Access

5. INTERFACES

5.1 C7

5.1.1 Signalling End Point

5.1.2 Multiple Point Code Operation

5.1.3 Message Transfer Part

5.1.4 Call Dropback Protocol

5.1.5 Additional Call Dropback Protocol Information

5.1.6 C7 Link Failure

5.2 ACD

5.3 PG Regulatory Interface

5.4 CTI

5.5 Physical Links

5.5.1 ICR WAN (I1)

5.5.2 ICR to PG and ICR to Customer AWS (I2)

5.5.3 ICR to BT AWS (I3)

5.5.4 IP Router Interfaces

5.5.5 PG to Remote ACD

5.5.6 C7 Links

5.5.7 Remote Access

5.5.8 Alarms

5.6 OSS

5.7 Administration Centres

5.8 MCI 800 Gateway

6. DIMENSIONING

6.1 Capacity

6.1.1 Central Systems

6.1.2 NIC Configuration

6.1.3 BT Operational Systems

6.1.4 Customer Systems

6.2 Dimensioning Model

6.3 Growth Strategy

6.3.1 Scalability

6.3.2 Critical Metrics

6.3.3 Tools

6.3.4 Reporting

6.3.5 Capacity Planning and Forecasting

6.4 Volumes

6.4.1 IP Routing Bandwidth

6.4.2 C7 Signalling Links

6.4.3 Disk Space

6.4.4 IP Address Space

6.4.5 CTN

6.5 Demonstration Model

6.6 Training Model

6.7 IVV&T Model

6.8 Support Model

6.9 Equipment Specification

6.9.1 Demonstration Model

6.9.2 IVV&T Model

6.9.3 Central Site Equipment

6.9.4 Customer Premise Equipment

6.9.5 BT Operations Support Equipment

6.9.6 Environmental Requirements

6.10 Additional Software

7. PERFORMANCE

7.1 Quality of Service

7.1.1 Call Handling Capacity

7.1.2 Call Failure Rates

7.1.3 Plant Engaged

7.1.4 Post-dial Delay

7.1.5 Overload Control

7.1.6 Basic Translations

7.2 Resilience

7.2.1 Mean Time Before Failure

7.2.2 Mean Time To Repair

7.3 Communications

7.4 Customer Premise Equipment

7.5 Impact to Customer ACD

7.6 Contracted Performance

7.6.1 Tools

7.6.2 Traffic Forecast

7.6.3 Critical Parameters

7.6.4 Reports

8. SECURITY

8.1 BT Security Requirements

8.2 Access Levels

8.2.1 BT Staff

8.2.2 Customers

8.3 Node Authentication

8.4 Domain Model

8.5 CLI

8.6 BT Central Site Access

8.7 AWS

8.7.1 BT AWS

8.7.2 CPE AWS

8.7.3 Customer Database Access

8.8 PG Management

8.9 Remote Access

8.10 Firewalls

8.11 Audit Trails

8.12 User Monitoring

8.13 Physical Security

8.14 Network Routes

9. ACCEPTANCE CRITERIA

9.1 Demonstration Model

9.2 C7 Compliance Testing

9.3 C7 Interworking Testing (IWT)

9.4 ACD Interworking Testing (IWT)

9.5 Bespoke Modifications

9.5.1 BT NUP Support

9.5.2 CLI Support

9.5.3 Destination Number Labels Support

9.5.4 UK Numbers Support

9.5.5 Basic Translations Support

9.5.6 Multi-customer Support

9.5.7 Restrictions on Customer Access to Configuration Parameters

9.5.8 Restrictions on Alarms Visible to Customers

9.5.9 Product Documentation and Help Screens

9.6 Initial Deployment - Central Sites

9.6.1 Hardware

9.6.2 Software

9.7 Initial Deployment – BT Operational Centres

9.7.1 Hardware

9.7.2 Software

9.8 Operational Readiness Testing

9.9 System Acceptance Test

9.10 Customer Premise Equipment

9.10.1 Hardware

9.10.2 Service

9.11 IVV&T Model

10. OPERATIONS AND SUPPORT

10.1 Roles and Responsibilities

10.1.1 BT Operations

10.1.2 Digital Support

10.1.3 GeoTel Support

10.2 Response Times

10.3 Quality of Service

10.4 Tools

10.5 Escalation

10.5.1 Fault Prioritisation

10.5.2 Third-party Suppliers

10.5.3 Documentation

10.5.4 Conference Calls

10.5.5 Metrics and Reporting

10.6 Remote Access

10.7 Trouble-ticketing

10.8 Alarms

10.8.1 Surveillance

10.8.2 Typical Failure Scenarios

10.8.3 Customer Alarms

10.9 Training

10.10 Routines

10.11 S/W and H/W Upgrades

10.11.1 Build Control

10.12 Design Support for Bespoke Systems

10.13 Disaster Planning and Fallback

10.14 Backups

11. TRAINING

11.1 BT Requirements

11.2 Digital/GeoTel Solution

12. PROVISIONING

12.1 Pre-sales Activities

12.2 Capturing Customer Requirements

12.3 Customer Ordering Process

12.4 Deliver Equipment and Services

12.5 BT Acceptance

12.6 Customer Acceptance

12.7 Customer Training and Migration

12.8 Customer “Hand-holding”

12.9 On-going Support

13. APPENDIX ONE - GLOSSARY OF TERMS

14. APPENDIX TWO - OUTSTANDING ACTIONS

15. APPENDIX THREE - SIGN-OFF SHEET

16. APPENDIX FOUR - LOCATION OF CSD COPIES

PREFACE

This document includes references to the following:

- NSO Operational Management Requirements for CPR0 (Rules Base Routing)

- CPR0, CPR1 and CPR2 Planning Requirements Capacity, Performance and Resilience

No attempt has been made to explicitly answer the points raised in these documents, preferring to provide a direct compliance statement against each document in it’s own right.

# INTRODUCTION

## Overview

In November 1995 BT issued a Request for Information (RFI) regarding a new Telemarketing Near Real Time Control and Reports (TNRTCAR) system.  In February 1996 BT issued a Request for Estimate (RFE) document.  Digital Equipment Co. Ltd. and GeoTel Communications Corp. issued a joint response to the RFE.  This was subsequently accepted by BT as the system most likely to fulfil BT’s TNRTCAR requirements, based upon information provided in the Digital/GeoTel RFE response.  At that point BT chose Digital/GeoTel as the single solution provider to move to the next phase of the project, production of the Customer Specification Document (CSD).

## Objective of the CSD

The objective of the CSD is fourfold:

- to produce a document that more closely defines BT’s requirements for the new Telemarketing service;

- to permit BT to gain an in-depth understanding of the GeoTel Intelligent CallRouter (ICR) product;

- to enable Digital and GeoTel to define and cost how their offering will match BT’s requirements;

- to identify and clarify any areas where there is a mis-match between the requirements and the solution.

## CSD Scope

The CSD document has two main purposes:

- To discuss with BT all aspects of the new Telemarketing implementation, known as the Telemarketing Near Real Time Control and Reports (TNRTCAR) system.  These discussions will centre around the following topics:

- Deployment Philosophy

- Delivery Methodology

- Escalation Procedures

- Training

- Review of the GeoTel Documentation

- Customer Design

- Customisation

- Communications

- Technical Interfaces

- Performance

- Dimensioning

- Security

- Integration

- Provisioning

- Support

- Acceptance Criteria

- VV&T

- Operational Policy

- To document the above discussions and produce an agreed Customer Specification document

The CSD must also take into account how the Digital/GeoTel system will fit into the existing BT architecture.  The following schematic outlines this, with the GeoTel ICR shown as a fundamental component:

Figure 1-1: CPR0 - Using BT NUP with Call Dropback

Note that the basic CPR0 call handling flow is as follows:

- Caller dials a number which requires translation (say 0800 123456).

- Local exchange (LE) forwards call to regional DMSU

- DMSU forwards call (using BT NUP) to IN/P1  (Intelligent Networking Phase 1) platform for translation

- IN/P1 sends Call Dropback message forwarding call to GeoTel ICR platform

- DMSU forwards call to GeoTel ICR platform using BT NUP

- ICR sends Call Dropback message forwarding call to translated number

- DMSU forwards call to final destination

This is the simplest form of the flow and further complexities are possible (e.g. involving calls originating from or being forwarded to the DDSN or international gateways).  However, the interface seen by the GeoTel ICR is the same as in the basic call flow and so these other flows are ignored in this document.

CPR0 (Cambridge Programme Release Zero) is the baselevel architecture for the initial TNRTCAR implementation and is what this document is concerned with.  CPR1, see below, is the architecture for the “NIP” deployment.  The “NIP” is BT’s replacement for IN/P1 (Intelligent Networking Phase 1).  This document does not concern itself with CPR1 (and the “NIP”).

Figure 1-2: CPR1 Using MCI 800 Gateway

Finally, it is not within the scope of the CSD document to produce detailed design documentation, functional specifications and associated documents.

## Methodology

To speed the implementation process BT wish to buy an “Out-of-the-box” solution, i.e. a ready made system, with an acknowledged customer base requiring as few modifications as possible.  Except where essential, changes to the system will be discouraged in favour of modifying existing processes and procedures.

A complete list of modifications that have been requested so far is detailed in Section 3.12, Bespoke Development.

The main sources of information for the CSD document have been derived from;

- the GeoTel documentation set, as follows;

- Product Description

- Supervisor Guide

- System Manager Guide

- Administrator Guide

- Admin Workstation Quick Start Guide

- Database Schema Handbook

- Installation Guide

- CSD discussion forums between BT, Digital and GeoTel;

- BT documentation relating to the project,

- Comments received on earlier drafts.

Any comments still outstanding when the document is approved may be resubmitted as change requests.

# DESIGN REQUIREMENTS

## Deployment

BT have a requirement that the customer hierarchy permits at least three levels:

- BT Operations (representing the product line).  Once the new system has been deployed the BT Telemarketing Services product line will be in overall control of it.  Telemarketing Services will then have the responsibility of marketing the new service to BT’s customers.  Telemarketing Services are effectively the TNRTCAR project’s customer.

- Customers.  There are two types of customers who will make use of the system, external and internal.  Each external customer organisation must use one customer licence and must not be aware of the presence of or have access to the data of other customers.  The system shall allow for from 1 to 350 customers.

- Moreover, BT Internal shall count as one customer and use a single customer licence.

- Customer Organisational Structures.  Each customer must be able to partition their own resources up in such a way that autonomous divisions are kept separate.

- One partition must be able to accommodate up to 34 ACDs.

Compliant.

The multi-customer extensions shall allow the hierarchy described above. Specifics regarding the dimensioning of the hardware configuration are described in Section 6.

Customers (including BT internal) shall be able to divide their operation up into a maximum of 5 partitions (if BT internal wishes to use more than 5 partitions, one customer licence will be required for each 5 partitions).  Each partition shall be independent and shall not be able to control, see or share information or agents with other partitions.  A partition must support up to 34 ACDs.  Currently the GeoTel multi-customer design allows for 20 ACDs per customer, this is being reviewed to become compliant with BT’s requirement for 34 per partition.  Further engineering of the multi-customer system will use algorithms to be provided by GeoTel that relate customer performance characteristics to basic computing capacity and storage, see Section 6.2 and Section 6.4.

The following diagram illustrates the above structure.

Figure 2-1: Customer Hierarchy

## Service Design

BT have a requirement to be able to offer four types of ICR service to their customers which does not preclude a mix and match scenario of these four, where a customer might choose to have ACD connected sites working with a non-ACD site/s in particular situations, eg additional agents required at short notice as an overflow.

  - Advanced (No ACD connectivity): this service provides intelligent call routing without using information from customer ACD equipment (i.e. as in the current Advanced Call Plan Service).  This further divides into two service types:

    - With customer statistics and control

    - Without customer statistics or control

  - “Stats-only” (ACD Connectivity): this service provides the customer with information about the performance of their telemarketing operation but does not make use of that information in call routing decisions.

  - Full ICR (ACD Connectivity): this service provides statistics and makes use of them in call routing.

  - Basic Translations (No ACD Connectivity): this service unconditionally translates a single dialled number into a single destination number.

Compliant.

The Digital/GeoTel solution shall support all the service types.  In all cases the customer information will be provisioned on the central ICR site but different CPE will be required as follows:

  - No ACD:

    - With statistics/control: only AWS systems will be required

    - Without statistics/control: no CPE required

  - “Stats-only”: both PG and AWS systems will be required

  - Full: both PG and AWS systems will be required

  - Basic Translation: no CPE required

Both “Stats-only” and “Full” will require identically configured CPE but no call plan will be required for “Stats-only” service.

BT require that they can offer service A2 (no ACD, no statistics, no control) and service 4 (basic translation) to customers without requiring further individual customer licences.  I.e. a customer licence shall only be required for services which involve CPE.

Compliant.

Basic translation services can be supplied without further individual

customer licenses using the capabilities of the Network-ICR as specified in

this document.  Product enhancements for this class of services, including

database routing and the VRU interface, may be requested as separate

contract items.

## Physical Design

BT intend to deploy the service in two central sites, with connections to various operational centres and customer sites (see Figure 2-2, below).  In addition BT has a requirement for model systems outlined below.

### Central Sites

BT require to deploy the solution in two central sites: Cambridge and Leeds with a high-speed connection between them supporting, among other things, a resilient link.

Compliant.

These two sites correspond to the two halves of the fault-tolerant GeoTel platform.  Each site will contain the following items:

- Multiplexers for connecting to the SPR signalling links

- Network Interface Controllers (NIC) for connection to the signalling network

- Network Intelligent Call Routers (ICR) for customer discrimination

- Network Logger for statistics functions associated with the network ICR

- Customer ICRs for executing customer call routing scripts

- Customer Loggers for statistics functions associated with those scripts

- LAN and WAN components providing internal and external connectivity

The inter-site link is described further in the Communications section.

### Operational Sites

BT have not yet specified the exact number of operational sites but they are currently intended to be:

- The Advanced Services NOU in London;

- The NBC operations centre in London;

- The COU at Oswestry;

- The Capacity Planning group in London;

- On-site management capability at each central site for access by on-site engineers.

- The Application Engineers’ centre (location not yet known)

Compliant.

Each site will contain one or more Administrative Workstations (AWS). The total number of AWS is specified in the Dimensioning section.  BT will be supplying the TCP/IP routing infrastructure required to connect these sites to the ICR sites.

### Customer Sites

The total number of customer sites is not yet known.  However, BT require the solution to be flexible enough to scale to 1200 customer locations, where a “customer location” contains either a PG connecting to an ACD or one or more AWS systems or both.

Compliant.

Each customer site will contain one or more Peripheral Gateways (PG) and/or one or more AWS.

### Demonstration Model

BT requires a “model” of the ICR to be used in demonstrations and solution design.

Compliant.

See Section 6.9.1 for details of model configuration.

The model system will be installed initially without the NIC or Larscom Orion 400.  These components will be added following completion of acceptance testing for the BT NUP protocol development work, see Section 5.

### Integration, Verification, Validation and Test Model

BT require a “model” of the ICR to be used in Integration, Verification, Validation and Test (IVV&T), probably located in the SSTF in Martlesham Heath.

Compliant.

See Section 6.9.2 for details of the model configuration to be supplied.

### Operational Support model

BT Requires that the supplier provide, own,  and maintain a model (on their premises) to be used for problem reproduction/verification, bug-fix and new release testing. This model should reflect the BT design and network infrastructure, multi-customer platform, C7 BT NUP signalling, and all other developments completed by the supplier for BT.

Compliant.

Digital would envisage that this support model would be resident on GeoTel’s premises and would be available to their developers, testers and support staff as part of the normal GeoTel support contract.

However, GeoTel will not have access to the BT C7 network and will require BT support to assess results of anomalous network behaviour.

## Pre-staging

BT require that the supplier shall pre-stage all configurations (including both BT equipment and CPE).

Compliant.

Digital and GeoTel shall pre-stage all configurations as part of the Value-added Implementation Service (VIS).  Pre-staging of the hardware will mean that the hardware configuration is thoroughly exercised and tested prior to delivery.  Furthermore, if configuration parameters such as IP addresses are known prior to delivery these will be inserted in configuration tables so that a minimum amount of on-site work is required to get equipment up and running.  Moreover, the GeoTel application software will also be installed and configured prior to delivery.

## Packaging

BT requires that the supplier shall prepare, supply and install all equipment including undertaking environmental surveys and packaging equipment appropriately for its target environment.

Compliant.

The detailed packaging requirements are described below, with Figure 2-2 illustrating the overall configuration.

Figure 2-2: Overall ICR Physical Design

The equipment within the dotted lines shall all be provided by the supplier (see also the Communications section) and shall be rack mounted, installed and tested.

Compliant.

The customer premises PG equipment shall be packaged as a “black box”, i.e. the customer sees a BT-supplied communications cabinet with the following external connections:

- Power (240V, 13A, single phase) — no UPS is expected to be required as calls are still routed even if this component is down (but the customer may provide an external UPS if they wish)

- Frame relay network connection (connects to BT supplied managed WAN)

- Primary ACD connection (depends on ACD type: may be more than one connector)

- Redundant ACD connection (optional)

- Customer TCP/IP network connection (10BaseT Ethernet connector for connection to a customer supplied router, bridge or concentrator)

- Telephone connection for remote access modem (connects to BT supplied telephone line)

Compliant.

The “black box” will contain the following components:

- Digital IP Router

- Private (within cabinet) Ethernet LAN

- Peripheral Gateway PC

- Optional redundant PG PC

- Internal cabling

See Section 6.9.4 for detailed equipment configurations.

The customer AWS shall also be supplied as a “black box” already installed and configured to provide the customer with immediate information.

Compliant.

This is a single workstation system which will be pre-configured by Digital to be immediately usable. See Section 6.9.4 for detailed equipment configurations.

For customer sites which require an AWS but which do not have connectivity to the customer’s internal WAN an “IP Router” configuration shall also be available.  This shall also be a “black box” communications cabinet with the following external connections:

- Power (240V, 13A, single phase)

- Frame relay network connection (connects to BT supplied managed WAN)

- Customer TCP/IP network connection (10BaseT Ethernet connector for connection to a customer supplied router, bridge, concentrator or direct to a local AWS)

Compliant.

The “black box” will contain the following components:

- Digital IP Router

- Internal cabling

See the Dimensioning section for detailed equipment configurations.

## OSS Integration

BT have a requirement to integrate the ICR platform into the OSS environment for provisioning, management and, possibly, billing purposes in the future.  However, there is no requirement for OSS integration in CPR0 (even into OSMF).

No OSS integration will be provided in CPR 0.  Future OSS integration will be possible by accessing the GeoTel database however detailed design will wait until the next release.

## Prompt and Collect

BT have a requirement to be able to integrate “prompt and collect” features into the ICR routing plans in the future.  However please note that  the use of BT NUP does not permit this capability.

# APPLICATION REQUIREMENTS

## Capabilities

BT require flexible and powerful intelligent call pre-routing which has the following improvements over current translation services:

- Near real time call reports

- Near real time control of call routing logic

- Easy to use customer programming of call routing logic

- Customer friendly graphical user interface

- Platform linkages to call centres and the core network to enable call routing logic to take account of actual agent, call centre and network conditions.

Compliant.

The GeoTel solution is fully described in the documentation which has already been supplied to BT and it provides the features described above.

## Documentation and Help Screens

BT require operational documentation and help screens reflecting the developments undertaken by the supplier on behalf of BT (for BT Operations staff).

Compliant.

The following GeoTel product documents,  updated to take into account the developments supplied to BT, shall be supplied to the BT designers and BT Operations staff:

- GeoTel Product Description

- GeoTel Administrator Guide

- GeoTel System Manager Guide

- GeoTel Supervisor Guide

- GeoTel Database Schema Manual

- GeoTel Installation Guide

- Microsoft Windows NT Server documentation

- Microsoft BackOffice documentation

- Microsoft SQL Server documentation

This updated documentation will include a description of the new two-tier architecture (Network ICR and Customer ICR split) for the multi-customer solution and will describe any resulting impact on the resilience of the product.

The following custom document shall be supplied to BT Operations:

- VIS Site Documentation

- Site layout, environmental design, etc. provided by Digital’s Value-added Installation Service.

BT also require documentation to be supplied to the customer again reflecting the modifications to the software as supplied to BT.

Compliant.

The following documents will be supplied to the end customer:

- BT ICR Product Description

- Customised from the GeoTel Product Description to describe those parts of the product which are visible to the end customers.

- BT ICR System Manager Guide

- Customised from the GeoTel System Manager Guide to describe creating and editing scripts and other control operations permitted to end customers

- BT ICR Supervisor Guide

- Customised from the GeoTel Supervisor Guide to describe using the Administrative Workstation (AWS) to report information permitted to end customers

- BT ICR Database Schema Handbook

- Customised from the GeoTel Database Schema Handbook to only describe those database tables and fields which end customers are permitted to use

- On-line help

- Customised from the existing GeoTel help to describe those parts of the product which are visible to the end customers.

BT require that the customisation of the end user documentation and help screens shall include the following changes:

- Restriction to those features/items being made available to end users

- No mention of GeoTel except in copyright notice

- British English spellings and grammar

- BT-specified covers and logos

- Addition of BT-specified trademark information

Compliant.

In addition, BT has the following requirements:

- The supplier shall provide and operate an Issue Control mechanism to deal with amendments and updates to documentation.

- Compliant.

- The supplier shall ensure that all documents and updates have a controlled distribution to agreed named BT distribution points.

- Compliant.

- The supplier shall grant BT the right for any purposes to reproduce, have reproduced, have modified, amend, have amended and use free of charge all documentation and information supplied by or to be supplied by the Supplier.  (Any disclosure to a third party shall require the Supplier’s approval.)

- Partially compliant.

- Subject to a review on a case by case basis, Digital will grant BT the right to reproduce, have reproduced, have modified, amend, have amended and  use free of charge all documentation and information supplied by the Supplier.

## Supported ACD/PBX Equipment

BT require that the solution shall support the following additional ACDs over and above the standard product:

- Northern Telecom - DMS100

- GPT - iSDX

- Ericsson - ACP1000

Partially compliant.

The solution supports the following ACDs:

- Northern Telecom - Meridian

- Aspect - CallCenter

- Rockwell - Galaxy and Spectrum

- AT&T - Definity

NOTE: Currently, support for the Northern Telecom Meridian is restricted to the SNN interface, GeoTel will provide support for the IPE interface build on the BT demonstration ACD system. GeoTel will also be providing support for the Northern Telecom Meridian 1 “MEI” interface which is independent of both the SNN and IPE interfaces. Support for the Northern Telecom DMS100, GPT iSDX and Ericsson ACP1000 ACDs could be added in the future if these manufacturers agree to co-operate with BT and GeoTel in building support.

BT require that the solution can co-exist with other CTI equipment, in particular Genesys.

Partially compliant.

The solution can co-exist with other CTI equipment as long as the interface presented by the ACD, and the information available over that interface, is unaffected.  In particular, the Genesys system will not disrupt the PG to ACD interface.  However, the Genesys Router application module may cause disruption in some circumstances, when the Open ACD Interface described in Section 5.3 should be used.

## Customer Database Access

BT require that customers shall not have direct access to the internal databases except through the supplied report generation tools.  However, BT also require that the customer shall be able to export the documented information from the database to their own favoured database system.

Partially compliant.

Customers can use the report generator tools to access the database.  Although customer ODBC access into the AWS or ICR database will not be supported, no code modifications will be made to explicitly prevent it.  Customers will be permitted to export data to their own systems using the standard GeoTel report generator features.

Note: customers will not be able to access data which is private to BT or which is about other customers. ODBC access to the database will be controlled by the same NT security features used to control other forms of access to the database, see Section 8.7.3.

## Calling Line Identity (CLI)

The BT network makes available two sorts of CLI: network CLI and presentation CLI.  Neither is included in the initial message (IFAM) but can be requested during the transaction (using ACI).  Presentation CLI can represent any telephone number (with the leading zero dropped).  Network CLI normally represents a line but can take any of the following forms:

- UK line — the telephone number of the line without the leading zero — normal case.  In this case the leading 6 digits have geographic significance.

- “Partial CLI” (PCLI) — not really a partial CLI but a completely differently formatted string identifying a point of entry to the network — used for a few remote exchanges (e.g. in Scotland) and some OLO lines.

- International line — a partial or complete international telephone number with a single leading zero — if call came via a BT international gateway and the calling country supplied CLI.

- International PCLI — a “partial CLI” as described above representing an international connection — used if call came via a BT international gateway and the calling country did not supply CLI.

- Unknown — if the call came through an OLO international gateway the CLI could be anything (possibly indistinguishable from one of the other cases above).

For regulatory reasons, BT require that customers shall not see or make use of full CLIs if release has not been authorised.  However, BT also require that geographic routing should make use of the geographically significant portion of network CLIs and that call plans can make use of full CLI if release is authorised. Plus BT wish to be able to recognise individual callers for “Group Link” applications.

Partially compliant.

In the first release, in order to avoid unauthorised CLI release and to avoid modifying the GeoTel application, the NIC shall perform the following process:

- Always request network CLI and always copy into the Call Dropback message.

- If the network CLI is a partial CLI discard it and pass a zero length CLID to the GeoTel ICR (and exit this procedure)

- Otherwise, add a 0 to the front of the CLI.

- If the CLI is releasable then provide the complete string to the GeoTel ICR.

- Else, truncate the CLI to the leading 7 digits (or fewer if fewer were supplied) and provide the resulting string (normally 7 digits but can be fewer) to the GeoTel ICR as the CLID.

This permits geographic routing to work correctly (except for the small percentage of calls which contain a PCLI) and the script will allow the use of the CLI to identify the particular customers unless the CLI is specifically withheld by the caller or OLO. The release information is held in flags returned with the CLI.

## C7

BT require that the application shall interface to the network using BT NUP as described in the Interfaces section.

Compliant.

## First Party Clear

BT has no requirements on this application to take any specific action to handle “first party clear”: all required actions occur elsewhere in the overall Solution Design.

## Labels

BT require that the application shall only permit the customer to use BT-defined labels instead of destination numbers.  The customer shall not be able to modify the definition of a label or to replace a label with a number or to use another customer’s label.  However, the customer shall be able to use any of their own labels at any position in the script.

Compliant.

The application shall enforce the use of labels in the way described above.  The Customer Intelligent Call Router (C-ICR) will exclusively use labels: i.e. the customer will only see labels in their call routing scripts and reports.  The Network Intelligent Call Router (N-ICR) will validate that the label returned is valid for that customer and will translate between that customer’s labels and real numbers.

## Year 2000

BT require that the platform operates correctly before, during and after the year 2000.

Compliant.

The software can handle dates before, during and after the year 2000.  It recognises the year 2000 as a leap year.

## Dial Plan

BT require that the platform shall handle dialled, destination and calling (CLI) numbers of 4 to 30 digits length and that it shall also be possible to handle longer numbers in the future.  The platform shall not be tied to the details of the existing UK numbering plan as it may change in the future.  The platform shall perform the following validation checks (and only the following checks) on dialled and destination numbers:

- A calling number is between 4 and 20 digits long

- A dialled number consists only of decimal digits, 8 to 16 digits in length

- A destination number can consist of decimal or hexadecimal digits between 4 and 30 digits in length.  For the coding of the digit strings in C7 messages (BCD or straight binary) see BTNR 5505 and Draft BTNR 5167.

Compliant.

GeoTel will modify its treatment of dialled and destination numbers to allow longer numbers and to remove any assumptions of US numbering plan structure.

## Basic Translations

BT have a requirement to support “basic translations”.  These are fixed translations from dialled number to destination number.  BT require that the solution can provide basic translations in an efficient way (see Performance section for basic translation performance requirements).

Compliant.

Basic translations can be performed using call plans executing on the Network ICR system.  However, no additional software development is planned to provide an automated user interface for provisioning these plans.  The following possible options exist, in increasing order of complexity:

- The minimum capability is to provision  these call plans using the standard GeoTel application tools with each basic translation having its own call plan, dialled number list (with one entry), destination number list (with one entry), etc.  This involves re-keying the data and performing multiple commands to set up the translation.

- A simple user interface into the GeoTel database could provide an easier to use interface but would still require rekeying.

- A Translation Manager interface could be developed to automatically manipulate the GeoTel database.

The first option will be available as part of the solution.  The other options can be investigated as change requests at BT’s discretion.

## Bespoke Development

The following changes shall be made to the standard GeoTel application (all are described in more detail in their respective sections of this document but are summarised here for clarity):

- The supplier shall develop a BT NUP implementation.

- The supplier shall develop an ICR hierarchy to allow multiple customer support.

- The product documentation and associated help screens shall be customised as described in Section 3.2.

- The solution shall support CLI in the way described in Section 3.5.

- The solution shall support destination number labels in the way described in Section 3.8.

- The solution shall support UK numbers as described in Section 3.10.

- The solution shall support basic translations as described in Section 3.11.

- Customers shall be prevented from modifying their configuration except for the call plan and agent skill sets.

- Customers shall only have visibility of a limited subset of alarms as described in Section 10.8

- The solution shall have a fully defined published generic interface to allow other ACD manufacturers to integrate their switches with the BT offering.

- The solution shall support the Northern Telecom DMS100, Ericsson ACP1000 and GPT iSDX switches.

Compliant.

All items are included, subject to the conditions of the respective sections, except that item 11 is the subject of a separate estimate which has not yet been accepted.

# COMMUNICATIONS

## Overall Architecture

The overall communications architecture consists of:

- Two central ICR sites (Leeds and Cambridge), connected together with a high speed inter-site link, with local LANs on each site connected via Ethernet.

- Connection of the central sites to the Public Switched Telephone Network (PSTN, also known as the Core Transport Network, CTN) via C7 signalling links.

- Customer (CPE) sites, connected to the central sites with a Frame Relay network provided by BT’s Managed Network Service group and locally connected to the customer’s ACD equipment. The customer sites may also be connected to the customer’s own TCP/IP network.

- Operations centres, connected to the central sites via a BT TCP/IP network, probably the MPRN.

The main aim behind the dual site architecture is to provide a resilient configuration that for short periods of time can cope with the total loss of either site, so that the service being provided is unaffected as far as the customers are concerned.  This is vitally important since this is a revenue generating system and will be a key component for BT when offering their customers improved service level agreements.

Compliant.

The overall network design is shown in Figure 4-1, illustrating the dual site architecture and the interfaces.

- Interface I1 is the inter-site link.

- Interface I2 is the interface between the central sites and the customer sites.  Please note discussions need to be completed regarding Frame Relay provision for this interface.

- Interface I3 is the interface between the central sites and the operations centres.

- Interface I4 is the interface between the BT-supplied CPE and the customer’s own internal TCP/IP network.

Figure 4-1: Overall ICR Communications Design

### Central Sites

Figure 4-2 shows the central site configuration, which will be identical at each site.  Please note the following regarding Figure 4-2:

- The NIC configuration is detailed in Section 6.1.2.

- The Larscom Orion 400 multiplexers each terminate one E1 connection and de-multiplex the four 64 Kbps channels to V.35 for the C7 cards.

- The three LAN segments connected by an Ethernet switch provides the ability to keep traffic between related components (e.g. Network ICRs and Network Logger) isolated on its own shared LAN without affecting bandwidth on the other two LANs.

- This allows the standard GeoTel public/private LAN configuration to be avoided in this more complex configuration.

- Interfaces I1, I2 and I3 are as defined in Section 4.1.

Figure 4-2: ICR Central Site

BT requires that the supplier shall provide all the equipment forming the ICR central site.

Compliant.

Digital shall supply the following equipment (see Section 6.9.1 for equipment specifications and number):

- Windows NT PCs running the GeoTel application

- Digital Ethernet switch (one per site) with a connection to each LAN segment

- Digital IP routers to connect to the ICR-to-PG WAN

- Firewalls for the connections to the ICR-to-PG WAN

- Firewall for the connection to the MPRN

- Multiplexers for connecting to the SPR signalling links

- Ethernet LAN segments

- Other cabling including power

NOTE: BT will be supplying the IP routers and network connections for the inter-site link (I1) and the central sites and BT operations centers (I3).

BT will supply E1 connections to the signalling network.  BT requires that the supplier provides all adaptation and terminating equipment.

Compliant.

Digital shall supply Larscom Orion 400  multiplexers to take an E1 “pipe” and de-mux four 64 Kbit channels into C7 cards in the  Network Interface Controllers (NICs).  For more details see Section 4.3.5.

### Customer Sites

At the customer call centres an ACD will be linked into the central configuration by a Peripheral Gateway connected to an IP router.  The ACD connection to the Peripheral Gateway is dependent upon the type of interface available on the ACD, but is generally one of: serial, Ethernet or X.25.

The Peripheral Gateway options available, mainly in terms of redundancy, are also dependent upon the make and model of ACD.

shows the CPE configuration, for a standard Peripheral Gateway set-up.

Figure 4-3: ICR Customer Premises Equipment

BT requires that the supplier shall provide the CPE equipment contained within the dashed line.

Compliant.

Digital shall supply the following equipment (see Section 6.9.4  for equipment specifications):

- Windows NT PC running the GeoTel PG software

- Optional second PG for redundant configuration

- Digital IP router to connect to the ICR-to-PG WAN

- Private (within cabinet) Ethernet LAN with cabling

Customers will also have administration sites containing one or more Admin Workstations (AWS).  These are connected to the Peripheral Gateway equipment either directly or via the customer’s own TCP/IP network (as shown in Figure 4-3).

BT requires that the supplier shall provide the AWS equipment.

Compliant.

Digital shall supply the following equipment (see Section  6.9.4 for equipment specifications):

- Windows NT PC running the GeoTel AWS software

## Roles and Responsibilities

As a general principle, BT requires that all equipment supplied under contract, except inter-site and ICR to PG communications links, are provided and supported by the supplier.  BT will provide inter-site communications links.

Compliant.

## Volumes

### ICR WAN (I1)

The design requires a highly available IP routing connection between the two central sites guaranteeing at least 6 Mbps of bandwidth (even under failure conditions) with no more than 25 ms one-way delay (latency).  This latency includes any delays introduced in the routers or the LAN in addition to the WAN link itself. The bandwidth corresponds to that required to handle the large system metrics (see Dimensioning section). The design also requires provision of two priority levels for traffic between the two sites.

This connection will be supplied by BT. In order to provide minimum latency on the LAN, this connection should be provisioned with a single IP router at each site. BT will supply this link to the interface specified in Section 5.5.1.

### ICR to PG and ICR to Customer AWS (I2)

The solution requires IP routing connectivity between each customer site (containing PG and/or AWS systems) and the central sites.

This connection must provide at least 64 Kbps of bandwidth from each central site to each customer site which contains an AWS (this bandwidth must be available from either central site to the customer site but not simultaneously from both) with no more than 1 second one-way delay (latency).  This latency includes any delays introduced in the routers or the LAN in addition to the WAN link itself.  Note that this bandwidth restricts the number of skill groups to less than 200 per partition. The IP network should be prioritised so that event notification messages can receive higher priority treatment than any half-hourly ACD report records.

This connection must also provide adequate bandwidth from each customer site which contains a PG to each central site (this bandwidth must be available from either central site to the customer site but not simultaneously from both) with no more than 1 second one-way delay (latency).  This latency includes any delays introduced in the routers or the LAN in addition to the WAN link itself.  The minimum bandwidth required is calculated by multiplying 8 Kbps by the expected number of calls/second routed to the ACD on that site.

Compliant.

Subject to BT supplying Frame Relay network as described below.

At the central sites, Digital will supply suitable IP routers to permit connectivity to the customer sites: see Section 6.9.1 for configuration details and equipment volumes and Section 5.5.2 for interface specifications.

At each customer site Digital will supply a single IP router to permit connectivity to the two central sites: see Section 6.9.4 for configuration details and Section 5.5.2 for interface specifications.

The WAN connection between the Digital supplied routers will be provided by a BT managed network frame relay service providing adequate committed information rate (CIR) to each customer site (this bandwidth will be available from either ICR site to the customer site but not simultaneously from both) and no more than 250 ms of delay (latency).  The required CIR is the greater of the bandwidth required in the two directions as described above (i.e. MAX(64 Kbps, Calls/second * 8 Kbps)).

At each central site, the connection to this WAN must provide aggregate bandwidth of at least 2 Mbps.  Please note that this connection will be sufficient to connect up to 32 customer sites and is sufficient to support the initial small configuration being installed. Additional WAN links will be required as the number of sites increases beyond this number. BT will supply this network with the interfaces specified in Section 5.5.2 and Section 5.5.4.

### ICR to BT AWS (I3)

Operational management requires IP routing connectivity between every site (central site, operations centre and customer site) and each operations centre (including NBC, ASNOU and others). This connection must provide at least 64 Kbps of bandwidth between each operations AWS and each central site with no more than 50 ms one-way delay (latency) to the central sites and no more than 200 ms latency to customer sites.  This latency includes any delays introduced in the routers or the LANs in addition to the WAN links themselves.

In order to provide this IP routing connectivity, BT will supply suitable IP routers to connect the ICR central sites to the MPRN.  This will provide IP routing connectivity from the operational centres to the central sites and, via them, to the customer sites.

BT will supply a routed WAN between the central sites and the operations sites allowing 64 Kbps of committed information rate for each AWS with latency of less than 30 ms. The connection to this WAN at the ICR must provide aggregate bandwidth of at least 2 Mbps (at each central site) and conform to the interface described in Section 5.5.3.

BT requires that the supplier shall provide suitable connections on the AWS systems supplied to the operations centres.  These systems will be connected to the WAN described above.

Compliant.

See Section 5.5.3 for more details.

### PG to Remote ACD

Support of remote ACD equipment (i.e. PG on one site, ACD on another) is excluded from this release in order to simplify the initial service design and deployment.  BT may wish to provide this capability in the future.

The GeoTel solution is able to offer this capability for Rockwell Galaxy ACDs and other ACD models that connect to Peripheral Gateways via Ethernet TCP/IP LAN.  If this is required, in a future release, the remote ACD link will be provided by BT and details will be designed at that time.

### C7 Links

BT will provide C7 signalling links between the SPRs and the ICR sites.  These links will consist of eight E1 connections per ICR site, where each E1 has four 64 Kbps channels in use.  Each ICR will be connected to each SPR by two E1 lines, diverse routed.  The connection consists of four link sets: each link set consists of a pair of channels, one from each physical line.  BT requires the supplier to connect to these links.

The above configuration reflects the maximum signalling interconnection requirements.  However, the C7 signalling design is still being reviewed and may be the subject of further rationalisation i.e. reduced for connections to "single cluster" SPRs.

Compliant.

See Section 5.5.6 for more details.

## Resilience

The inter-site link is critical to providing the fault-tolerant behaviour of the GeoTel application and must be fully redundant.

BT will supply this link which is fully described in Section 4.3.1.

The routed TCP/IP wide area network from the ICR to the operations centres must be resilient to single points of failure outside the ICR itself.  If this network fails, the operations centres can use the remote access connection but this severely limits the number of operators and the bandwidth available to them.

BT will supply this network which is fully described in Section 4.3.3.

The ICR-to-PG managed WAN must also be resilient to single points of failure outside the ICR itself.  If this network fails, such that connectivity is lost between one ICR site and one CPE site, the customer will still be able to fully use the service as long as connectivity remains to the other ICR site.  If connectivity to an ACD site is completely lost calls will still be routed (without the benefit of ACD information) but real-time statistics will not be available.

BT will supply this network which is fully described in Section 4.3.2.

## IP Routing

In order to reduce operational problems, and assist security analysis, BT require that all IP routers supplied as part of the platform shall be configured to use static routing.

Partially compliant.

The CPE routers and the frame relay interfaces on the central site I2 routers will be configured to use static routing.  The I1 inter-site WAN routers will also be configured to use static routing.  However, the I3 routers (connecting to the MPRN) and the LAN interfaces on the central site I2 routers will use dynamic routing using OSPF.  This is to allow the routes between the operations centres and the customer sites to be correctly calculated if a frame relay PVC or interface is down.

Note that the use of static routing is subject to the subnet addresses being allocated as described below (use of variable length subnet masks).

BT require that the IP routers are configured to prevent IP traffic being passed between customer premises (whether the same or different customers).

Compliant.  The use of static routing described above will achieve this.

BT will supply IP addresses for each system.

Individual BT-supplied IP addresses are required for each of  the following components:

- 4 addresses for each Network ICR (high and low priority, visible and private)

- 4 addresses for each Network Logger (visible, private and 2 x remote access)

- 4 addresses for each Customer ICR (high and low priority, visible and private)

- 4 addresses for each Customer Logger (visible, private and 2 x remote access)

- 1 address for each BT AWS

- 1 address for each NIC

- 1 address for each Larscom Orion multiplexer

- 1 address for each central site IP Router’s LAN interface

- 1 address for each end of the inter-site link

- CPE: 1 address for each PG

- CPE: 1 address for each IP Router (private LAN interface only)

- CPE: 2 addresses per customer site (for remote access)

If the customer has their own IP network then customer AWS addresses and CPE IP Router public LAN addresses should be assigned by the customer.  However, if the customer does not already have a TCP/IP network then the following addresses are also needed:

- CPE: 1 address for each customer AWS

- CPE: 1 address for each IP Router public LAN interface

- CPE: 2 addresses for each site for remote access

Note: Digital recommends that the WAN connections between the central sites and the customer sites should use unnumbered links, to save IP addresses.  However, Digital recommends that the inter-site link should be a numbered link, with its own subnet,  to assist in problem solving for this critical connection.

These IP addresses must be grouped into the following subnets:

- Leeds ICR subnet (all Leeds equipment)

- Cambridge ICR subnet (all Cambridge equipment)

- Inter-site link subnet

- One subnet for each operational site LAN (ASNOU, COU, NBC, etc.)

- One subnet for each customer site private LAN and remote access

- One subnet for each customer site public LAN for which BT is assigning addresses

The customer site subnets must be allocated from contiguous ranges such that all customer subnets connected to a particular I2 router in the central site can be configured using a shorter subnet mask.  This avoids having to have a static routing table entry for every individual customer subnet in every router.

For example, using the rules described above, a customer site private LAN subnet generally must contain 4 addresses and so can use a 29 bit subnet mask (i.e. 255.255.255.248).  Each central site router can connect 64 customer sites and so can use a 23 bit subnet mask (i.e. 255.255.254.0).

## Backups

See the Operations section for backup procedures.  As no backups cross the network there are no Communications requirements for backup.

## Security

See the Security section.

## Network Management

BT requires that the internal (“private”) LANs (at the central sites and at the customer sites) and all the IP routers shall be remotely managed from the BT operations centres.

Compliant.

Digital shall provide a NetView system at two of the operational centres, which can be used to manage the communications components.  The LAN hubs and IP routers have their own management tools which are integrated with NetView and which use SNMP and other TCP/IP protocols (e.g. PING and TELNET) to control and monitor the network components.

### Surveillance

BT have a requirement that all links can be monitored and will generate operator alarms if they fail.

Compliant

- C7 links

- The multiplexers shall generate SNMP alarms if  the E1 connections fail

- NIC links

- The NICs shall generate GeoTel alarms if the channel to the SPR fails or if any of the C7 signalling protocols fail.

- ICR site internal LAN

- The application shall maintain “heartbeats” to all ICR components and shall report all loss of connectivity as GeoTel alarms

- Inter-site WAN

- The application shall maintain “heartbeats” to all ICR components and shall report all loss of connectivity as GeoTel alarms

- ICR-to-PG WAN

- The application shall maintain “heartbeats” to all PG components and shall report all loss of connectivity as GeoTel alarms

- Operational centre WAN

- The AWS will provide visual indications to the user of the status of all the links monitored by the central control system. The AWS will also detect loss of its real-time feed from the central control system and will provide a corresponding visual indication to the user.

In addition, the IP routers shall generate SNMP traps if any of the IP routing links fail, even if application connectivity is not lost.

### Customer Network

BT requires that the AWS systems on the customer’s own TCP/IP network shall be remotely managed  from the BT operations centres.

Partially Compliant.

The AWS systems will be remotely manageable as long as the customer network (to which they are attached) provides full connectivity to the IP router in the CPE.  If the customer wishes to put any sort of router or firewall between the CPE and the AWS it must be configured to pass all IP traffic (UDP and TCP, all ports) between BT IP addresses and the AWS systems.

## Remote Access

BT requires that the solution shall include one modem per physical site (central sites, operational centres and customer sites)  to permit remote access from BT operational centres and supplier support.  BT will supply one analogue telephone line per site connected to this modem.

Compliant.

Digital shall supply at least one modem per physical site.

# INTERFACES

This section addresses BT requirements on the various INTERFACES between the solution and the BT network and the Customer Premise Equipment (CPE). The interfaces discussed include both those related to physical connectivity as well as the interfaces to various systems and processes used to manage the system. It will include discussion of requirements in the following areas:

- C7 signalling connections

- ACD connections

- PG Interface to ACDs

- CTI interfaces to ACDs

- Physical links

- OSS system interfaces

- Administration centre interfaces

## C7

The call centre handling platform must interface to the existing BT network by use of the existing network interconnection mediums and signalling interfaces. These are  defined in the following BT Network Requirement (BTNR) documents:

- BTNR 146 (MTP)

- BTNR 167, Issue 3 (BT NUP)

- BTNR 164, Issue 1 - Enhancements to National User Part to Support Intelligent Network (Phase 1)

- BTNR 5505,  Vol I&II, Issue 1 Draft b, 8th August 1996 - Signalling and Associated Call Control Specification for Intelligent Network Service Control Point Using Call Drop-Back.

- BTNR 5167, Section 2, Issue 2 Draft E, July 1996 - CCITT Signalling Specification (BT Version), National User Part (NUP)

Note 1: Peter Donovan of BT has stated that the two further BTNRs referenced from BTNR 146 are not relevant.

Compliant.

NOTE: The proposed BT NUP support will isolate all  knowledge of the BT NUP interactions in the Network Interface Controllers (NIC). The other components of the solution interact with the NIC via a proprietary signalling interface independent API.

Specific requirements are identified below.

### Signalling End Point

The implementation should only act as a signalling end point and should not provide MTP Relay (as defined in Annex A in BT’s purchase order for BT NUP Support).

Compliant.

This will be addressed during the planned development of the BT NUP support.

### Multiple Point Code Operation

The implementation must be capable of supporting Multiple Point Code Operation with a minimum of ONE and a maximum of EIGHT multiple point codes per node.

Compliant.

This will be addressed during the planned development of the BT NUP support.

### Message Transfer Part

The implementation must support and interwork with the C7 MTP as specified in BTNR 146.

Compliant.

This will be addressed during the planned development of the BT NUP support.

### Call Dropback Protocol

The implementation should use the Call Dropback protocol as defined in BTNR 167 and modified in BTNR 164 and as defined in BTNR 5505.

Compliant.

This will be addressed during the planned development of the BT NUP support.

NOTE: the Call Dropback protocol is only a small subset of the overall BT NUP. Therefore, it is not expected to introduce a high degree of risk in the solution. GeoTel will be using Danar Corp. to implement the Call Dropback protocol as they have experience of implementing other areas of BT NUP for GeoTel and other companies. Danar is the company that GeoTel use to provide the majority of their protocol support code.

The implementation should “gracefully” reject/fail all other BT NUP protocols received, these will always be error conditions.

Compliant.

The implementation will reject all other BT NUP protocol messages as required by BTNR 164 and BTNR 5505.

### Additional Call Dropback Protocol Information

The following additional information on the Call Dropback protocol was specified during the Requirements Workshop:

- Always uses IFAM (Initial and Final Address Message)

- IFAM only contains dialled number, not Network CLI

- Network or presentation CLI needs to be separately requested using an ACI (type 7) request

- ICR appears as pseudo SSP so interaction is similar to normal call set-up

- The ICR will need to be modified to understand the UK dialplan

- The NIC will always populate the CLI in the Call Dropback message

Compliant.

The above points will be addressed during the GeoTel development of the BT NUP Support. See also Section 3.5.

### C7 Link Failure

In the event of failure of a site ICR the implementation shall take the following action:

- If the ICR or Loggers fail but the NIC remains functional, then the NIC shall issue BT NUP Blocking messages via the connected SPRs to the DMSUs to have them use their secondary links to the other central site.

- If a single NIC PC system fails, then the SPRs will detect linkset failure and the MTP will determine that there is no connectivity available to that NIC. This will cause the DMSU to switch to using its secondary point code  and the message will be directed to the other site.

- The  SPR will detect a single C7 or link failure and will cause a changeover of traffic within the linkset.

Compliant.

## ACD

The implementation is required to work against a range of ACD equipment typically installed on BT Customer sites, including:

Northern Telecom - Meridian, DMS100

GPT - iSDX

Ericsson - ACP1000

The supplier is required to demonstrate interworking with the above ACDs and to document hardware and software interoperability combinations.

Partially compliant.

The GeoTel solution currently supports interworking with the Northern Telecom Meridian. GeoTel are happy to work with Northern Telecom (DMS100),  GPT and Ericsson to provide support for their ACD systems but this requires agreements to be established with these parties.  It may be useful to involve BT in these discussions to demonstrate the value of Northern Telecom, GPT and Ericsson working with GeoTel to develop this support.

## PG Regulatory Interface

For regulatory reasons, BT would like the interface between the BT-based equipment and the CPE to be “open” and available for other ACD vendors to write interfaces for.

Compliant.

GeoTel will publish a generic, standards-based message interface to the Peripheral Gateway (the “Open ACD Interface”) which will allow communication with additional ACD equipment beyond those currently supported by GeoTel today.  Today, there exists an undocumented internal interface against which four ACD drivers are already available, these drivers support:

- AT&T Definity G3 (EAS or non-EAS)

- Aspect Call Center

- Northern Telecom Meridian 1

- Rockwell Galaxy and Spectrum

There is a further generic driver, which supports statistics collection but not control functions.

The Open ACD Interface within the peripheral gateway will permit comprehensive access to the functionality of the peripheral gateway on a vendor-independent basis.  This interface is expected to be a subset of the functions provided by the CSTA standard.  See Figure 5-1: Peripheral Gateway - Open ACD Interface for diagram of internal interface.

Figure 5-1: Peripheral Gateway - Open ACD Interface

To ICR

To ACD

Internal Interface in PG

ACD specific interface

Open ACD Interface

Internal Interface to ICR

Peripheral Gateway - Open ACD Interface

## CTI

BT require the solution to co-exist with ACD add-on packages (such as those marketed by Genesys)  which use the CTI links to the ACD.

Compliant

In most cases, the PG and the add-on application can share the ACD CTI links and can co-exist with no extra work to either application.  In some cases, however, either the ACD cannot handle more than one CTI application or the operation of the add-on package causes information from the ACD to be invalid (some modes of operation of Genesys Router application module cause this behaviour, see Section 3.3).  In those cases, the add-on package can be modified to present data to the PG using the regulatory interface described in Section 5.3.

## Physical Links

The purpose of this section is to identify the physical interface requirements for the various links to the BT network and the customer ACDs (see Figure 4-1 for definition of the interface numbers).

### ICR WAN (I1)

BT will provide connection between the BT-supplied inter-site link and the ICR LAN at each of the Central sites.

BT requires that the supplier  shall provide suitable connectors on the central site systems to allow BT to connect in the inter-site link.

Compliant.

Digital will supply a 10BaseT (UTP Ethernet) port on the DECrepeater connecting the LAN segment used to provide the external WAN connections to the central site system.

### ICR to PG and ICR to Customer AWS (I2)

BT requires that the supplier shall provide the connections at each central site between the ICR LAN and the BT-supplied Frame Relay Managed Network WAN which connects to the customer sites.

Compliant.

Digital shall supply an IP router with a 2 Mbps V.35 interface supporting Frame Relay, see Section 5.5.4 for further details. Please note that this configuration supports up to 32 customers and is sufficient for the initial small configuration planned. Additional interface cards and IP routers may be added as the number of customers increases.

BT requires that the supplier shall provide the connection between the BT-supplied Frame Relay Managed Network WAN and the CPE LAN at the customer site.

Compliant.

Digital shall supply an IP router with a 64 Kbps, V.35 Frame Relay interface, see Section 5.5.4 for further details.

### ICR to BT AWS (I3)

BT will provide the connections at each central site between the ICR and the BT-supplied WAN connections to the BT operations centres.

BT requires that the supplier shall provide suitable connections on the AWS systems supplied to the operations centres.  These systems will be connected to the WAN described above.

Compliant.

Digital will supply 10BaseT (UTP Ethernet) connectors on each AWS.  Digital assumes the LAN in the operational centre will be provided by BT.

### IP Router Interfaces

Please note the following extract from the DECNIS Software Product Description describing the 2 Mbps interface requirements:

The DEC WANcontroller 622 and 622/HS support only clear channel interfaces. Connection to a T1 or G.703 line requires a V.35/RS422 toT1/G.703 DSU/CSU converter to provide framing to meet G.704. The DEC WANcontroller 622 and 622/HS do not provide either T1 framing pulses or separate channels for 64 Kbps control data for G.703.

The DEC WANcontroller 622 and 622/HS use CCITT 114 to clock data out of the DTE and will source a CCITT 113 transmit clock. CCITT 113 should be used by the DCE to receive the transmit data. The DEC WANcontroller 622 and 622/HS implement CCITT 113 as a buffered version of CCITT 114.  This clocking scheme ensures both network synchronisation and minimised skew between the clock and data for a range of cable lengths and parameters.

Please note the following extract from the DECNIS Software Product Description describing the Frame Relay standards supported:

The DECNIS software conforms to the following Frame Relay standards:

- Frame Relay Specification with Extensions document V1.0, September 1990

- Frame Relay Forum's (FRF) User-Network Interface (UNI) agreement

- ANSI T1.617 DSS1 - Signalling Specification for Frame Relay Bearer Service (including the PVC management procedures specified in Annex D)

- ANSI T1.618 DSS1 - Core Aspects of Frame Relay Protocol for use with Frame Relay Bearer Service

- ITU-T Q.922 ISDN Data Link Specification for Frame Mode Bearer Services

- ITU-T Q.933 ISDN Signalling Specification for Frame Mode Bearer Services (including the PVC management procedures specified in Annex A)

### PG to Remote ACD

Support of remote ACD equipment (i.e. PG on one site, ACD on another) is excluded from this release in order to simplify the initial service design and deployment.  BT may wish to provide this capability in the future.

The GeoTel solution is able to offer this capability for Rockwell Galaxy ACDs and other ACD models that connect to Peripheral Gateways via Ethernet TCP/IP LAN.  If this is required, in a future release, the remote ACD link will be provided by BT and details will be designed at that time.

### C7 Links

BT will provide C7 signalling links between the SPRs and the ICR sites.  These links will consist of eight E1 connections per ICR site, each of which has four 64 Kbps channels in use.  Each ICR will be connected to each SPR by two E1 lines, diverse routed.  The connection consists of four link sets:  each link set consists of a pair of channels, one from each physical line.  BT requires the supplier to connect to these links.

The above configuration reflects the maximum signalling interconnection requirements.  This may be reduced for connection to "single cluster" SPRs.

Compliant.

For each site Digital shall supply eight Larscom Orion 400 multiplexers, one connected to each E1 line.  These shall extract the four 64 Kbps channels from each E1 and present them to network interface controllers (NIC) using V.35.  Digital shall also supply thirty two NIC connections grouped into sixteen link sets as described above and shown in Figure 6-3.

The multiplexers shall provide a pair of 75 Ohm BNC connectors to interface to the E1 line.  The transmit and receive “Telecommunications Reference Conductors” (TRCs, also known as “Telecommunications Functional Earths”) will be isolated from protective earth and from each other within the multiplexers.  The E1 signals must be presented to the multiplexers with less than -10dB attenuation.

The multiplexers will implement E1 Common Channel Signalling (CCS) framing.  BT must specify whether CRC-4 shall be used or not and whether they wish to use the national bits, performance monitoring or yellow signal features of the Orion.

BT require that transmit and receive clock for the 64 Kbps channels shall be derived from the timing data in the E1 interface.

Compliant.

### Remote Access

BT requires that the solution shall include at least one modem per physical site (central sites, operational centres and customer sites)  to permit remote access from BT operational centres and supplier support.  BT will supply at least one telephone line per site connected to this modem.

Compliant.

Digital shall supply one BABT approved modem per physical site, the modem will be supplied with a standard UK telephone jack.

### Alarms

During the requirements workshop there was discussion of an electronic interface between the platform hardware and the ASNOU NOMS1 alarm system. This interface would be used to carry alarm information which is not currently generated by the standard solution product. BT sometimes require equipment, such as the multiplexers, to be fitted with power-loss detectors that can feed the NOMS 1 alarm system.

Roger Sutton of BT has stated this is not required.

As this is not a requirement, there has been no effort budgeted for providing additional alarm facilities beyond those supplied with the standard products e.g. SNMP traps.

For your information, the Larscom Orion is fitted with Form C relays and connection block to allow external alarms to be triggered. It offers a power loss signal and a user-defined alarm signal. It may be that these can be used by BT to provide the required alarm for these devices.  No other hardware being supplied by Digital  provides these features: all other alarms will only be reported as software alarms.

## OSS

The implementation should be capable of being integrated into BT’s existing OSS (Operational Support Services) management interfaces and in particular with BT’s ServiceView customer management system.

Non-compliant.

For this initial release it is not possible to meet both this requirement and the time-to-market requirement. Therefore, there will be no attempt to integrate the ICR management tools with BT’s existing ServiceView package.

BT have requested that Digital/GeoTel investigate the possibility of providing modification to the standard product to enable BT to “easily” develop tools required to test database consistency between the BT’s provisioning databases and the ICR database and to reconcile database differences.

Partially compliant.

As the standard GeoTel ICR product supports ODBC access to the central site database, it would be straightforward to develop validation queries that make direct comparisons of ICR data with data from the BT provisioning systems.  However, no such validation is included in the solution currently estimated.  It is assumed that, if required, BT will generate these tools.

As BT will be managing this service on behalf of customers, all customer-platform related alarms should be returned to the BT operations systems and be displayed to the operators. Alarms shall be identified by customer and priority.

Compliant.

Digital/GeoTel will include in the solution the GeoTel CSC support tool known as the “traffic light grid”. The platform alarm input will be communicated to the tool using the planned ICR-PG communication links.

## Administration Centres

Operational support information of differing levels has to be made available to a number of different administration centres.

Compliant.

The current design proposes that this information is made available to AWS systems located at the administration centres using standard product facilities. This will require IP network connectivity between the ICR  and the BT MPRN.

## MCI 800 Gateway

BT require that the ICR platform can also interface to the MCI 800 Gateway, as required for CPR1 and with use on the demonstrator model.

Compliant.

The GeoTel product is in use with this gateway in the US.  So, assuming that the interface is identical to the one used in the US, this is a latent capability which is included in the solution but which has not been accounted for in the dimensioning, or operational training design.  The capability is present and is supported but if it requires any changes to the existing MCI 800 gateway interface code, this will be at extra cost.

# DIMENSIONING

## Capacity

BT require that the delivered system be capable of meeting the key metrics identified in Table 6-1 below. This table shows metrics required for three different sizes of system: small; medium and large. Additional metrics are identified in the following sections.

Table 6-1: Dimensioning Metrics

| Metric | Small | Med | Large |
| --- | --- | --- | --- |
| TRANSLATION CAPACITY |  |  |  |
| Call Screening and BASIC translations (# of dialled numbers) | 100,000 | 250,000 | 500,000 |
| Dialled numbers for RBR Customers (# of dialled numbers) | 700 | 2000 | 4500 |
| CALL HANDLING |  |  |  |
| Busy Second Call Attempts (total call throughput) | 100 | 300 | 600 |
| CUSTOMER |  |  |  |
| Number of customers | 20 | 100 | 350 |
| Number of customer terminals | 200 | 600 | 1200 |
| Peak simultaneous customer access (# of terminals) | 200 | 600 | 1200 |
| CALL CENTRE LINKS |  |  |  |
| Number of CC Connections (assumes 1/CC) | 200 | 600 | 1200 |
| NETWORK AND SYSTEM MANAGEMENT |  |  |  |
| Number of BT Network Management Terminals | 15 | 15 | 15 |
| SERVICE MANAGEMENT AND ADMIN |  |  |  |
| Number of BT Service Management Terminals | 4 | 8 | 10 |
| SIGNALLING TERMINATIONS - BOTH NUP AND INAP |  |  |  |
| # DMSU Supported by NUP | 80 | 100 | 100 |
| # NUP Linksets/SCP | 16 | 16 | 16 |
| # NUP Links/SCPError: Reference source not found | 32 | 32 | 32 |
| # INAP Links/SCPError: Reference source not found | 0 | 0 | 0 |
| OTHER METRICS Pt4 sect 6 |  |  |  |
| Calls millions/year | 150 | 600 | 1500 |
| Call minutes millions/year | 500 | 2000 | 4000 |

Compliant.

The solution offered is based on a combination of central site systems, BT operational support systems and customer-specific components. The two configurations described are designed to meet the call traffic handling metrics for the small and large configurations.

We believe that the required numbers of customers, 20 for the

small and 350 for the large, can be met by the proposed

configurations.  However, the large configuration is based on

assumptions about the projected improvements in hardware

platform performance over the expected roll-out period.

### Central Systems

BT requires that the central system can be deployed in more than one site, for resilience purposes.

Compliant.

The central system will be deployed at TWO BT sites, Cambridge and Leeds offering full redundancy in case of failure at one site. Each system will be capable of supporting the full capacity requirements.

BT requires that the central site can support the metrics defined in Table 6-1.

Compliant.

Based on many years experience of supplying and supporting systems delivered to the call centre marketplace GeoTel staff have proposed a two-stage approach to the implementation of a small system, allowing the modularity to grow to  the large system model, shown below in Figure 6-1 and Figure 6-2.

The increased customer capacity of the new C-ICR systems in Figure 6-2 is an estimate of what will be possible with the improved price/performance of equipment purchased, as the upgrade to a large configuration model, 9-12 months after the initial, small configuration model, equipment is supplied.

Figure 6-1: Small Configuration (Network and Customer ICR only)

Figure 6-2: Large Configuration (Network and Customer ICR Only)

### NIC Configuration

BT requires that the central sites link into their existing signalling network. The GeoTel NICs will be connected into the BT signalling network via C7 links to four BT SPRs. The NICs will be connected via linksets, containing exactly two links, all links in the linkset terminating on the same NIC, with the total number of links still to be defined. For signalling traffic management, BT requires that changeover be supported between links within a linkset but that Forced Rerouting and Controlled Rerouting procedures between signalling links is not required.

Each DMSU switch will have exactly one C7 MTP route to each central site of the GeoTel system. As a consequence, each DMSU will have two distinct MTP addresses to communicate with the GeoTel system and at any given time exactly one of these routes will be active. Whenever the current primary route becomes inactive, traffic will be redirected, by the DMSU, to the other route i.e. the other central site.

Compliant.

The NIC configuration is not shown in the above as it does not change with size of system. This is a result of agreements reached during the Requirements Workshop that the maximum number of links to the CTN would remain at 32 links per site even when supporting the large configuration. To support this number of links, each site will require FOUR NIC systems, each system configured with SIX C7 cards plus TWO NIC system configured with FOUR C7 cards, all links in a linkset will terminate at the same NIC. The NIC systems are proposed as being single processor Prioris HX  PCs - see equipment specification in Table 6-7. To connect the CTN DMSUs to the NICs there will be EIGHT Larscom Orion 400 multiplexers per site.  See Figure 6-3 for C7 signalling link configuration.

NOTE: the C7 signalling link design is still not finalised and there may be further rationalisation and reduction in the numbers of links/linksets required.

The signalling links configuration shown in  omits to show the BT-supplied systems required to multiplex the connections between the DMSUs and the SPRs.

Figure 6-3: Single Site C7 Signalling Links

### BT Operational Systems

BT require systems in their operations centres to manage the capacity defined in Table 6-1.

Compliant.

The BT operational support systems, e.g. those systems used to provision new customers, operate the system and support the system will comprise FIFTEEN Administration Work Stations (AWS). It is currently anticipated that these will connect to the BT MPRN which in turn will be connected via a firewall to the ICR private LAN.  Dimensioning for these AWS will follow the specifications used by GeoTel for the GeoTel Customer Support Center (CSC) AWS systems.

BT also require a total of eight AWS laptop systems capable of modem access, three for their Application Engineers, three for operational support and two for Product marketing. These systems will be used to assist in pre-sales and direct customer support and training.

Compliant.

### Customer Systems

BT require the supplier to provide customer premises equipment to the dimension defined in Table 6-1.

Compliant.

The CPE AWS and PG systems will be standard systems dimensioned according to current GeoTel specifications (and detailed in Section 6.9.4  and Table 6-13).  In addition, an IP Router and a modem will be supplied for each customer site.

## Dimensioning Model

BT require a description of the model used to dimension the system.

Compliant.

It is GeoTel’s experience that, on average, their large customers are currently using 20 ACDs for the their call centre operations and that they are processing up to approximately 30 calls per second.

During the Requirements Workshop, BT raised the fact that they have an internal customer who is using 34 ACDs and is expected to grow to 70 ACDs. GeoTel has suggested that this is an exceptional case and that, purely for the purposes of dimensioning the system requirements, this customer should be accounted for as requiring two times the normal resources (growing to four times).  NOTE: this customer will still only require a single customer license as long as they do not require more than five partitions.

BT require the small configuration for initial deployment and then  growing  to the larger configuration as needed.  The solution proposed is based on initial delivery of an N-ICR comprising two dual-processor Pentium PCs (one at Cambridge and one at Leeds) and a C-ICR based on two quad-processor Pentium PCs (one at Cambridge and one at Leeds.)

The small  configuration N-ICR will handle 150 tps (or cps) and when expanded to quad-processor class machines will handle 300 tps. With the addition of a second N-ICR PC on each site, the large configuration will support 600 tps.

The proposal is that the large configuration infrastructure (racks, comms cabinets and wiring be installed at day one so that evolution to the large configuration is simply a case of adding extra pairs of quad-processor Pentium PCs.  (However, it must be noted that over the course of time, equipment sizes may change. Hence it is impossible to guarantee the total number of cabinets required.) The small configuration will initially handle approx. 20-25 customers on the C-ICR.  Although the original requirements was for a medium configuration supporting 100 customers the requirement is being addressed in providing an initial configuration that will have capacity for one year allowing the extra capacity to be delivered either in the form of cheaper, lower spec PCs or by using similar cost equipment but which would buy extra capacity in a year’s time (cost/CPU and cost/Mbyte curve suggests a doubling in capacity/pair within a year). On this basis we have estimated  that in one year a single pair of similarly priced systems would be capable of supporting an extra 50 customers.

Prior to delivery of the production system, GeoTel will provide algorithms to permit effective determination of a customer's system capacity requirements for both the network and customer ICRs.

## Growth Strategy

BT require that the initially deployed system be dimensioned to support the small configuration allowing for modular growth of the system at BT’s discretion.  However, BT would require that the initial installation of infrastructure, e.g. number of racks, wiring etc., be that required for the large configuration so that the infrastructure will not also need to be modified during the roll out of the large configuration system.

Compliant.

The proposed solution includes initial installation of the “large” configuration infrastructure, whilst only incorporating the “small” configuration number of PCs.

### Scalability

BT require the solution to be scalable to provide BT with the most cost effective deployment up to the large configuration.

Compliant.

The proposed solution is scalable in both the front and back end processing.

Front end processing ( N-ICR) is scalable through the addition of extra processor boards, i.e. converting from dual processors up to quad processors. N-ICR processing can also be augmented by additional PC systems.  This could be done, for example, if the demand for basic translation services should increase beyond the values given in this document.

The number of links into the NIC does not change between the small  and large configuration. If at some point in the future, BT require extra links between the SPRs and the NICs, these may be incorporated through increasing the number of Larscom Orion multiplexers and adding extra NIC units.

A feature of the solution is that both the NIC and the N-ICR are  very basic translation and routing engines with little complexity in processing and little disk I/O.

Backend (C-ICR) scalability is achieved through the addition of extra C-ICR system pairs.

Prior to system delivery GeoTel will determine procedures to “move” a customer,  with a minimum of disruption to the customer, from one C-ICR system to a different one within a central site (to balance load, address exceptional growth in the customer call volume, or to provide additional security). These procedures will permit customer data to be migrated from the operational Customer-ICR (C-ICR)  to the target C-ICR without impact on customer service.  Customer downtime will be limited to the switchover between configured C-ICR instances and will be bounded by one hour.

### Critical Metrics

BT require Digital/GeoTel to supply information on the critical parameters that should be monitored in order to provide indicators for capacity planning.

Compliant.

The primary capacity issues are system real time and disk storage space.  Both of these can be directly assessed through NT and SQL Server tools.  In addition, the system provides very detailed data on response time for routing queries in the Routing Client Five Minute database table.  For each five minute period this table shows the number of queries that timed out as well as the number of queries for which the responses were later than a BT-specified time threshold.

### Tools

BT Require the supplied solution to include tools to assist in performance monitoring and capacity planning.

Compliant.

The ICR comes with a range of reports both standard and custom which can be tailored to deliver capacity planning based on both historical and near-real time data received from the customer ACDs.

GeoTel provide a spreadsheet that calculates customer data storage requirements based upon customer traffic and data retention times.  Sample values are shown in Section 6.4.3.

The Windows NT operating system software includes a powerful and comprehensive “Performance Monitor” utility. This utility allows system administrators to review both current and historical data captured for a wide range of selected objects and platforms. Selected objects include disks, processors, processes, threads, queues, etc. and data recorded includes items such as: percentage CPU utilisation; disk I/O rates; memory usage; queue status; plus many more.  This data may be collected against the local system or across the network.

Performance Monitor offers four views :

- Chart - graphical display or histogram display of selected data

- Alert - display of alerts triggered by user-defined data targets

- Log - displays log entries and files

- Reports - allows configuring and running of user reports

The user can select data for historical recording. Other associated tools include:

- Event Viewer - used to log alerts

- windisk and diskperf - tools that allow configuring of physical and logical disks and enabling statistics collection

The Microsoft Windows NT Server documentation and MCSE training provides guidance on monitoring, tuning and troubleshooting tools. In particular, see:

- Concepts and Planning Guide, Microsoft Windows NT Server, V3.51

- System Guide, Microsoft Windows NT Server, V3.51

for references to “Optimising Performance” and “Performance Monitoring”

NOTE: other non-ICR components, e.g. IP routers, Larscom Orions, provide SNMP agents to collect a range of statistics relating to throughput and performance. These statistics may be collected by a suitable SNMP director. Digital proposes that BT use POLYCENTER Manager on NetView for Windows NT  to collect, display and log data from these components.

### Reporting

BT require the solution to include report generation facilities.

Compliant.

The ICR provides a rich and highly customisable report generation facility. This can be used to generate standard or user-defined reports for performance and capacity planning review purposes.

The Windows NT system is also provided with report generation packages associated with the performance monitoring tools described in Section 6.3.3.

The non-ICR components, e.g. IP routers, Larscom Orions, may be configured to report data through their SNMP director. Digital proposes that this be a POLYCENTER Manager on Netview for Windows NT  system, which can provide performance related reports.

### Capacity Planning and Forecasting

BT require the solution to provide capacity planning and forecasting tools and guidelines.

Partially compliant.

All performance related data (both from the ICR and the Windows NT platform) may be exported into spreadsheets (or documents). Therefore, the data may be used to feed into BT’s capacity planning. The supplied documentation includes guidelines for performance monitoring and capacity planning. Forecasting will be possible through analysis of historical trend data.

Digital will not be supplying a capacity planning system as such.  Digital assume that BT will collate the information described above and perform their own analysis and planning.

## Volumes

This section describes the various capacity metrics within the system.

### IP Routing Bandwidth

The bandwidth and latency requirements for the physical links are addressed in Section 4.3.5, based on the call rates in Table 6-1.  The relationship of the calculated bandwidth requirements to the dimensioning information is also described in that section.

### C7 Signalling Links

The capacity of the C7 links is specified in Section 4.3 and Section 5.5.6.  These were specified by BT’s signalling experts: their relationship to the call load is complex and is not specified here.

### Disk Space

BT require guidelines on required disk space both for the delivered solution and for the provisioning of customers moving to the solution.

Compliant.

Digital/GeoTel will specify appropriate sized disks for the various components delivered.

Details for how to size customer’s CPE are well-described within the current GeoTel documentation (in particular, the Supervisor Guide and the Database Schema Handbook). The most variable component is dependent on call rate (for temporary storage of NRT data), the number and complexity of the call plans, the number of reports and the amount of historical data required to be kept by the customer.

GeoTel’s database sizing utility enables calculation of customer data storage requirements based upon customer traffic and data retention times.  Example input and output values (not BT-specific) from this utility are given in the following tables:

Table 6-2: Example Traffic Metrics

| Traffic Metrics | Count |
| --- | --- |
| Number of Routes | 71 |
| Number of Trunk Groups | 83 |
| Number of Skill Groups | 173 |
| Number of Services | 293 |
| Routing Clients | 6 |
| Calls Routed Per Day (Monthly Avg.) | 67,860 |
| Calls Terminated Per Day (Monthly Avg.) | 114,677 |

Table 6-3: Example Disk Storage Requirements

|  | Daily (MB) | Retention (Days) | Storage (MB) |
| --- | --- | --- | --- |
| Route Call Detail | 12.61 | 390 | 4918 |
| Termination Call Detail | 24.17 | 180 | 4350 |
| Half Hour | 5.86 | 45 | 264 |
| Five Minute (Routing Client) | 0.36 | 45 | 16 |
| Total | 42.63 |  | 9531 |
| Plus 100 MB for Config Data |  |  | 9631 |
| Plus 100 MB for Events |  |  | 9731 |
| Plus 15% |  |  | 11191 |
| Add 100 MB Log Segment |  |  | 11291 |
| Add 25 MB TempDb |  |  | 11316 |
| Total Disk Storage Needed |  |  | 11316 |

In addition to the on-line disk storage, this also allows backup storage requirements to be calculated and for verification that there is enough on-line and backup storage available before provisioning the customer.

NOTE: The customer is responsible for long term storage (including backup) of their statistical data and their reports on their own computer systems.  It is proposed that the BT retention period will be one calendar month or less. Further BT has the option to determine which tables will be retained for this period. The 5-minute summary data, in particular, is quite lengthy and is not referenced in any of the reports.

Both the ICR and Windows NT have extensive capabilities for recording historical data and reports, plus have extensive logging capabilities. It is a policy issue as to how much is kept and how frequently logs are backed up and archived off the machine.

Digital has specified the data storage capacity of the system taking into account BT’s strategy for data storage and backup as defined in the operational requirements document.

### IP Address Space

The solution will require IP addresses to be allocated for the ICR central system, the operations AWS PCs and the CPE connected to the BT ICR network.

Compliant

See Section 4.5.  Note that the number of addresses required is dependent on the number of systems deployed and hence can be related to the equipment volumes in Section 6.9.

### CTN

During the Requirements Workshop a requirement was identified as part of the solution but which affects only BT.  The requirement was that prior to moving a customer from the existing Advanced Services CallCentre service to this new service, NBC should ensure that the CTN is properly provisioned to accommodate the customer calls that will now travel over the CTN and no longer over the DSN.

## Demonstration Model

BT require a demonstration model to be supplied at the earliest opportunity. This model should have the currently shipping version of the GeoTel software installed to test amongst other functionality, the MCI 800 Gateway connectivity. This system is primarily intended as an evaluation tool for BT engineers but may also be used to demonstrate the system capabilities to potential BT customers.

Compliant.

The demonstration model configuration is given in Section 6.9.1.

## Training Model

BT have no requirement for a training model.

## IVV&T Model

BT requires a “model” of the ICR to be used in Integration, Verification, Validation and Test, probably located in the SSTF building at Martlesham Heath. The equipment shall also be used for the BT NUP compliance testing.

Compliant.

The IVV&T model configuration is given in Section 6.9.2.

## Support Model

BT require the solution to include a support model to allow supplier support staff to troubleshoot problems reported against the system and to test new software releases prior to them being supplied to BT. This model will reflect the BT design and network infrastructure, multi-customer platform, C7 BT NUP signalling, and all other developments completed by the supplier for BT.

Compliant

See Section 2.3.6.

## Equipment Specification

BT require the supplier to provide a list of all the components that will comprise the solution.

Compliant.

See sections  below for a list and initial specification for the main components of the proposed solution.  A complete list will be submitted as part of the contract negotiations.

### Demonstration Model

Table 6-4: Demonstration Model Configuration

| Part Number | Description | Qty |
| --- | --- | --- |
| GENERIC CONSOLE |  |  |
| FR-PCSRB-AC | 8 Port Console | 1 |
| FR-PCXCV-SA | 14" 1024 Colour Monitor | 1 |
| ROGGER |  |  |
| FR-A63WW-AX | Prioris ZX 5000 5166 | 2 |
| FR-PCA6U-AD | Additional Processor | 2 |
| FR-PCSBM-AD | 64 MB ECC SIMM (2x32 | 2 |
| FR-PCWVR-AY | 2.1GB Wide Disk | 2 |
| FR-PCXAT-AF | 4/8GB DAT SCSI Tape | 2 |
| FR-DE450-CA | PCI Ethernet Adapter | 4 |
|  | M/S BackOffice Server | 1 |
|  | MS BackOffice V2 Add | 1 |
|  | PC Anywhere for Win | 2 |
| NIC |  |  |
| FR-970WW-A9 | Prioris XL 5133 Kernel | 1 |
| FR-PC77M-AD | 64MB 70 ns parity SIMM | 1 |
| FR-PCXAR-AR | 1Gb Wide SCSI HDD | 1 |
| FR-DE450-CA | PCI Ethernet Adapter | 1 |
|  | MS-DOS V6.22 | 1 |
| PG1 |  |  |
| FR-A60WW-AX | Prioris ZX 5133/1P Kernel | 1 |
| FR-PCSBM-AD | 64 MB ECC SIMM (2x32) | 1 |
| FR-PCWVR-AY | 2.1GB Wide Disk | 1 |
| FR-PCXAT-AF | 4/8GB DAT SCSI Tape | 1 |
| FR-DE450-CA | PCI Ethernet Adapter | 1 |
| Modem | US Robotics V34 Int. | 1 |
|  | Eicon Dual Port X.25 | 1 |
|  | M/S BackOffice Server | 1 |
|  | MS BackOffice V2 Add | 1 |
|  | PC Anywhere for Win | 1 |
| PG2 |  |  |
| FR-970WW-A9 | Prioris XL 5133 Kernel | 1 |
| FR-PC77M-AD | 64MB 70 ns parity SIMM | 1 |
| FR-PCXAR-AR | 1Gb Wide SCSI HDD | 1 |
| FR-PCXAT-AF | 4/8GB DAT SCSI Tape | 1 |
| FR-DE450-CA | PCI Ethernet Adapter | 1 |
| Modem | US Robotics V34 Int. | 1 |
|  | M/S BackOffice Server | 1 |
|  | MS BackOffice V2 Add | 1 |
|  | PC Anywhere for Win | 1 |
| AWS |  |  |
| FR-A60WW-AX | Prioris ZX 5133/1P Kernel | 2 |
| FR-PCSBM-AD | 64 MB ECC SIMM (2x32) | 2 |
| FR-PCT7M-DA | 512Kb VRAM U/G | 2 |
| FR-PCWVR-AY | 2.1GB Wide Disk | 2 |
| FR-PCXAT-AF | 4/8GB DAT SCSI Tape | 2 |
| FR-DE450-CA | PCI Ethernet Adapter | 2 |
| FR-PCXAV-ZB | 21" Digital Monitor | 2 |
|  | M/S BackOffice Server | 1 |
|  | MS BackOffice V2 Add | 2 |
|  | InfoMaker V5 Win 3.1 | 1 |
|  | PC Anywhere for Win | 2 |
|  | POLYCENTER Mgr Netview | 1 |
| ANCILLARY EQUIPMENT |  |  |
|  | 42U Rack Inc Door 4X | 3 |
| DEX2R-FE | RouteAbout Access E/I | 2 |
| DETMI-SE | DEC REPEATER 90TS | 2 |

NOTE: Due to on-site configuration changes this table may not reflect the current configuration.

### IVV&T Model

Table 6-5: IVV&T Model Configuration - Site A

| Part Number | Description | Qty |
| --- | --- | --- |
| GENERIC CONSOLE |  |  |
| FR-PCSRB-AC | 8 Port Console | 1 |
| FR-PCXBV-TD | 15" Monitor | 2 |
| NETWORK ROUTER |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 1 |
| FR-PCA6K-AE | Country Kit | 1 |
| FR-PC77M-BF | 128Mb Memory | 1 |
| FR-PCTAR-BE | Raid 3 | 1 |
| FR-PCTAR-UB | Battery Backup | 1 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 2 |
| FR-PCXLT-AB | Digital Linear Tape | 1 |
| FR-BA356-KB | StorageWorks Shelf | 1 |
| FR-BN31S-1E | SCSI Cables | 1 |
| FR-DE500-XA | Network Card | 1 |
|  | PC Anywhere | 1 |
|  | Virus Guard | 1 |
|  | MS Back Office Server Full | 1 |
|  | MS Back Office Server MLP | 1 |
| NETWORK LOGGER |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 1 |
| FR-PCA6K-AE | Country Kit | 1 |
| FR-PC77M-BG | 128Mb Memory | 1 |
| FR-PCTAR-BE | Raid 3 | 1 |
| FR-PCTAR-UB | Battery Backup | 1 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 6 |
| FR-PCXLT-AB | Digital Linear Tape | 1 |
| FR-BA356-KB | StorageWorks Shelf | 1 |
| FR-BN31S-1E | SCSI Cables | 1 |
| FR-DE500-XA | Network Interface | 1 |
| Modem |  | 1 |
|  | PC Anywhere | 1 |
|  | Virus Guard | 1 |
|  | MS Back Office Server Full | 1 |
|  | MS Back Office Server MLP | 1 |
| CUSTOMER LOGGER 1&2 |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 2 |
| FR-PC93U-AB | Dual Proc. Upgrade | 2 |
| FR-PCA6K-AE | Country Kit | 2 |
| FR-PC77M-BG | 256Mb Memory | 2 |
| FR-PCTAR-BE | Raid 3 | 2 |
| FR-PCTAR-UB | Battery Backup | 2 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 20 |
| FR-PCXLT-AB | Digital Linear Tape | 2 |
| FR-BA356-KB | StorageWorks Shelf | 4 |
| FR-BN31S-1E | SCSI Cables | 4 |
| FR-DE500-XA | Ethernet Card | 2 |
| Modem |  | 2 |
|  | PC Anywhere | 2 |
|  | Virus Guard | 2 |
|  | MS Back Office Server Full | 2 |
|  | MS Back Office Server MLP | 2 |
| CUSTOMER ROUTER 1&2 |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 2 |
| FR-PC93U-AB | Dual Proc. Upgrade | 2 |
| FR-PCA6K-AE | Country Kit | 2 |
| FR-PC77M-BH | 512Mb Memory | 2 |
| FR-PCTAR-BE | Raid 3 | 2 |
| FR-PCTAR-UB | Battery Backup | 2 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 4 |
| FR-PCXLT-AB | Digital Linear Tape | 2 |
| FR-BN31S-1E | SCSI Cables | 2 |
| FR-DE500-XA | Ethernet Card | 2 |
|  | PC Anywhere | 2 |
|  | Virus Guard | 2 |
|  | MS Back Office Server Full | 2 |
|  | MS Back Office Server MLP | 2 |
| NIC PRIORIS HX |  |  |
| FR-882WW-AY | HX 133 Mhz | 2 |
| FR-PC77M-AA | 8Mb Memory | 2 |
| FR-PC88K-AE | Country Kit | 2 |
| FR-PCXVR-AR | 1Gb Disk Drive | 2 |
| NIC Card |  | 8 |
| 4N-APACC-BS | 900VA UPS | 2 |
| FR-DE500-XA | Ethernet Card | 2 |
|  | DOS TCP/IP | 2 |
|  | PC Anywhere | 2 |
|  | Virus Guard | 2 |
|  | MS DOS & Windows |  |
|  | MS Win 95 Upgrade | 2 |
| DECNIS For FRAME RELAY |  |  |
| DNSBA-CC | DECnis 600 SPX Platform | 1 |
| DNSAE-BB | Ethernet Card | 2 |
| DNSCB-AB | V.35 Card | 1 |
| QA-GX7AF-HC | Media and Documentation | 1 |
| QL-GX7A9-AA | Licence | 1 |
| ANCILLARY EQUIPMENT |  |  |
| DMHUB-AE | DEChub with power supply | 1 |
| DEBMP-MA | DECSwitch 900EE | 1 |
| DETMI-AE | DECRepeater 90TS | 1 |
|  | Firewall Equipment and S/W | 1 |
|  | Larscom Orion 400 | 2 |
|  | Communications Cabinets | 5 |

Table 6-6: IVV&T Model Configuration - Site B

| Part Number | Description | Qty |
| --- | --- | --- |
| GENERIC CONSOLE |  |  |
| FR-PCSRB-AC | 8 Port Console | 1 |
| FR-PCXBV-TD | 15" Monitor | 2 |
| NETWORK ROUTER |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 1 |
| FR-PCA6K-AE | Country Kit | 1 |
| FR-PC77M-BF | 128Mb Memory | 1 |
| FR-PCTAR-BE | Raid 3 | 1 |
| FR-PCTAR-UB | Battery Backup | 1 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 2 |
| FR-PCXLT-AB | Digital Linear Tape | 1 |
| FR-BN31S-1E | SCSI Cables | 1 |
| FR-DE500-XA | Ethernet Card | 1 |
|  | PC Anywhere | 1 |
|  | Virus Guard | 1 |
|  | MS Back Office Server Full | 1 |
|  | MS Back Office Server MLP | 1 |
| NETWORK LOGGER |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 1 |
| FR-PCA6K-AE | Country Kit | 1 |
| FR-PC77M-BF | 128Mb Memory | 1 |
| FR-PCTAR-BE | Raid 3 | 1 |
| FR-PCTAR-UB | Battery Backup | 1 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 6 |
| FR-PCXLT-AB | Digital Linear Tape | 1 |
| FR-BA356-KB | StorageWorks Shelf | 1 |
| FR-BN31S-1E | SCSI Cables | 1 |
| FR-DE500-XA | Ethernet Card | 1 |
| Modem |  | 1 |
|  | PC Anywhere | 1 |
|  | Virus Guard | 1 |
|  | MS Back Office Server Full | 1 |
|  | MS Back Office Server MLP | 1 |
| CUSTOMER LOGGER |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 1 |
| FR-PC93U-AB | Dual Proc. Upgrade | 1 |
| FR-PCA6K-AE | Country Kit | 1 |
| FR-PC77M-BG | 256Mb Memory | 1 |
| FR-PCTAR-BE | Raid 3 | 1 |
| FR-PCTAR-UB | Battery Backup | 1 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 8 |
| FR-PCXLT-AB | Digital Linear Tape | 1 |
| FR-BA356-KB | StorageWorks Shelf | 2 |
| FR-BN31S-1E | SCSI Cables | 2 |
| FR-DE500-XA | Ethernet Card | 1 |
| Modem |  | 1 |
|  | PC Anywhere | 1 |
|  | Virus Guard | 1 |
|  | MS Back Office Server Full | 1 |
|  | MS Back Office Server MLP | 1 |
| CUSTOMER ROUTER |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 1 |
| FR-PC93U-AB | Dual Proc. Upgrade | 1 |
| FR-PCA6K-AE | Country Kit | 1 |
| FR-PC77M-BG | 512Mb Memory | 1 |
| FR-PCTAR-BE | Raid 3 | 1 |
| FR-PCTAR-UB | Battery Backup | 1 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 4 |
| FR-PCXLT-AB | Digital Linear Tape | 1 |
| FR-BA356-KB | StorageWorks Shelf | 1 |
| FR-BN31S-1E | SCSI Cables | 1 |
| FR-DE500-XA | Ethernet Card | 1 |
|  | PC Anywhere | 1 |
|  | Virus Guard | 1 |
|  | MS Back Office Server Full | 1 |
|  | MS Back Office Server MLP | 1 |
| NIC PRIORIS HX |  |  |
| FR-882WW-AY | HX 133 Mhz | 1 |
| FR-PC77M-AA | 8Mb Memory | 1 |
| FR-PC88K-AE | Country Kit | 1 |
| FR-PCXVR-AR | 1Gb Disk Drive | 1 |
| NIC Card |  | 1 |
| 4N-APACC-BS | 900VA UPS | 1 |
| FR-DE500-XA | Ethernet Card | 1 |
|  | DOS TCP/IP | 1 |
|  | PC Anywhere | 1 |
|  | Virus Guard | 1 |
|  | MS DOS & Windows |  |
|  | MS Win 95 Upgrade | 1 |
| DECNIS For FRAME RELAY |  |  |
| DNSBA-CC | DECnis 600 SPX Platform | 1 |
| DNSAE-BB | Ethernet Card | 2 |
| DNSCB-AB | V.35 Card | 1 |
| QA-GX7AF-HC | Media and Documentation | 1 |
| QL-GX7A9-AA | Licence | 1 |
| ANCILLARY EQUIPMENT |  |  |
| DMHUB-AE | DEChub with power supply | 1 |
| DEBMP-MA | DECSwitch 900EE | 1 |
| DETMI-AE | DECRepeater | 1 |
|  | 130GB Tape Stacker | 1 |
|  | Larscom Orion 400 | 1 |
|  | Communications Cabinets | 4 |
|  | Cables and fixing kits | 1 |

### Central Site Equipment

#### Small Model Central Site Configuration

Table 6-7: Small Model Central Site Configuration

| Part Number | Description | Qty/Site | Total Qty |
| --- | --- | --- | --- |
| GENERIC CONSOLE |  |  |  |
| FR-PCSRB-AC | 8 Port Console | 4 | 8 |
| FR-PCXBV-TD | 15" Value Monitor (Grey) | 4 | 8 |
| NETWORK ROUTER |  |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 1 | 2 |
| FR-PCA6K-AE | Country Kit | 1 | 2 |
| FR-PC77M-BF | 128Mb Memory | 1 | 2 |
| FR-PCTAR-BE | Raid 3 | 1 | 2 |
| FR-PCTAR-UB | Battery Backup | 1 | 2 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 3 | 6 |
| FR-PCXLT-AB | Digital Linear Tape | 1 | 2 |
| FR-BN31S-1E | SCSI Cables | 1 | 2 |
| FR-DE500-XA | Network card | 1 | 2 |
|  | PC Anywhere | 1 | 2 |
|  | Virus Guard | 1 | 2 |
|  | MS Back Office Server Full | 1 | 2 |
|  | MS Back Office Server MLP | 1 | 2 |
| NETWORK LOGGER |  |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 1 | 2 |
| FR-PCA6K-AE | Country Kit | 1 | 2 |
| FR-PC77M-BF | 128Mb Memory | 1 | 2 |
| FR-PCTAR-BE | Raid 3 | 1 | 2 |
| FR-PCTAR-UB | Battery Backup | 1 | 2 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 8 | 16 |
| FR-PCXLT-AB | Digital Linear Tape | 1 | 2 |
| FR-BA356-KB | StorageWorks Shelf | 1 | 2 |
| FR-BN31S-1E | SCSI Cables | 1 | 2 |
| FR-DE500-XA | Network Card | 1 | 2 |
| Modem |  |  |  |
|  | PC Anywhere | 1 | 2 |
|  | Virus Guard | 1 | 2 |
|  | MS Back Office Server Full | 1 | 2 |
|  | MS Back Office Server MLP | 1 | 2 |
| CUSTOMER LOGGER |  |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 1 | 2 |
| FR-PC93U-AB | Dual Proc. Upgrade | 1 | 2 |
| FR-PCA6K-AE | Country Kit | 1 | 2 |
| FR-PC77M-BG | 256Mb Memory | 1 | 2 |
| FR-PCTAR-BE | Raid 3 | 2 | 4 |
| FR-PCTAR-UB | Battery Backup | 2 | 4 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 20 | 40 |
| FR-PCXLT-AB | Digital Linear Tape | 1 | 2 |
| FR-BA356-KB | StorageWorks Shelf | 2 | 4 |
| FR-BN31S-1E | SCSI Cables | 4 | 8 |
| FR-DE500-XA | Network Card | 4 | 8 |
| Modem |  | 1 | 2 |
|  | PC Anywhere | 1 | 2 |
|  | Virus Guard | 1 | 2 |
|  | MS Back Office Server Full | 1 | 2 |
|  | MS Back Office Server MLP | 1 | 2 |
| CUSTOMER ROUTER |  |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 1 | 2 |
| FR-PC93U-AB | Dual Proc. Upgrade | 1 | 2 |
| FR-PCA6K-AE | Country Kit | 1 | 2 |
| FR-PC77M-BH | 512Mb Memory | 1 | 2 |
| FR-PCTAR-BE | Raid 3 | 1 | 2 |
| FR-PCTAR-UB | Battery Backup | 1 | 2 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 4 | 8 |
| FR-PCXLT-AB | Digital Linear Tape | 1 | 2 |
| FR-BA356-KB | StorageWorks Shelf | 2 | 4 |
| FR-BN31S-1E | SCSI Cables | 2 | 4 |
| FR-DE500-XA | Network Card | 4 | 8 |
|  | PC Anywhere | 1 | 2 |
|  | Virus Guard | 1 | 2 |
|  | MS Back Office Server Full | 1 | 2 |
|  | MS Back Office Server MLP | 1 | 2 |
| NIC PRIORIS HX |  |  |  |
| FR-882WW-AY | HX 133Mhz | 4 | 8 |
| FR-PC77M-AA | 8Mb Memory | 4 | 8 |
| FR-PC88K-AE | Country Kit | 4 | 8 |
| FR-PCXVR-AR | 1Gb Disk Drive | 4 | 8 |
| FR-DE500-XA | Network card | 4 | 8 |
| 4N-APACC-BS | 900VA UPS | 4 | 8 |
|  | DOS TCP/IP | 4 | 8 |
|  | PC Anywhere | 4 | 8 |
|  | Virus Guard | 4 | 8 |
|  | MS DOS & Windows | 4 | 8 |
|  | MS Win 95 Upgrade | 4 | 8 |
| DECNIS For FRAME RELAY |  |  |  |
| DNSBA-CC | DECnis 600 SPX Platform | 2 | 4 |
| DNSAE-BB | Ethernet Card | 4 | 8 |
| DNSCB-AB | V.35 Card | 4 | 8 |
| QA-GX7AF-HC | Media and Documentation | 1 | 2 |
| QL-GX7A9-AA | Licence | 2 | 4 |
| ANCILLARY EQUIPMENT |  |  |  |
| DMHUB-AE | DEChub with power supply | 1 | 2 |
| DEBMP-MA | DECSwitch 900EE | 1 | 2 |
| DETMI-AE | DECRepeater 90TS | 3 | 6 |
|  | 130GB Tape Stacker | 1 | 2 |
|  | Firewall Equipment and S/W | 4 | 8 |
|  | Larscom Orion 400 | 8 | 16 |
|  | Communications Cabinets | 10 | 20 |
|  | Cables and fixing kits | 1 | 1 |

#### Medium Model Central Site Configuration

Table 6-8: Medium Model Central Site Configuration

| Part Number | Description | Qty/Site | Total Qty |
| --- | --- | --- | --- |
| GENERIC CONSOLE |  |  |  |
| FR-PCSRB-AC | 8 Port Console | 4 | 8 |
| FR-PCXBV-TD | 15" Value Monitor (Grey) | 4 | 8 |
| NETWORK ROUTER |  |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 2 | 4 |
| FR-PCA6K-AE | Country Kit | 2 | 4 |
| FR-PC77M-BF | 128Mb Memory | 2 | 4 |
| FR-PCTAR-BE | Raid 3 | 2 | 4 |
| FR-PCTAR-UB | Battery Backup | 2 | 4 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 16 | 32 |
| FR-PCXLT-AB | Digital Linear Tape | 2 | 4 |
| FR-BA356-KB | StorageWorks Shelf | 2 | 4 |
| FR-BN31S-1E | SCSI Cables | 2 | 4 |
| FR-DE500-XA | Network Card | 2 | 4 |
|  | PC Anywhere | 2 | 4 |
|  | Virus Guard | 2 | 4 |
|  | MS Back Office Server Full | 2 | 4 |
|  | MS Back Office Server MLP | 2 | 4 |
| NETWORK LOGGER |  |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 2 | 4 |
| FR-PCA6K-AE | Country Kit | 2 | 4 |
| FR-PC77M-BG | 128Mb Memory | 2 | 4 |
| FR-PCTAR-BE | Raid 3 | 2 | 4 |
| FR-PCTAR-UB | Battery Backup | 2 | 4 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 16 | 32 |
| FR-PCXLT-AB | Digital Linear Tape | 2 | 4 |
| FR-BA356-KB | StorageWorks Shelf | 2 | 4 |
| FR-BN31S-1E | SCSI Cables | 2 | 4 |
| FR-DE500-XA | Network Card | 2 | 4 |
| Modem |  | 2 | 4 |
|  | PC Anywhere | 2 | 4 |
|  | Virus Guard | 2 | 4 |
|  | MS Back Office Server Full | 2 | 4 |
|  | MS Back Office Server MLP | 2 | 4 |
| CUSTOMER LOGGER |  |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 2 | 4 |
| FR-PC93U-AB | Dual Proc. Upgrade | 2 | 4 |
| FR-PCA6K-AE | Country Kit | 2 | 4 |
| FR-PC77M-BG | 256Mb Memory | 2 | 4 |
| FR-PCTAR-BE | Raid 3 | 2 | 4 |
| FR-PCTAR-UB | Battery Backup | 2 | 4 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 32 | 64 |
| FR-PCXLT-AB | Digital Linear Tape | 2 | 4 |
| FR-BA356-KB | StorageWorks Shelf | 8 | 16 |
| FR-BN31S-1E | SCSI Cables | 8 | 16 |
| FR-DE500-XA | Network Card | 2 | 4 |
| Modem |  | 2 | 4 |
|  | PC Anywhere | 2 | 4 |
|  | Virus Guard | 2 | 4 |
|  | MS Back Office Server Full | 2 | 4 |
|  | MS Back Office Server MLP | 2 | 4 |
| CUSTOMER ROUTER |  |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 2 | 4 |
| FR-PC93U-AB | Dual Proc. Upgrade | 2 | 4 |
| FR-PCA6K-AE | Country Kit | 2 | 4 |
| FR-PC77M-BH | 512Mb Memory | 2 | 4 |
| FR-PCTAR-BE | Raid 3 | 2 | 4 |
| FR-PCTAR-UB | Battery Backup | 2 | 4 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 16 | 32 |
| FR-PCXLT-AB | Digital Linear Tape | 2 | 4 |
| FR-BA356-KB | StorageWorks Shelf | 4 | 8 |
| FR-BN31S-1E | SCSI Cables | 4 | 8 |
| FR-DE500-XA | Network Card | 2 | 4 |
|  | PC Anywhere | 2 | 4 |
|  | Virus Guard | 2 | 4 |
|  | MS Back Office Server Full | 2 | 4 |
|  | MS Back Office Server MLP | 2 | 4 |
| NIC PRIORIS HX |  |  |  |
| FR-882WW-AY | HX 133 Mhz | 4 | 8 |
| FR-PC77M-AA | 8Mb Memory | 4 | 8 |
| FR-PC88K-AE | Country Kit | 4 | 8 |
| FR-PCXVR-AR | 1Gb Disk Drive | 4 | 8 |
| FR-DE500-XA | Network Card | 4 | 8 |
| 4N-APACC-BS | 900VA UPS | 4 | 8 |
|  | DOS TCP/IP | 4 | 8 |
|  | PC Anywhere | 4 | 8 |
|  | Virus Guard | 4 | 8 |
|  | MS DOS & Windows |  |  |
|  | MS Win 95 Upgrade | 4 | 8 |
| DECNIS For FRAME RELAY |  |  |  |
| DNSBA-CC | DECnis 600 SPX Platform | 3 | 6 |
| DNSAE-BB | Ethernet Card | 6 | 12 |
| DNSCB-AB | V.35 Card | 6 | 12 |
| QA-GX7AF-HC | Media and Documentation | 1 | 2 |
| QL-GX7A9-AA | Licence | 3 | 6 |
| ANCILLARY EQUIPMENT |  |  |  |
| DMHUB-AE | DEChub with power supply | 1 | 2 |
| DEBMP-MA | DECSwitch 900EE | 1 | 2 |
| DETMI-AE | DECRepeater 90TS | 3 | 6 |
|  | 130GB Tape Stacker | 1 | 2 |
|  | Firewall Equipment and S/W | 4 | 8 |
|  | Larscom Orion 400 | 8 | 16 |
|  | Communications Cabinets | 10 | 20 |
|  | Cables and fixing kits | 1 | 1 |

#### Large Model Central Site Configuration

Table 6-9: Large Model Central Site Configuration

| Part Number | Description | Qty/Site | Total Qty |
| --- | --- | --- | --- |
| GENERIC CONSOLE |  |  |  |
| FR-PCSRB-AC | 8 Port Console | 4 | 8 |
| FR-PCXBV-TD | 15" Value Monitor (Grey) | 4 | 8 |
| NETWORK ROUTER |  |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 4 | 8 |
| FR-PC93U-AB | Dual Proc. Upgrade | 4 | 8 |
| FR-PCA6K-AE | Country Kit | 4 | 8 |
| FR-PC77M-BG | 256Mb Memory | 4 | 8 |
| FR-PCTAR-BE | Raid 3 | 4 | 8 |
| FR-PCTAR-UB | Battery Backup | 4 | 8 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 32 | 64 |
| FR-PCXLT-AB | Digital Linear Tape | 4 | 8 |
| FR-BA356-KB | StorageWorks Shelf | 8 | 16 |
| FR-BN31S-1E | SCSI Cables | 8 | 16 |
| FR-DE500-XA | Network Card | 4 | 8 |
|  | PC Anywhere | 4 | 8 |
|  | Virus Guard | 4 | 8 |
|  | MS Back Office Server Full | 4 | 8 |
|  | MS Back Office Server MLP | 4 | 8 |
| NETWORK LOGGER |  |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 4 | 8 |
| FR-PC93U-AB | Dual Proc. Upgrade | 4 | 8 |
| FR-PCA6K-AE | Country Kit | 4 | 8 |
| FR-PC77M-BG | 256Mb Memory | 4 | 8 |
| FR-PCTAR-BE | Raid 3 | 4 | 8 |
| FR-PCTAR-UB | Battery Backup | 4 | 8 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 32 | 64 |
| FR-PCXLT-AB | Digital Linear Tape | 4 | 8 |
| FR-BA356-KB | StorageWorks Shelf | 8 | 16 |
| FR-BN31S-1E | SCSI Cables | 8 | 16 |
| FR-DE500-XA | Network Card | 4 | 8 |
| Modem |  | 4 | 8 |
|  | PC Anywhere | 4 | 8 |
|  | Virus Guard | 4 | 8 |
|  | MS Back Office Server Full | 4 | 8 |
|  | MS Back Office Server MLP | 4 | 8 |
| CUSTOMER LOGGER |  |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 4 | 8 |
| FR-PC93U-AB | Dual Proc. Upgrade | 4 | 8 |
| FR-PCA6K-AE | Country Kit | 4 | 8 |
| FR-PC77M-BG | 256Mb Memory | 4 | 8 |
| FR-PCTAR-BE | Raid 3 | 4 | 8 |
| FR-PCTAR-UB | Battery Backup | 4 | 8 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 64 | 128 |
| FR-PCXLT-AB | Digital Linear Tape | 4 | 8 |
| FR-BA356-KB | StorageWorks Shelf | 16 | 32 |
| FR-BN31S-1E | SCSI Cables | 16 | 32 |
| FR-DE500-XA | Network Card | 4 | 8 |
| Modem |  | 4 | 8 |
|  | PC Anywhere | 4 | 8 |
|  | Virus Guard | 4 | 8 |
|  | MS Back Office Server Full | 4 | 8 |
|  | MS Back Office Server MLP | 4 | 8 |
| CUSTOMER ROUTER |  |  |  |
| FR-933WW-AX | Dual Proc. 200Mhz ZX | 4 | 8 |
| FR-PC93U-AB | Dual Proc. Upgrade | 4 | 8 |
| FR-PCA6K-AE | Country Kit | 4 | 8 |
| FR-PC77M-BH | 512Mb Memory | 4 | 8 |
| FR-PCTAR-BE | Raid 3 | 4 | 8 |
| FR-PCTAR-UB | Battery Backup | 4 | 8 |
| FR-PCWVR-AZ | 4.2Gb Disk Drives | 32 | 64 |
| FR-PCXLT-AB | Digital Linear Tape | 4 | 8 |
| FR-BA356-KB | StorageWorks Shelf | 8 | 16 |
| FR-BN31S-1E | SCSI Cables | 8 | 16 |
| FR-DE500-XA | Network Card | 4 | 8 |
|  | PC Anywhere | 4 | 8 |
|  | Virus Guard | 4 | 8 |
|  | MS Back Office Server Full | 4 | 8 |
|  | MS Back Office Server MLP | 4 | 8 |
| NIC PRIORIS HX |  |  |  |
| FR-882WW-AY | HX 133 Mhz | 4 | 8 |
| FR-PC77M-AA | 8Mb Memory | 4 | 8 |
| FR-PC88K-AE | Country Kit | 4 | 8 |
| FR-PCXVR-AR | 1Gb Disk Drive | 4 | 8 |
| FR-DE500-XA | Network Card | 4 | 8 |
| 4N-APACC-BS | 900VA UPS | 4 | 8 |
|  | DOS TCP/IP | 4 | 8 |
|  | PC Anywhere | 4 | 8 |
|  | Virus Guard | 4 | 8 |
|  | MS DOS & Windows |  |  |
|  | MS Win 95 Upgrade | 4 | 8 |
| DECNIS For FRAME RELAY |  |  |  |
| DNSBA-CC | DECnis 600 SPX Platform | 6 | 12 |
| DNSAE-BB | Ethernet Card | 12 | 24 |
| DNSCB-AB | V.35 Card | 12 | 24 |
| QA-GX7AF-HC | Media and Documentation | 1 | 2 |
| QL-GX7A9-AA | Licence | 6 | 12 |
| ANCILLARY EQUIPMENT |  |  |  |
| DMHUB-AE | DEChub with power supply | 1 | 2 |
| DEBMP-MA | DECSwitch 900EE | 1 | 2 |
| DETMI-AE | DECRepeater 90TS | 3 | 6 |
|  | 130GB Tape Stacker | 1 | 2 |
|  | Firewall Equipment and S/W | 8 | 16 |
|  | Larscom Orion 400 | 8 | 16 |
|  | Communications Cabinets | 10 | 20 |
|  | Cables and fixing kits | 1 | 1 |

#### Central Site Firewall Configuration

Table 6-10: Central Site Firewall Configuration

| Part Number | Description | Total Qty |
| --- | --- | --- |
| FR-957WW-MH | 32Mb 845Mb disk CD | 1 |
| FR-PC7XR-AA | 1.44Mb drive | 1 |
| FR-PCCAM-AC | 32Mb Memory | 2 |
| FR-DE500-XA | Etherworks card | 2 |
| FR-PC95N-AE | Country kit | 1 |
| FR-PCXAV-ZB | 21" Colour screen | 1 |
|  | PC Anywhere | 1 |
|  | Virus Guard | 1 |
|  | Firewall Software | 1 |
|  | MS Back Office Server MLP | 1 |

### Customer Premise Equipment

Table 6-11: CPE - PG Configuration

| Part Number | Description | Simplex | Duplex |
| --- | --- | --- | --- |
| Base Components |  |  |  |
|  | IP Router - DECbrouter 90 | 1 | 1 |
|  | LAN-in-a-box - DECbridge 90 | 1 | 1 |
|  | 28.8 Kbps modem | 1 | 1 |
|  | Comms Cabs, Cables and Wiring Concentrators |  |  |
| AWS |  |  |  |
|  | Pentium (optional) | 1 | 1 |
|  | 100 MHz single processor |  |  |
|  | 48 MB Memory |  |  |
|  | 500 MB hard drive |  |  |
|  | Internal 3.5” 1.44 MB diskette drive |  |  |
|  | Internal CD-ROM |  |  |
|  | 21” colour monitor, 1024x768 |  |  |
|  | Country kit |  |  |
|  | Ethernet network interface cards |  |  |
|  | Windows NT Server |  |  |
|  | Microsoft BackOffice Server |  |  |
| Peripheral Gateway - Generic |  |  |  |
|  | 100 MHz single processor | 1 | 2 |
|  | 32 MB Memory |  |  |
|  | 500 MB hard drive |  |  |
|  | Internal 3.5” 1.44 MB diskette drive |  |  |
|  | Internal CD-ROM |  |  |
|  | Ethernet Adapters |  |  |
|  | 14” VGA monitor |  |  |
|  | Country kit |  |  |
|  | Windows NT Workstation |  |  |
|  | Microsoft BackOffice Client |  |  |
| NorTel Meridian |  |  |  |
|  | Two available serial lines |  |  |
|  | Digiboard 8-port serial i/f |  |  |
| Rockwell Galaxy |  |  |  |
|  | Eicon single-port X.25 interface card |  |  |

Table 6-12: Customer Administration Workstation

| Part Number | Description | Total Qty |
| --- | --- | --- |
| ADMIN WORKSTATION |  |  |
| FR-A73WW-BE | VentFX 5133,16mb,2.4gb | 15 |
| FR-PCXRN-AE | CD-ROM | 15 |
| FR-PCCAM-BA | 16Mb Memory | 15 |
| FR-PCCAM-BB | 32Mb Memory | 15 |
| FR-DE500-XA | Network Card | 15 |
| FR-PCA7N-AE | Country kit | 15 |
| FR-PCXAV-ZZ | 21" Colour screen | 15 |
| FR-PC83M-BE | 1-2MB VRAM upgrade | 15 |
|  | PC Anywhere | 15 |
|  | Virus Guard | 15 |
|  | MS Back Office Server MLP | 15 |
|  | Infomaker | 15 |
|  | 28.8 Kbps modem |  |
|  | IP router (if required) |  |
|  | Comms Cabs, Cables and Wiring Concentrators |  |

### BT Operations Support Equipment

Table 6-13: BT Operations Administration Work Stations

| Part Number | Description | Total Qty |
| --- | --- | --- |
| ADMIN WORKSTATION |  |  |
| FR-A73WW-BE | VentFX 5133,16mb,2.4gb | 15 |
| FR-PCXRN-AE | CD-ROM | 15 |
| FR-PCCAM-BA | 16Mb Memory | 15 |
| FR-PCCAM-BB | 32Mb Memory | 15 |
| FR-DE500-XA | Network Card | 15 |
| FR-PCA7N-AE | Country kit | 15 |
| FR-PCXAV-ZZ | 21" Colour screen | 15 |
| FR-PC83M-BE | 1-2MB VRAM upgrade | 15 |
| QB-4MTAA-SA | PolyCenter Netview for NT | 2 |
|  | PC Anywhere | 15 |
|  | Virus Guard | 15 |
|  | MS Back Office Server MLP | 15 |
|  | Infomaker | 15 |
| QB-MQDAB-SA | Hubwatch for Windows | 2 |
| LAPTOP ADMIN WORKSTATION |  |  |
| FR-P8HWG-AE | 150MHz, 16MB, 1.3GB, WNT | 8 |
| FR-PCP8M-AC | 16MB x 2 | 16 |
| FR-PCP8D-W2 | 6 Speed Multi-media | 8 |
| FR-PCP8E-AD | Docking Station with NIC | 8 |
| FR-PCP8X-AB | Leather Carrying Case | 8 |
| FR-PCP8H-AE | Batt Charger with AC Adaptor | 8 |
| Modem |  | 8 |
|  | PC Anywhere | 8 |
|  | Virus Guard | 8 |
|  | Windows NT Server | 8 |
|  | Powersoft Infomaker | 8 |
|  | Back Office Server | 8 |

### Environmental Requirements

BT require the supplier to provide information regarding environmental requirements for the solution components.

Compliant.

Digital will provide detailed information regarding power supply and air conditioning requirements for all hardware components supplied.

Digital will also  provide detailed information regarding space requirements for hardware, desk space (for monitors) and documentation once the final hardware list is confirmed.

This action is currently in progress: initial surveys of the Leeds and Cambridge sites have been performed.

## Additional Software

BT require the supplier to provide the means to manage and monitor all the components that comprise the solution.

Compliant.

The ICR and Windows NT platforms are supplied with extensive system management and monitoring facilities.

The Digital offer includes licenses and media for POLYCENTER Manager on NetView for Windows NT and HUBwatch for Windows to system manage and monitor the IP routers, switches, hubs and Larscom Orion multiplexers that are also part of the solution.

# PERFORMANCE

The following section describes the performance requirements of the solution. The categories discussed include:

- Quality of service

- Resilience

- Communications

- Customer Premise Equipment

- Impact to customer ACD

- Contracted performance

## Quality of Service

BT have requirements for the quality of service as defined by:

- Call handling capacity

- Call failure rates

- Plant engaged

- Post-dial delay

- Overload control

- Basic translations

### Call Handling Capacity

BT require that the Intelligent Call Routing platform will be capable of a call handling capacity as identified in Table 6-1.

Compliant.

### Call Failure Rates

BT require that the introduction of the Intelligent Call Routing platform will result in a call failure rate of less than 0.01% over any one day period at up to the rated load specified in Table 6-1. A call is deemed to have failed if no response is sent within 1 second.

Compliant.

### Plant Engaged

BT require that the introduction of the Intelligent Call Routing platform will result in an equipment engaged response for less than 0.01% of calls over any one day period at up to the rated load specified in Table 6-1.

Compliant.

### Post-dial Delay

BT require that the introduction of the call centre handling platform in the signalling path shall result in no more than an extra 250 ms in post-dial delay.

The delay to be measured from the network side of the Digital supplied Larscom Orion 400 multiplexers.

The 250 ms shall exclude the time between the transmission of any ACI request from the platform to the time the response is returned from the network.

Partially compliant

The post-dial delay is measured from the time that responses to all CLI queries are present at the NIC to the time that the Call Dropback message is transmitted by the NIC.  Due to latency considerations within a two-tier N-ICR/C-ICR architecture, the following maximum post-dial delay values are committed in the absence of equipment hardware faults:

- basic maximum post-dial delay values for a call processed by the C-ICR:  350 ms

- additional maximum post-dial delay for external database lookup:  100 ms

Note that external database lookups may take place at the N-ICR level, the C-ICR level or both but that no currently planned services use external database lookups and this feature is not part of the current solution.

### Overload Control

BT require that in the event the call centre handling platform becomes overloaded all calls will have, as a minimum, default routing and no calls should be lost.

Compliant.

It is a standard feature of the solution that if the ICR fails to respond with a call-specific number, the N-ICR will return a standard number translation. Whilst this fails to make use of any intelligence available to the system the call will get through to a call centre and the call will not be dropped by the ICR system.

BT must configure the signalling network such that the number of BT NUP virtual circuits connecting to the ICR limits the possible call arrival rate under overload to not significantly exceed the rate defined in Table 6-1.  I.e., assuming 350 ms per call and 300 calls per second (medium configuration) only 105 virtual circuits should be configured to terminate on the ICR.  For the large configuration 210 virtual circuits should be configured.  The precise number of virtual circuits will be selected based on performance tests carried out during the acceptance test phase to characterise the behaviour of the system under overload.

The ICR will not send call gapping messages.

### Basic Translations

BT require the call handling platform to be able to provide basic translations to provide extra capacity to the existing DSN and IN/P1. The platform shall be able to handle the total call handling traffic identified in  Table 6-1 which includes basic translation services. Any combination of different service types shall be supported within the overall load specified in the table.

Compliant.

Provision of basic translations is via the Network ICR as described in Section 3.11, Basic Translations. No additional software development is required to deliver this service.

## Resilience

BT’s definition of performance also covers adherence to agreed levels of hardware availability and uptime.

### Mean Time Before Failure

BT have a requirement to be provided with information on the Mean Time Before Failure (MTBF) rating for the system being provided.

Compliant.

Digital will provide MTBF figures once the configuration is fixed and firm.

### Mean Time To Repair

BT have a requirement to be provided with information on the Mean Time To Repair (MTTR) rating for the hardware being provided.

Compliant.

Digital will provide MTTR figures once the configuration is fixed and firm.

## Communications

BT require that the performance of all communication links be monitored and reported on.

Compliant.

BT are responsible for monitoring the performance of all BT supplied communication links and systems (e.g. the SPRs).

However, data collected by the ICR may be used to assist in identifying causes of problems, the ICR will raise alarms when communications links are detected as having failed and the AWS displays may be configured to provide visual indications when customer queue lengths grow beyond configurable threshold values.

The Larscom Orion will provide performance statistics via SNMP GET requests to an authorised SNMP director e.g. POLYCENTER Manager on Netview for Windows NT.  It may also be configured to alarm on certain performance related events.

## Customer Premise Equipment

BT require that the Peripheral Gateway has sufficient capacity to handle the call processing rates defined in Table 6-1.

Compliant.

BT require that the customer Admin Workstation has sufficient capacity to generate 5 minute and 30 minute reports with no more than 60 seconds delay from real time.

Partially compliant.

Because BT wish to permit the customer to install and run other applications on the same systems, no performance level can be guaranteed.  Also, the load on the Admin Workstation depends on how many reports are generated and of what type and complexity.  However, if no other applications or reports are running then the Admin Workstation will generate reports with no more than 60 seconds delay from real time, subject to the data requirements of the report itself.

## Impact to Customer ACD

BT require that the connection of the intelligent call routing CPE to the customer ACD will result in minimal impact to the ACD operation.

Compliant.

The GeoTel system can pose an additional load on the supervisory subsystem of an ACD switch.  This is not an issue for either Aspect or Northern Telecom switches.  For AT&T and for Rockwell, it has been necessary in some cases to adjust the refresh rate for the statistics to accommodate a ACD that was already loaded to capacity.

BT has identified that the data collection from the MAX system connected to a Meridian ACD is the subject of a long delay for updates.  BT require that this combination of systems do not impact the near-real time statistics capability of  the GeoTel solution.

Compliant.

GeoTel has confirmed that the MAX system can incur significant internal delays in updating its displays of the Meridian ACD status.  However, the GeoTel system is connected directly to the High-Speed Link that acts as input to MAX, so it incurs no delays associated with the MAX system itself.  For this reason, the GeoTel system receives one-second status updates from the Meridian switch, regardless of the delays visible on MAX.  This has been confirmed in discussions with BT personnel familiar with the operation of MAX.

## Contracted Performance

BT require that any performance metrics included in the solution specification must be capable of being measured.

Compliant.

See the following sections.

### Tools

BT require tools to provide both a monitoring capability (to observe system performance) and tools to tune the system (to maximise performance).

Compliant.

The Digital/GeoTel documentation will describe the various tools (both GeoTel and Windows NT) that may be used to monitor and tune the performance of the system.

The monitoring capability will be provided through a combination of live near-real time data displays plus historical reports. The ICR product comes with extensive near-real time data capture and reporting capabilities allowing customers to generate comprehensive performance reports. These are detailed in the GeoTel product documentation. The Windows NT operating system comes supplied with several standard system monitoring tools which include tools to measure different aspects of system performance. Windows NT Performance Monitor (supplemented with the disk performance monitor and the Event Viewer) provides a comprehensive monitoring capability covering items like %CPU used, disk I/O rates, paging rate down to the levels of processes (and individual threads) both on the local machine and across the network, both for the current status and historical data. See Section 6.3.3 for further details.

The Windows NT documentation provides additional suggestions for metrics to watch and system attributes to adjust to optimise general Windows NT system performance.  Based upon GeoTel's experience to date, specialised tuning of Windows NT should not be necessary.

### Traffic Forecast

BT require tools to assist in capacity planning and forecasting traffic growth.

Partially compliant.

The ICR report package and Windows NT can generate a wide range of standard or custom performance related reports. BT may use these as input to BT’s standard capacity planning tools and procedures.

### Critical Parameters

BT require information on the most critical parameters that should be monitored as giving advance warning of performance-related problems.

Compliant.

The ICR provides detailed data on call routing performance in the Routing Client Five Minute database table.  For each five minute period this table shows the number of queries that timed out as well as the number of queries for which the responses were later than a BT-specifiable time threshold.

The Windows NT documentation provides suggestions on metrics and parameters to watch for early warning of system performance-related problems (see Windows NT V3.51 see “Concepts and Planning Guide” and the “System Guide” under “Optimising Performance” and “Performance Monitoring”).

### Reports

BT require that the solution be capable of generating both standard and custom performance-related reports.

Compliant.

See Section 6.3.4.

# SECURITY

## BT Security Requirements

BT have the following specific security requirements:

- Customer data must be maintained in confidence.

- Ability to build administration user profiles to meet varying levels of access for both System Administrators, network managers and operators.

- Passwords must not be “shared” particularly at the root/superuser level.

- Suppliers should state the security mechanisms employed to prevent unauthorised access by BT and customer users.

Five levels of access are anticipated:

- BT superuser — operating system

- BT systems administration

- BT service management

- End customer administration

- End customer user

Compliant.

NOTE the above is focused on UNIX as the operating system. The operating system for the solution will be Windows NT. Windows NT has a richer and more powerful security mechanism than UNIX and Digital/GeoTel believe that it is fully capable of meeting BT’s security requirements.

The security features of Windows NT are highly configurable and allow users to set-up many different security models and levels of access control both on a specific machine and across the network. The ICR solution is intended to fit within most Windows NT security configurations and imposes minimum constraints on the security environment. The actual security model required by BT for the supplied systems needs to be designed/defined by the BT design team with guidance from the BT Security team.

BT has indicated that there is very little Windows NT deployed within BT and  as yet there are no clear NT-specific security requirements identified.  It was agreed that the BT Security people (Colin Blanchard, Steve Tovey) will investigate the NT Security configuration options  and identify appropriate security guidelines and processes.

Compliant.

The GeoTel security model makes use of standard Windows NT security features.

## Access Levels

### BT Staff

BT require that the implementation provide security access appropriate to the functions of different users, these users include:

- BT NBC — initial provisioning and enrolment of new customers

- BT Operations and Support (ASNOU) — day to day operation of the system and problem investigation and remedial action

- BT Network Management (COU, Oswestry) — traffic monitoring

- BT Capacity Planning group

- Customer — call plan maintenance, report generation and customer data backup

- Digital/GeoTel — problem investigation and remedial action

Compliant.

The current GeoTel  implementation provides four levels of access control identified as:

- GeoTel local supervisor

- GeoTel global supervisor

- GeoTel system manager

- GeoTel administrator

The supervisor role is primarily monitoring either the local system or the global virtual call centre. The system manager is the person who is able to modify the ICR configuration and call plans. The Administrator is simply the standard NT Administrator i.e. there is nothing specific to GeoTel and the role beyond normal administrator functions. This reflects the fact that today the ICR is customer owned and maintained and it is the customer who assigns the administrator for the PC. The role of administrator  will certainly not be something offered to the customer though a subset of the functions may be made available as a pseudo-administrator.

The access levels are defined using the standard Windows NT security features. If BT wish to apply a more refined model this will need to be defined during the design phase.

### Customers

Customers access should be limited to their own customer-specific data and should not include any access to BT configuration data nor permit modification to the telephone number configuration data. Customers should not be able to change any system configuration data, including but not limited to :

- Number-label assignment

- Billing records or configuration

- CPE connection configuration

- Authorised records for AWS users

Compliant.

GeoTel confirms that the multi-customer implementation will maintain complete separation of each customer's data without requiring physical separation of customer data (i.e. customers may share a physical system without compromising data partitioning).

Note: there is no billing data on the ICR.

BT have a requirement to be able to offer customers a fully BT-managed  mode of operation. In this mode, the customer is fully managed by BT and can only modify their routing script and agent skill sets, not any other data.  Customers will normally start off in this mode for some number of weeks until they are familiar with the system.

Compliant

## Node Authentication

When a user logs into the NT AWS, PG, NIC, N-ICR or C-ICR they are required to supply userid and password information. During login, the node, userid and password information is also validated against the Domain primary controller, at which point the user’s network-wide security access is looked up and granted. Failed logins should give no information as to the cause of login failure.

Compliant.

Windows NT provides secure access control by associating access permissions with each user profile invoked when the user logs in. A user may login either as their network identity or as a user restricted to the local machine, the choice is offered during the login process. As a network user, their login information is checked against the domain primary controller database and, if successful, the user is granted a pre-defined  list of access permissions to different network resources. Network access and local machine access are separately configurable e.g. a user may be able to act as administrator on the local machine, but only have access to a subset of customer data on a remote system.

Login failures simply result in a brief message stating login not authorised, with the user being prompted to check their userid and domain and reminding them that the password is case sensitive.

## Domain Model

BT require a security domain model that ensures security control resides with authorised BT personnel. The implementation shall conform to standard Windows NT security practices and use of network access controls.

Compliant.

The standard security configuration for the implementation is based on a single security domain which is centred on the primary domain controller which is one of the Network Logger PCs. For redundancy purposes, the Network Logger at the other site will be designated as a secondary domain controller and will take over primary controller functions in the case of the primary controller not being available. It does not use “trusted domains” but may be configured to do so if BT so decide.

## CLI

BT has a regulatory requirement that any CLI information made available to the implementation should honour the caller’s release requirements, i.e. unreleased CLIs shall be truncated to contain only the exchange or area information. It shall not be possible for any CLI information recorded in any logs available to BT customers or displayed on any screens visible to BT customers  be capable of being used to identify the actual caller if the caller has not released their CLI.

Compliant.

The CLI features to be provided are described in Section 3.5.

## BT Central Site Access

The central site equipment will comprise the NICs, N-ICRs, C-ICRs, ICRs, Loggers and the Larscom Orion multiplexers.  It is a requirement that access to these systems will be restricted to authorised BT operations and support personnel and Digital/GeoTel support personnel only.

These systems will normally be managed by AWS systems located at various BT, Digital and GeoTel sites.  Users of these AWS systems will be subject to access control as defined below.

Access to the systems on-site will be via a locally located AWS. This system will conform to the standard security model applied across the system.

Compliant.

It will be possible to configure the security access to these systems such that no customer has access to anything other than their database on the appropriate C-ICR.  The authorised users will be able to login onto the site-specific AWS and be granted access permissions based on userid, node and password.

For support reasons, access to the Central sites will also be provided via dial-back  modems, the configuration and management of which will be BT’s responsibility.

## AWS

In the event that a user logs off of the AWS the screen should clear and remove all traces of any information from the previous login session.

Compliant.

The Windows NT logoff process clears the screen and displays a predefined customer screen, by default this is the Windows NT logo but this can be replaced with a customer specific logo. Windows NT will be configured to remove the name of the last person to login to the system.

### BT AWS

Access to BT AWS systems will be restricted to identified users registered within BT. Individual users should be given access levels appropriate to their function. Access should be restricted to only those parts of the system necessary, e.g. the level of granularity should be down to individual customer databases.

Compliant.

Windows NT has the flexibility to define access controls down to the level of individual users against individual files. Since the solution makes no special  use of Windows NT security features, it should be possible for BT to define a security policy and configuration to control access at the required level.

Digital has scheduled a joint activity with BT to  determine the number and type of security levels necessary to meet BT’s requirements for internal operations and customer access.  Access to files and database tables can be controlled through the security features of Windows NT and SQL Server.  GeoTel and Digital will use these features to customise system security to BT’s environment.  Any other aspects of access control are beyond the scope of this proposal.

### CPE AWS

CPE AWS systems (hardware and application software) will be owned and managed by BT but normally supplied and supported by Digital. In some circumstances BT’s customers may decide to purchase their own hardware: in this instance ownership, management, supply and support of the hardware shall be the sole responsibility of that customer.

Where BT manage the systems they will be configured to allow customers to add their own users and to assign limited access rights to those users (e.g. add a user with backup privileges). However, access to customer data held on the BT systems will always be controlled by BT in that BT will grant users, identified by responsible persons within the customer domain, with the necessary permissions to connect to and access customer specific data. It will be up to the customer whether they wish to apply a policy of named individuals or if they choose an authorised “role” (e.g. supervisor) to whom they will give passwords to identified personnel.

Compliant.

Access to the customer data is controlled by the Windows NT security domain model which shall be centred on the primary domain controller at one of the central sites. Only network users granted appropriate access by BT operations shall be able to access data held on the BT system. BT has the responsibility  to maintain password protection to Windows NT and all SQL Server engines so that other security features of the system are not compromised.

### Customer Database Access

Customer database information copied to an AWS shall be subject to the same levels of access control as that on the BT central systems. Furthermore, where the AWS is a BT operations AWS the customer-specific database access shall also be restricted to authorised users.

Access by non-standard tools, e.g. other ODBC compliant applications, shall not introduce a security breach where non-authorised users may gain access to confidential information.

Compliant

ODBC application access is based on the underlying security model and features of Windows NT.  Therefore, access levels defined for users of SQL Server access will be equally applied to any other ODBC application trying to access the database.

## PG Management

BT will own management of the PG. There will be no customer login capability and no customer operations  required on this machine .

Compliant.

Digital will supply the PG with software pre-installed and pre-configured. The security access for various users (to be defined during the design phase) will be pre-configured, the administrator password will only be given to the BT responsible authority (to be identified by BT).

Note: as described in Section 8.13, even though the customer has physical access to the PG no security breach of the BT Central Site systems will occur, no other customer data or BT configuration data will be impacted.

## Remote Access

BT will recommend to customers that additional modem and telephone line be installed to allow BT/Digital/GeoTel engineers access to CPE in the case of network communication failure.  This access will be controlled by BT.  The system must be capable of quick reconfiguration to allow BT to enable access to either Digital or GeoTel support engineers. By default this access will be disabled.

To provide reasonable levels of security the modem should be configured for dial-back operation. Provision of additional dial-back numbers (to provide Digital and GeoTel with access) should be limited to authorised BT support engineers.

NOTE: the customer AWS system (hardware and application software) first line support will be by BTNBC Customer Service Centre. This means that these systems will also require modem access,

Compliant.

Digital will supply one modem per physical site (even if the site only contains a single AWS). BT will supply the necessary telephone line. GeoTel and Digital will inform BT ASNOU staff of appropriate dial-back numbers. Access via the modem will be controlled by BT.

## Firewalls

At this stage of the high-level design, there are a number of potential connection points between different systems with differing security models. For example (see Figure 4-1):

- BT ICR private network and the managed network WAN (I2)

- BT CPE and the customer network (I4)

- BT ICR and BT MPRN (I3)

Unless the design significantly changes BT will require firewall systems to be incorporated at interfaces I2 and I3. BT will require that customers be allowed to specify a firewall at interface I4, where their own security procedures require this.

Compliant.

Digital will design, supply and configure firewalls for interfaces I2 and I3 (see Figure 4-2 for detailed firewall locations).

The customer firewall is not included in the menu-pricing options since this might result in complications in terms of liabilities and responsibilities if BT supply the firewall. It is proposed that the CPE equipment will include a connection to which a customer can connect a firewall of their choice, configured by them to meet their security requirements.

## Audit Trails

BT require that all security related activity, e.g. adding new users, changing a users security access, user logins and logoffs, failed logins and process stop and start, be audited.

Compliant.

Windows NT provides extensive information in the security log, specifically covering the information requested.

## User Monitoring

BT require that a log be maintained of all “significant” user activity, e.g. modifications to the system configuration, call plans, generation of reports, copying of customer data to local AWS, backups etc.. It is acceptable for this log to be generated by a mixture of implementation driven logging and Windows NT standard logging, provided documentation is supplied explaining in which log users need to look to find specific information.

Compliant.

The implementation maintains extensive user activity logging information, particularly for call plan development. Windows NT also provides extensive log data on user activity, particularly in the area of backups and system wide activities. Documentation will be supplied describing where users should find specific information.

## Physical Security

Physical access control of equipment on BT sites will be the responsibility of BT.

The CPE shall be provided within a locked cabinet and be designed such that even if the customer gains physical access to the contents of the cabinet or intercepts or modifies internal or external wiring, no security breach of the BT Central Site systems can occur.

Partially compliant.

The CPE cabinet will not be locked (or, if it is locked, the customer will have the key) due to the problem of key management.  However, the system design is such that even if the customer gains physical access to the components or wiring, the only effect can be to disrupt their own service.  They cannot read or modify data beyond that to which they normally have access from their AWS and will not be able to gain access to the BT Central Site systems, other customer data or any BT configuration data.

## Network Routes

The IP Routers located on either the BT central sites or the customer systems must be configured with static routing, no routing protocols (to provide dynamic routing information to the routing tables) will be enabled (or allowed) on these routers.

Partially compliant.

See Section 4.5.

# ACCEPTANCE CRITERIA

The TNRTCAR system will be accepted in a number of stages.  At each stage acceptance testing appropriate to that stage will be demonstrated by Digital and GeoTel to BT.  The following stages are required for testing:

- Demonstration model

- C7 Compliance and Interworking Testing

- ACD Interworking Testing

- Bespoke product enhancements

- Initial deployment of central systems

- Initial deployment of BT operations centre systems (NBC, ASNOU, COU)

- System Acceptance

- Initial installation and configuration of individual customer premise equipment

At each stage, where appropriate, there will be an acceptance test for hardware, including system software e.g. operating system.  This will demonstrate that the hardware has been supplied in good functional order and that all necessary connections have been correctly made.

This will then be followed by a second acceptance test of the configured and operational application software. This will focus on demonstrating the correct functional behaviour of the software.

In the case of the CPE installation, the second acceptance test will be  undertaken to demonstrate to BT’s customer that the system is correctly installed and configured and that the service is acceptable to the customer.

Compliant.

Once an order has been placed for the operational system BT, Digital and GeoTel will prepare a detailed test plan to cover all  the different types of deliverables as identified above. This will be reviewed and agreed with BT.

## Demonstration Model

There is an initial requirement for Digital to supply to a demonstration system (model). This system will be PC-based on which will be installed the current standard GeoTel software with MCI 800 Gateway and connectivity to a Meridian 1 ACD. This system will initially be used as an evaluation and familiarisation system by BT engineers particularly around the MCI 800 Gateway.  To expedite the delivery process this system will be supplied direct to Martlesham Heath.

BT has a requirement for the system to be accepted in a documented manner.

Compliant.

Once the hardware has been racked, configured and the GeoTel software installed there will be a single acceptance test covering both hardware and software.  The level of detail of the test plan will need to be agreed.  However, as this is a “standard” installation of the currently shipping product, it is envisaged that the current GeoTel acceptance test plan will be sufficient.

## C7 Compliance Testing

The supplier will be required to demonstrate compliance to the BT C7 and BT NUP specifications.

Compliant.

Historically BT expect the  supplier to demonstrate compliance to requirements through submission of test scripts and test results. BT only performs a sample of the compliance tests after the supplier’s tests have been completed, BT then  performs interworking tests (IWT). BT do not release test scripts or procedures to suppliers.

However, in the US market-place,  GeoTel have generally received detailed test specifications and scripts from their customers and have then simply demonstrated compliance to them.  GeoTel have explained that their customers usually have specific areas of sensitive functionality that they want to be “thoroughly” tested and that the customer is normally the best person to specify the testing. Moreover, GeoTel have expressed a  concern that no matter how good the BTNR, there are bound to be areas of interpretation, especially around error handling, that are best clarified by accessing the customer test equipment and test scripts. The development timescale estimates supplied by GeoTel were prepared on the assumption of close co-operation with BT for the compliance test activity.

It has been agreed that BT will review their test requirements and facilities to determine if there are ways to co-operatively address these concerns.

The supplier is required to provide BT with a compliance test plan which will be reviewed and agreed with BT prior to commencement of compliance testing. The test plan should include details of proposed test hardware and software to be used both for the CCITT SS7 testing for the MTP layers and the required components of BT NUP. The GeoTel NIC will be provided with two C7 interface cards providing V.35 interfaces to the remaining network equipment.

Compliant.

In order to expedite the creation of the test plan it is proposed that BT make a suitably knowledgeable and responsible person available for one week to review the plan during preparation. This person should have access to the necessary BT personnel to validate that the testing proposed within the plan will satisfy BT’s compliance testing requirements. It is also proposed that where possible, BT make available to Digital/GeoTel copies of available and relevant test scripts (specifically for the BT NUP sections) for inclusion within the test plan. Finally it is imperative that BT provide personnel for prompt resolution of all protocol questions that arise during implementation.

The supplier is required to produce a test results report which will be reviewed and approved by the authorised BT SIRO (or NIRO). BT require that the SIRO, or suitably authorised BT person, be in attendance for parts of the test campaign.

Compliant.

The acceptance criteria for the overall GeoTel product are described in the "BT Signalling Requirements for the SSC7 DMSU to RBR Interface (CPR0), Issue 1 Draft a, 25/7/96, author Dave Smith."  The document describes a seven step acceptance process to validate both the network interface and the GeoTel product itself. The seven steps are:

- Early System Proving (ESP)

- Final System Proving (FSP)

- Network Integration Testing (NIT) at the NIF

- Site acceptance demo

- End to end service testing

- Operational Readiness Tests (ORT)

- Pre-Release Assessment (PRA) / First Office Application (FOA).

Only the first three of these steps are applicable to the validation of the BT NUP interface.  Step 1 will include exercise of individual protocol steps using a protocol simulator developed by GeoTel.  Step 2 will comprise exercise of protocol validation scripts as agreed with BT.  These scripts will include MTP test scripts provided by a commercial protocol tester, as well as BT NUP Call Dropback test scripts prepared by BT.  The Step 2 tests will be performed both on GeoTel premises and as part of final acceptance at BT.  Step 3 will comprise network interworking tests as documented in the following section.

The Acceptance Tests will be performed on the IVV&T test system, which will be delivered to GeoTel for hardware and software configuration three weeks prior to the start of the testing.

BT personnel, in the form of the SIRO, NIRO or other authorised representatives, would be invited to attend the test campaign.

60 days into the BT NUP protocol development, there will be a proof of concept demonstration at BT’s Martlesham Heath facility.  It may be possible to use this occasion also for early-stage testing of the BT NUP implementation against the BT network.  For this to happen, GeoTel would need to receive a description of BT interworking and other tests, so that we can determine which of these could be supported by the implementation at that stage.  This activity is not currently included in the plan, but could be included based upon BT’s perception of value-added.  Schedule impacts would be determined based upon the specifics of the testing.

## C7 Interworking Testing (IWT)

The implementation will be required to undergo interworking test (IWT) by BT engineers to demonstrate satisfactory interworking with existing systems within the BT CTN. Interworking tests will take the form of, but not be limited to:

- Power down/up

- Power down and recovery of the NIC and ICR

- Breaking and re-establishment of links to the SPR/ Larscom Orion 400 multiplexer

- Handling of non Call Dropback BT NUP protocols

- Connection to test CTN switches

Compliant.

It is agreed that the NIC component will be made available for BT engineers to undertake a program of interworking tests against BT test CTN switches.

## ACD Interworking Testing (IWT)

The implementation will be required to undergo interworking test (IWT) by BT engineers to demonstrate satisfactory interworking with a number of captive ACDs, representing those used by the target customer base. Interworking tests will take the form of, but not be limited to:

- power down/up

- power down and recovery of Peripheral Gateway

- Connection to and control of a number of captive ACDs

Compliant.

It is agreed that the ICR will be made available  for BT engineers to undertake a program of interworking tests against BT test ACDs.

## Bespoke Modifications

Note: warranty duration and terms for the bespoke modifications will be negotiated as part of the terms of the service contract.

### BT NUP Support

BT require that the modifications to the standard GeoTel product to support the BT NUP and interconnection with the BT CTN be acceptance tested.

Compliant.

The acceptance test will take the form of the C7 Compliance testing and the C7 Interworking Testing. Upon satisfactory completion of these tests, see Section 9.2 and Section  9.3, the development will be deemed to be accepted.

### CLI Support

BT require that the implemented support for CLI handling is acceptance tested.

Compliant.

This will be demonstrated as part of the C7 compliance testing.

### Destination Number Labels Support

BT require that the implemented solution for destination number labels is acceptance tested.

Compliant.

The acceptance test plan will include specific tests to demonstrate this functionality.

### UK Numbers Support

BT require that the implemented solution for UK number handling be acceptance tested.

Compliant.

The acceptance test plan will include specific tests to demonstrate this functionality.

### Basic Translations Support

BT require that the implemented solution of basic translations will be acceptance tested.

Compliant.

The acceptance test plan will include details of testing for basic translations.

### Multi-customer Support

BT require that the modifications to the standard GeoTel product to provide Multi-customer support be acceptance tested.

Compliant.

The acceptance test plan will include specific tests to demonstrate this functionality.

### Restrictions on Customer Access to Configuration Parameters

BT require that the modifications to the standard GeoTel product to limit customer access to core configuration parameters be acceptance tested.

Compliant.

The acceptance test plan will include specific tests to demonstrate this functionality.

### Restrictions on Alarms Visible to Customers

BT require that the modifications to the standard GeoTel product to limit the system level alarms viewable by customers acceptance tested.

Compliant.

The acceptance test plan will include specific tests to demonstrate this functionality.

### Product Documentation and Help Screens

BT require that supplied documentation and help screens, both Operations Documentation and BT Customer Documentation, is acceptance tested.

Compliant.

The supplied acceptance plan will include the criteria for documentation testing.

## Initial Deployment - Central Sites

The central system will be deployed at two BT sites; Leeds and Cambridge.  However, the hardware and software will be pre-staged by the Digital systems integration group, VIS (Value-added Implementation Service).  During this staging process the hardware will be exercised  and there will be consequential acceptance test, which will be followed by a second acceptance test on site.

BT may require to be present during this staging activity testing. BT will require to review results of the testing.

Compliant.

BT are welcome to visit VIS whilst their equipment is staged.

Once the equipment has been shipped to site and installed there will follow two levels of acceptance.  The hardware will be accepted to be performing as specified and the complete system will be accepted as offering the required service.

### Hardware

BT has a requirement for the hardware to be accepted in a documented manner.

Compliant.

Hardware acceptance will be undertaken immediately after installation, connection and configuration at the final BT site.  The acceptance test plan, including scripts, will be defined, and agreed with BT, during the writing of the Acceptance Test Specification.  Testing will include but not be limited to:

- Bill of Materials (BOM) check, including any requirement for spares

- Installation of racks, wiring and PCs

- Installation and normal operation of Windows NT base system

- Connectivity to Frame Relay Managed Network service, including operation with Firewalls

- Connectivity to MPRN, including operation with Firewalls

- Inter-site connectivity

- Supply of documentation and help screens

NOTE: The testing of normal operation of the Windows NT base system will simply cover that no problems are reported with the installation and that an agreed subset of operations behave as expected

### Software

The application software will be acceptance tested once the hardware has been accepted.

BT has a requirement that the application software be tested in a documented manner which shows that the platform is ready for service.

Compliant.

The testing will focus on demonstrating the correct operation of the software under normal and abnormal conditions. Tests will include, but not be limited to:

- NIC interaction with CTN via C7 signalling links

- WAN connectivity and Frame Relay support

- Operation of multi-customer system (using dummy databases for more than two customers) - configuration, provisioning, statistics collection, reports, trouble-shooting

- Performance (e.g. pass 400 messages, statistics OK, throughput OK), grade of service

- Alarm generation and handling

- Handling of test calls

## Initial Deployment – BT Operational Centres

The requirement is for criteria to demonstrate that the supplied AWS systems in the operations centres are correctly installed, connected and configured.

Compliant.

### Hardware

Each supplied AWS will be subject to a hardware acceptance test plan that includes but is not limited to :

- BOM check

- All connectors supplied and correctly fitted

- Base software (base system and Comms) installed and configured

- Access to network and ICR systems

- Compliant.

NOTE: The testing of normal operation of the Windows NT base system will simply cover that no problems are reported with the installation and that an agreed subset of operations behave as expected

### Software

The initial supplied AWS will undergo an extensive acceptance test to demonstrate:

- Applications software correctly installed and configured

- Connectivity to ICR components and databases

- Alarms are correctly displayed

- Security access is properly configured, only access to authorised databases and components permitted

- Real time data properly captured and displayed

- Display of and modification of call plan database

- Generation and printing of standard reports

Subsequent AWS equipment will be subjected to an agreed subset of the acceptance criteria, this should include :

- Application software correctly installed and configured

- Connectivity to ICR components

- Subset of alarms displayed correctly

- Security access is properly configured

Compliant.

## Operational Readiness Testing

BT require that the solution be subject to an Operational Readiness Test (ORT). This test will demonstrate that the system, the service and the associated operational procedures are functioning correctly and are ready for production use.

Compliant.

It is proposed that BT will enter an operational readiness testing period. The ORT will commence upon successful acceptance testing of the central site and BT ASNOU systems and an actual or simulated customer site. The ORT will examine the entire process of delivering the service to a customer. It will include:

- Process by which NBC gather customer requirements, place order for CPE systems, provision the C-ICR databases and configure the N-ICR

- Delivery of and acceptance testing of CPE systems

- Customer training

- Customer configuration of call plans, reports and alarms

- Customer operation of the live system

- Operational procedures to support and maintain the system during sustained operations

- Problem reporting, troubleshooting and escalation procedures

Preparation and performance of the ORT will be the responsibility of BT. Digital/GeoTel will provide input to the ORT plan and will support the Technical Evaluation by BT customers.

## System Acceptance Test

BT require that following the completion of the ORT that there be a further period of production operation of the service to demonstrate sustained operation of the system and to provide the BT operators with an extended handover period.

Compliant

Digital/GeoTel will provide a suitably qualified and experienced ICR engineer to support BT operations staff for a period of three months. During this period any problems will be dealt with under the normal escalation procedures.

## Customer Premise Equipment

BT require that the CPE supplied be acceptance tested to demonstrate:

- All ordered components are supplied

- Systems are properly installed, configured and connected

- Connectivity to Frame Relay Managed Network is established

- Connectivity to customer network (where applicable) is established

Compliant.

All components will be pre-staged at a Digital or GeoTel facility to ensure all supplied components are operational. Once tested in-house, the equipment will be shipped to the customer site and installed by Digital’s Value-Added Implementation Services. Once installed, configured and connected, the components will be subjected to an AGREED acceptance test plan.

NOTE: The testing of normal operation of the Windows NT base system will simply cover that no problems are reported with the installation and that an agreed subset of operations behave as expected.

Customer Premises Equipment will be exercised against an ICR test application at Digital’s staging facility.  This test application will be compatible with test applications loaded on BT’s N-ICR and C-ICR.  In this way, application-level interactions will be tested prior to system installation at the customer site.

### Hardware

BT require that the CPE supplied be acceptance tested to demonstrate:

- All ordered components are supplied

- Hardware components are properly installed, configured and connected

- Connectivity to Frame Relay Managed Network is established

- Connectivity to customer network (where applicable) is established

- Where the CPE is a PG system, that the PG is properly connected to the customer ACD equipment

- The software is properly installed

- The software is properly configured

- The BT Customer ICR system is correctly provisioned for the CPE

- Where the CPE is a PG system, that ACD data is properly and accurately collected and forwarded to the C-ICR, and can be seen at a selected AWS

- Where the CPE includes an AWS, that the AWS can access the correct C-ICR databases, collect live data (where available), generate reports, set and display alarms and modify call plans.

Compliant.

All components will be pre-staged at a Digital or GeoTel facility to ensure all supplied components are operational. Once tested in-house, the equipment will be shipped to the customer site and installed by Digital’s Value-Added Implementation Services. Once installed, configured and connected, the components will be subjected to an AGREED acceptance test plan. Acceptance Test Plan to be prepared by Digital/GeoTel and reviewed and agreed by BT.

NOTE: The testing of normal operation of the Windows NT base system will simply cover that no problems are reported with the installation and that an agreed subset of operations behave as expected

### Service

BT require that once the all the customer-specific CPE has been acceptance tested, the SERVICE will be subject to an acceptance test to demonstrate:

- That the complete system is correctly and fully installed, configured and operational

- Customer data can be collected, displayed and configurable features modified against the C-ICR

- The system is working to agreed service levels and performance metrics

The test will include an extended period of live operation to demonstrate sustained performance and quality of service.

Compliant.

This is a requirement against BT rather than against the solution but is included for completeness. Digital/GeoTel will provide input to the preparation of the service acceptance test plan but BT will be responsible for defining the test plan and for negotiating elements of it with individual customers.

## IVV&T Model

BT require that the supplied IVV&T model be acceptance tested as per the demonstration model, see Section 9.1.

Compliant.

# OPERATIONS AND SUPPORT

## Roles and Responsibilities

BT requires the following principles regarding roles and responsibilities:

- BT owns all equipment (internal to BT and CPE, including customer AWS systems).

- BT manages (including data) on central site and operations centre equipment and on PG systems.

- BT manages customer AWS systems but is not responsible for data in user directories, including templates for customer-specified reports or alarm filters (customer must back up or store on another system as required).  BT maintains 1 month of historical data — longer term archives must be maintained by the customer (by exporting from the BT databases to a database on another machine).

- Digital performs all hardware maintenance on BT and CPE equipment.

- Digital and GeoTel provide support for BT operations staff with application, software and hardware faults

Compliant.

Digital will perform hardware repair and maintenance and will be the escalation route for all application, software and hardware faults.  GeoTel will provide application support, including guidelines for determining customer data requirements, see Section 6.4.3 .  Digital will provide hardware support for customer data storage.

BT have service metrics which require that hardware faults on the ICR hardware shall be fixed within those times specified in the SRS.  The overall intention is to improve service times to guarantee a 2 hour fix to customer affecting problems, therefore the support should be based on mission critical to achieve speed of repair.

Compliant

### BT Operations

BT requires to be able to maintain and restore the service using NBC Ops and ASNOU operations staff.  These staff will be trained on the platform to a fairly high skill level but will not necessarily have call centre experience.

Compliant.

The alarm handling and management tools for first line support users permit fault isolation and restoration and the training for these staff will provide information on how to restore around common faults.  Higher skilled users (e.g. Application Engineers) will be required to fix problems involving interaction with customer ACD equipment.

### Digital Support

BT require that Digital provide hardware and software support in line with existing NACC support arrangements although, with mission critical response and repair times as specified in the SRS document.

Compliant

### GeoTel Support

BT requires that for a period to be determined (but expected to be the first 3 months), GeoTel shall provide an experienced support engineer to work on-site at the ASNOU (on-site during office hours, on-call out of hours).

Compliant.

GeoTel shall provide an experienced support engineer on-site for 3 months.

BT requires that for a period to be determined (but expected to be the first 6 months), GeoTel will provide experienced Application Engineers to support the BT Application Engineers.

Compliant.

GeoTel shall provide 1 experienced Application Engineer full-time, in the UK, for 6 months  to support the BT Application Engineers.

## Response Times

The overall BT requirement is to restore service to a customer (for Priority 1 faults) within 2 hours.  In order to meet that response time, BT will automatically raise a priority 1 call with the supplier within 30 minutes of the report of the outage if the problem is not understood.  This permits the supplier to be involved in case the problem turns out to be with the supplier. To facilitate response to Priority 1 faults, GeoTel and Digital will be notified in parallel in these cases.  All other faults will be directed to Digital, which will involve GeoTel as necessary.

BT requires that the supplier shall respond (e.g. call back) to a priority 1 call within 10 minutes.

Compliant

BT requires that the supplier shall have restored service to the customer within 90 minutes of the priority 1 call.

Compliant

Note that this time is measured until the customer service is restored, repair may take longer and during that time there may be exposure to a risk of a multiple failure.

## Quality of Service

See the SRS document and Section 7.1.

Compliant.

## Tools

BT requires that the solution shall include the operational tools to permit alarm handling, efficient test and diagnosis and capacity planning.

Compliant.

Digital will supply the following tools:

- GeoTel AWS for management, reporting, alarm handling, test and diagnosis of the GeoTel application

- GeoTel “traffic light grid” (see Alarms, below)

- Microsoft System Management Server for remote management of all Windows NT systems

- POLYCENTER Manager on NetView for Windows NT for integration of SNMP alarm feeds

- Windows NT Performance Monitor utility (see Section 6.3.3 and Section 7.6.1).

- Tools and procedures to move a customer from one C-ICR to another.  This procedure must be a simple operational procedure usable remotely from the operations centres (without an on-site visit) by ASNOU staff. These procedures will permit customer data to be migrated from the operational C-ICR to the target C-ICR without impact on customer service.  Customer downtime will be limited to the switchover between configured C-ICR instances and will be bounded by one hour.  Also, historical statistics for the day of the move may be impacted.  GeoTel will document impacted dynamic routing elements and historical statistics when the procedures have been designed.

## Escalation

BT requires Digital to provide an escalation process.

Compliant.

All calls will be escalated to the Digital PACE desk by designated BT operations staff. Priority 1 calls will always automatically be escalated from Digital to GeoTel immediately. If a Priority 1 application problem is suspected this will be escalated to Digital and GeoTel in parallel to achieve the two hour restore time.

If BT desire they can log calls to the Digital PACE desk electronically using Infoman.

### Fault Prioritisation

BT require the ability to decide the priority of a call when it is logged.

Partially compliant.

BT can decide the priority of the call.  However, no repair of CPE can be guaranteed within the 90 minute window for priority 1 faults so all CPE hardware faults will be limited to priority 2 or below.

### Third-party Suppliers

BT requires that Digital supports all components of the solution, including those provided by third parties.

Compliant

Digital shall support all hardware and software which it supplies, including those sourced from third parties (e.g. Microsoft).  All support calls will be logged with Digital.

### Documentation

BT require that the suppliers provide a support plan documenting support and escalation processes and commitments.

Compliant.

Digital will supply a support plan.

### Conference Calls

BT require that priority 1 support calls are managed by a BT support manager using conference calls of relevant parties.  BT require that the suppliers participate in the these conference calls.

Compliant.

### Metrics and Reporting

BT require regular reports showing supplier support performance metrics, as detailed in the SRS and as supplied under the NACC support contract.

TBA.

Digital’s PACE desk produce monthly reports showing analysis of calls for other solutions.  Digital is happy to discuss providing a similar service for this solution.

## Remote Access

BT has taken GeoTel’s advice and requires that BT operations centres and supplier support can access the equipment even if the IP connectivity has failed.

Compliant.

The solution shall include at least one modem per physical site (central sites, operational centres and customer sites)  to permit remote access from BT operational centres and supplier support.  BT must supply at least one telephone line per site connected to this modem.  BT will also be responsible for managing user accounts with appropriate security authorisations for the supplier support personnel to perform support activities.

Remote access will always be required by GeoTel when a Priority 1 call is logged with them.  Any delay in BT providing that access will not be included in the agreed restoration time (e.g. 90 minutes or whatever is agreed).

## Trouble-ticketing

BT require trouble-ticketing systems.

BT to supply.  Digital and GeoTel are not supplying a trouble-ticketing system.

## Alarms

BT require access to all alarms, with customers only permitted access to a subset of alarms.

Compliant.

The product shall be modified to only permit the customer to have access to a defined subset of alarms (the subset will be defined by GeoTel, in conjunction with BT, during the project).

BT require rapid and effective alarm handling.

Compliant.

The solution shall include a “traffic light grid” tool to summarise the status (Red, Amber, Green) of each customer on a single screen.  This tool will be deployed at one or more operational centres.

Digital and GeoTel shall propose recommended alarm filters during the BT operational training.

The GeoTel alarms accessible to SNMP are listed in Appendix A of the Administrator Guide.

### Surveillance

BT have a requirement that all equipment can be monitored and will generate operator alarms if they fail.

Partially compliant.

All equipment (i.e. that includes multiplexers, IP routers, hubs as well as PCs on BT and customer sites and customer AWS systems) shall be “pinged” from NetView.  NetView shall generate an alarm if connectivity is lost (which may be a communications problem or a fault with the target equipment).  Note that in the case of the customer AWS this may be a normal situation (e.g. customer powers off the equipment at night).

In addition, the operations centre AWS systems shall provide visual indications they lose contact with the central ICR systems and the ICR systems shall generate alarms if they lose contact with each other or with the CPE PG systems (but not the AWS systems).

### Typical Failure Scenarios

BT require to perform a complete failure mode analysis on the solution.

Compliant.

Digital and GeoTel will attend a workshop with BT to perform the failure mode analysis. The time, location and duration of this workshop is to be determined by all parties.

### Customer Alarms

Customers may choose to alarm on items that are nothing to do with the performance of the platform (e.g. on call routing to a non-configured skill group).  For that reason, BT will not be automatically notified when a customer alarm is raised.  However, all platform faults are expected to cause alarms which BT will be monitoring.

Compliant

## Training

BT require that operations training shall include training on failure cases (e.g. induce a failure, see what alarms are produced, debug failure).

Compliant.

## Routines

BT requires that the supplier shall document “routine” operational procedures.

Compliant.

Routine procedures are documented  in the GeoTel System Manager’s manual already reviewed by BT.

## S/W and H/W Upgrades

BT requires that hardware and software upgrades shall be performed without disruption to customer service.

Compliant.

However, it is possible that in some cases a version incompatibility could require a switchover from one side to the other during the upgrade process.  System downtime in such a case would be bounded by 3 minutes. This may result in a short period of time when the full capability of the system is not available and default translations (at the NICs) will occur.

BT requires that the upgrade process be documented.

Compliant.

The upgrade process shall be defined in the release notes accompanying the upgrade.

### Build Control

BT requires that build levels are documented.

Partially compliant.

Software and hardware build levels and serial numbers shall be supplied with all equipment supplied and with all upgrades.  However, BT is to supply a process and system for maintaining the build level information and making sure that it is up to date.

Software upgrades shall include revision control information (including compatible revision levels of related software) in the release notes accompanying the upgrade.

## Design Support for Bespoke Systems

BT requires that design support (e.g. bug fixing) for the bespoke software shall align with BTs standard practice and be included in the solution capital price.

To be agreed.

Digital and GeoTel will include support  for fixing non-compliance of the bespoke software to the requirements in the overall capital price.  Note that this does not include future enhancements or changes due to changes in the requirements (e.g. new versions of BT NUP).

Note: warranty duration and terms for the bespoke modifications will be negotiated as part of the terms of the service contract.

## Disaster Planning and Fallback

BT will consider a disaster planning methodology.  The following principles will apply:

- There will be no need for another “fallback” ICR site.

- The operational centres will back each other up so there will be no need for additional equipment for operational centre fallback.

BT require that the supplier shall use “best endeavours” to assist BT in recovering after a disaster (e.g. losing one of Cambridge or Leeds).

Compliant.

## Backups

BT require that ICR data is backed up and can be restored remotely (i.e. without a site visit).

Compliant.

All backups of ICR data will be performed to locally attached stacker tape drives.  Operators may initiate backup operations remotely but the backup data does not pass over any network.

Operators may initiate restore operations remotely as long as the system to be restored is still running and the data is on the tape currently inserted in the drive.  Otherwise, restores will involve a site visit.

Customer owned data (including customer reports and historical call routing data beyond 1 month old) will be the responsibility of the customer to back up if they wish.  That data will be stored on the customer AWS systems and can be backed up by the customer over their network connection to the AWS.  No customer backup tools are supplied with the system as it is assumed customers will have their own standard tools.

# TRAINING

## BT Requirements

There are three types of  people that a training requirement has been identified for:

- internal BT people who will manage and support the platform;

- internal BT Application Engineers who will work with customers to define requirements and manage with the implementation process;

- customers.

For internal people BT has identified a requirement for training in a number of areas and covering a range of topics, as follows:

- System Appreciation

- System Management

- Platform Problem Management

- Network Configuration of Platform Elements

- Performance Measurement

- Planning and Capacity Management

- Security Access and Administration

- Platform Internal Operation

The suitability of the above topics is highlighted in Table 11-1, the Training Matrix.

The training requirements of Application Engineers and customers has yet to be clearly defined.  However, it is likely that much of the support training can be re-used by Application Engineers and customers.

In addition BT has identified the following requirements:

- Initial training shall be linked to the overall Project Plan.

- Compliant.

- All training supplied shall be acceptable to, and agreed with BT prior to delivery, and should be suitable with respect to; objectives, content, duration, skill level, type of skill, location, cost, timescales, numbers, safety and prerequisites.

- Compliant.

- Courses shall be supported by lectures and documentation.  Any course which includes practical elements shall have at least 50% hands-on.  “Hands-on” means that 50% of the course work should be practical and this work should be on an individual basis.

- Compliant.

- BT require that all training courses be held in the UK.

- Partially compliant.

- The majority, if not all, of the application training will done on GeoTel’s premises in Littleton, Massachusetts.

- The supplier shall provide and operate an Issue Control mechanism to deal with amendments and updates to documentation.

- Compliant.

- The supplier shall ensure that all documents and updates have a controlled distribution to agreed named BT distribution points.

- Compliant.

- The supplier shall grant BT the right for any purposes to reproduce, have reproduced, have modified, amend, have amended and use free of charge all documentation and information supplied by or to be supplied by the Supplier.  (Any disclosure to a third party shall require the Supplier’s approval.)

- Partially compliant.

- Subject to a review on a case by case basis, Digital will grant BT the right to reproduce, have reproduced, have modified, amend, have amended and  use free of charge all documentation and information supplied by the Supplier.

## Digital/GeoTel Solution

To meet BT’s training requirements will require three types of courses; standard Digital, standard GeoTel and specifically developed courses.  A large part of the standard training requirement will involve BT operations staff becoming trained-up on Windows NT.  Digital would recommend that BT aim to get their people trained as Microsoft Certified System Engineers (MCSE).  There are two main advantages to training BT staff to MCSE standard:

- The MCSE qualification is rapidly becoming a de-facto standard for Microsoft products and technologies.

- To obtain an MCSE six exams have to be passed, four core and two elective, this helps to concentrate the mind of the attendee when attending the various courses.  Also, it can be a good measure for the business of the suitability of various candidates to perform the job.

Of the available courses Digital has chosen those detailed in the table below as the most appropriate.  Please see course documentation previously supplied. The following table summarises the overall course requirement that Digital and GeoTel  would recommend to meet BT’s immediate requirements.

Table 11-1: Training Matrix

| Course Description | Course | Days | Suitability | Topics | Type |
| --- | --- | --- | --- | --- | --- |
| General |  |  |  |  |  |
| Appreciation Course |  | 1 | OPS, AE | 1 | DDEC |
| Windows NT |  |  |  |  |  |
| Supporting MS Windows 95 | 540 | 5 | OPS | 2,3 | SDEC |
| Supporting MS Windows for Workgroups 3.11 | 315 | 3 | OPS | 2,3 | SDEC |
| Supporting MS Windows NT 3.51 | 505 | 5 | OPS | 2,3 | SDEC |
| Supporting MS Windows NT Server 3.51 | 659 | 5 | OPS | 2,3 | SDEC |
| Supporting MS Systems Management Server (SMS) v1.2 | 732 | 5 | OPS | 2,3 | SDEC |
| System Admin for MS SQL Server for MS Windows NT | 756 | 5 | OPS | 2,3 | SDEC |
| Internetworking MS TCP/IP on MS Windows NT 3.51 | 472 | 4 | OPS | 2,3 | SDEC |
| Network Management |  |  |  |  |  |
| Managing the TNRTCAR Network |  | 5 | OPS | 2,3,4 | DDEC |
| Performance Management |  |  |  |  |  |
| Performance Management Principles |  | 2 | OPS | 5 | DDEC |
| Capacity Planning |  | 2 | OPS | 6 | DDEC |
| GeoTel Application |  |  |  |  |  |
| GeoTel Customer Course |  | 5 | AE, CUST |  | SGEO |
| GeoTel Application Management Course |  | 3 | OPS, AE | 7,8 | DGEO |

OPS means BT staff who will have responsibility for the management of and resolution of issues associated with the platform.  AE means Application Engineer and CUST means BT customers.

The type codes indicate whether the course is a standard portfolio type course or specifically developed for the TNRTCAR project; SDEC - standard Digital course, DDEC - developed Digital course, SGEO - standard GeoTel course, DGEO - developed GeoTel course.

Additional training for Support Engineers (SEs) and Application Engineers (AEs) will be provided through staff training assignments with GeoTel operations in the US.  The objective of these assignments is to train a core of BT staff who participate in training others.  GeoTel will also assign personnel on-site in the UK to work with BT people until they become adept in the SE and AE roles.  GeoTel personnel will be required in these roles for at least 3-6 months.

Once BT has placed an order for the TNRTCAR system and identified the operations staff and Application Engineers who will be involved with the project Digital will be happy to discuss individual training needs based upon the past experience of the individuals concerned.  Furthermore, at this stage, the course content for specifically developed courses will be discussed and agreed.

# PROVISIONING

The provisioning  process is concerned with how BT sell, implement and support the TNRTCAR system to internal and external customers.  The following diagram outlines the basic process.  Clearly, within each activity there are numerous subsidiary tasks and some of these will be different dependent upon whether the customer is internal or external.

|  |  |  |  | Pre-sales Activity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Capture Customer Requirements |  |  |  |  |  |  |
|  |  |  |  | Customer Order |  |  |  |  |  |  |
|  | Place Customer on BT systems |  |  | Order Frame Relay |  |  | Third- Party Orders |  |  |  |
|  |  |  |  | Deliver Equipment & Services |  |  |  |  |  |  |
|  |  |  |  | BT Acceptance |  |  |  |  |  |  |
|  |  |  |  | Customer Acceptance |  |  |  |  |  |  |
|  |  |  |  | Customer Training and Migration |  |  |  |  |  |  |
|  |  |  |  | Customer “Hand-holding” |  |  |  |  |  |  |
|  |  |  |  | On-going Support |  |  |  |  |  |  |

Figure 12-1: Provisioning Workflow Diagram

There is a requirement for the provisioning process to be owned and managed by a single organisation.  BT will own this process and will use other organisations such as Digital and GeoTel to handle specific tasks on an as-needed basis.  Moreover, BT will use the GeoTel model for implementing new customers, adapted to suit their own individual needs.  The GeoTel approach relies heavily on Application Engineers - people experienced in the Call Centre market-place who are totally responsible for the end-to-end management of the new installation for the majority of activities from Customer Requirements Capture onwards.

## Pre-sales Activities

There will be a range of pre-sales activities such as; mailshots, demonstrations, understanding business requirements and reference site visits.  These will be the sole responsibility of BT, although in the very early life of the platform it may be necessary to make use of GeoTel’s customers and experience.  These activities will be the responsibility of the Product Line and the individual BT Account Managers.

One of the pre-sales activities maybe to offer the customer a “Stats-only” service to demonstrate what the system can provide.

## Capturing Customer Requirements

Capturing customer requirements will be the job of the BT Application Engineer and will fall into a number of areas:

- business

- environmental/location

- call plan design

- technical

- migration planning

- training

- support

GeoTel Application Engineers will take primary responsibility for the first two customer installations and will train BT personnel in these activities.  The need for longer-term pre-sales co-operation remains to be determined.

To assist in the data collection for these activities a number of documents will be available, particularly menu-pricing sheets, training schedules and the existing GeoTel documentation.  These will help in providing an outline customer quotation.  For the initial one or two installations it is likely that BT will make use of GeoTel for customer training, whether this is a suitable longer term option has still to be decided.  In the longer term it may be more appropriate for BT to do the customer training.

Before a quote can be verified all third-party organisations need to have confirmed  prices and delivery lead-times to BT.  At this stage Digital will be happy to assist BT in determining accurate costings and timescales.  For example, if a customer requires environmental work, e.g. enhancements to a power distribution unit or improved air-conditioning for the new equipment, Digital will be pleased to provide a quotation.

In addition to the administration tasks outlined above BT has a requirement to check the additional capacity that the new customer will bring with what is available on the platform and network.  This will also be the job of the Application Engineer working in conjunction with Capacity and Performance Planners.

## Customer Ordering Process

Once BT has received a customer order for the new service, internal and third-party orders will be raised in a timely manner.  In addition administration processes will be activated to register the customer on the various BT systems.  It is important that the BT processes and procedures are understood and easily initiated otherwise customer delivery timescales could be impacted.

Upon receipt of a BT order Digital (and other third-parties) will confirm prices and timescales.  This will enable BT to provide their customer with an accurate costing and project plan for the implementation.

## Deliver Equipment and Services

The BT Application Engineer will liaise with the customer and suppliers regarding delivery of equipment and services.  These tasks will be scheduled into the BT project plan created by the Application Engineer in conjunction with the various suppliers.

Prior to delivery Digital will build, customise and configure the equipment to meet the needs of the customer.  This work will be based upon information supplied by the Application Engineer.

## BT Acceptance

Once the equipment (Peripheral Gateways, Admin Workstations and networking equipment) has been delivered and installed there is a requirement for BT to formally accept the installation.  To do this BT and Digital will run through an acceptance test programme to ensure that the equipment is functioning as required.  Once this process has been completed satisfactorily, BT will sign an acceptance certificate.

## Customer Acceptance

Following closely on BT acceptance will be the customer’s acceptance of the service from BT.  In some cases these activities may happen simultaneously.  However, the actual timing will be on a case by case basis and will be dependent upon the customer and the complexity of the installation.

## Customer Training and Migration

A major part of the installation process will be defining a clear customer training schedule and an unambiguous migration plan, including call plan design, to ensure that the customer has a trouble-free installation.  This will be the job of the Application Engineer.

## Customer “Hand-holding”

For a period of time after the initial installation customers may require ready access to application expertise, either directly on site or via the telephone.

## On-going Support

BT has a target to improve their current customer support metric for performance affecting problems from a four hour clear to a two hour clear.  This is an end-to-end target, i.e. the clock starts ticking once the customer has logged a fault with the BT Support Desk.  Because this is a service supplied by BT any user problems will be registered with BT.  The BT operations staff will act as the first line of support and will try to resolve the problem.

If the BT staff can not fix the problem and it is perceived to be connected with the Digital platform or GeoTel application BT will call the Digital BT PACE desk (01256 373377) to log a support call.  When the call is logged it will be allocated a number.  This number will remain with the call through its life until it is closed.  It is important that the BT operations staff have a process in place to record the call number; should BT have a query regarding the call this is the first thing Digital staff will ask for in the Technical Support Centre.

Once logged one of three actions will generally take place:

- the call will be passed to a specialist to begin resolution, should a specialist be unavailable one will call back as soon as possible;

- an engineer will be despatched to site to repair a hardware malfunction;

- the call will be diverted to GeoTel to resolve an application issue.

Through the life of the call it will be managed by the Digital support escalation processes until it is closed.

All GeoTel issues will be communicated, by Digital, to GeoTel customer support.

# APPENDIX ONE - GLOSSARY OF TERMS

| ACI | Additional Call Information - (part of BT NUP) |
| --- | --- |
| AE | Application Engineer |
| ASNOU | Advanced Services Network Operations Unit |
| AWS | Administration Work Station (GeoTel) |
| BCD | Binary Coded Decimal |
| BOM | Bill of Materials |
| BT NUP | British Telecom - National User Part (part of C7) |
| BTNR | British Telecom Network Requirements |
| C7 | CCITT System Signalling 7 |
| C-ICR | Customer Intelligent Call Router |
| CCS | Common Channel Signalling |
| CIR | Committed Information Rate |
| CLI | Calling Line Identity (US=CLID) |
| COU | Central Operations Unit |
| CPE | Customer Premise Equipment |
| CPR0 | Cambridge Program Release 0 |
| CPR1 | Cambridge Program Release 1 (“NIP”) |
| CPS | Calls per second |
| CPU | Central Processor Unit |
| CSC | Customer Support Centre |
| CTN | Core Transport Network |
| DDSN | Distributed Diagnostic and Service Network (GeoTel) |
| DMSU | Digital Main Switching Unit |
| DSN | Derived Services Network |
| ESP | Early System Proving |
| FOA | First Office Application |
| Gb | Gigabyte |
| FSP | Final System Proving |
| ICR | Intelligent Call Router |
| IFAM | Initial and Final Address Message |
| IN/P1 | Intelligent Network / Phase 1 |
| IP | Internet Protocol |
| IVV&T | Integration, Validation, Verification and Test |
| IWT | Interworking Tests (BT) |
| Kbps | Kilobits per second |
| ms | millisecond |
| Mb | Megabyte |
| Mbps | Megabits per second |
| MTBF | Mean Time Between Failures |
| MTP | Message Transfer Part (part of C7) |
| MTTR | Mean Time To Repair |
| N-ICR | Network Intelligent Call Router |
| NBC | National Business Communications |
| NIC | Network Interface Card (PC Network Adaptor) |
| NIC | Network Interface Controller (GeoTel |
| NIP | Network Intelligence Platform |
| NIRO | Network Interconnect Responsible Officer |
| NIT | Network Integration Test |
| O/S | Operating System |
| ODBC | Open Data-Base Connection, SQL Server API |
| OLO | Other Licensed Operators |
| ORT | Operational Readiness Test (BT) |
| OSS | Operator Support Service |
| PACE | Pro-Active Call Environment |
| PCLI | Partial Calling Line Identification |
| PG | Peripheral Gateway |
| PRA | Pre-Release Assessment |
| PSTN | Public Switched Telephone Network |
| PVC | Permanent Virtual Circuit |
| RBR | Rules Based Router |
| RT | Record Type (BT billing record format) |
| SCCP | Service Connection Control Point |
| SCP | Service Control Point |
| SE | Support Engineer |
| SIRO | Systems Interconnect Responsible Officer |
| SPR | Signal Transfer Point with Relay (Relay at SCCP layer) |
| SRS | Support Requirements Specification |
| SSP | Service Switch Point |
| SSTF | ???? MH |
| STP | Signal Transfer Point |
| TBA | To Be Agreed |
| TBD | To Be Defined |
| TBS | To Be Supplied |
| TCAP | Transaction Capabilities Application Part |
| TCP | Transmission Control Protocol (part of TCP/IP ) |
| TNRTCAR | Telemarketing Near Real Time Control And Reports |
| TPS | Transactions per second (= cps) |
| UDP | User Datagram Protocol |
| VIS | Value-added Implementation Service |
| VV&T | Validation, Verification and Test |

# APPENDIX TWO - OUTSTANDING ACTIONS

| Action No. | Section No. | Description | With | Status |
| --- | --- | --- | --- | --- |
| CSD001 | 2.3.6 | Operational support model - provide details of strategy and requirements. | BT | Closed |
| CSD002 | 5.5.4 | Confirmation of router interface specification. | BT | ClosedError: Reference source not found |
| CSD003 | 5.7 | Acceptability of MPRN for operations traffic. | BT | ClosedError: Reference source not found |
| CSD004 | 6.9.5 | Number of laptops required. | BT | ClosedError: Reference source not found |
| CSD005 | 6.2 | More information on numbers of ACDs and call rates. | BT | ClosedError: Reference source not found |
| CSD006 | 6.4.3 | Data storage requirements. | BT | ClosedError: Reference source not found |
| CSD007 | 6.5 | Type of ACD for model system. | BT | ClosedError: Reference source not found |
| CSD008 | 6.6 | Definition of training model. | BT | ClosedError: Reference source not found |
| CSD009 | 6.7 | Definition of IVV&T model | DEC | Closed6 |
| CSD010 | 6.8 | Definition of support model. | BT | ClosedError: Reference source not found |
| CSD011 | 6.9.1 | Tape streamer requirement - backup strategy? | BT | ClosedError: Reference source not found |
| CSD012 | 7.1, 10.3 | Quality of service metrics. | BT | ClosedError: Reference source not found |
| CSD013 | 7.2.1 | MTBF figures. | DEC |  |
| CSD014 | 7.2.2 | MTTR figures. | DEC |  |
| CSD015 | 938.1 | Win NT specific security requirements. | BT | Deferred |
| CSD016 | 10.1 | Repair time requirements. | BT | Closed |
| CSD017 | 6.4.3 | Data Storage capacity | DEC | Closed |
| CSD018 | 4.6, 6.4.3, 10.14 | Customer backup strategy | DEC | ClosedError: Reference source not found |
| CSD019 |  | Quality of service metrics. | BT | Duplicate CSD012 |
| CSD020 | 2.1 | Dimensioning characteristics of agents per partition | DEC | ClosedError: Reference source not found |
| CSD021 | 4.1 | Firewalls in diagrams | DEC | ClosedError: Reference source not found |
| CSD022 | 3.3, 5.4 | CTI links | BT | ClosedError: Reference source not found |
| CSD023 | 6.9 | Dimensioning of AWS systems | DEC | ClosedError: Reference source not found |
| CSD024 | 6.3.1, 10.4 | "Moving" a customer | DEC | ClosedError: Reference source not found |
| CSD025 | 8.7.3 | Customer Security granularity | DEC | ClosedError: Reference source not found |
| CSD026 | 8.7.1 | BT AWS security granularity | DEC | ClosedError: Reference source not found |
| CSD027 | 5.7, 8.10 | MPRN security | DEC | ClosedError: Reference source not found |
| CSD028 | 10.10, 8.12 | Typical scenarios : Documentation detailing faulting and repair flows from GeoTel to be included, also User monitoring | DEC | ClosedError: Reference source not found |
| CSD029 | 2.1 | If more than 5 partitions do we need to pay for another license — Yes | DEC | ClosedError: Reference source not found |
| CSD030 | 2.5 | Access to UPS on systems, and explanation of cutover operation | DEC | ClosedError: Reference source not found |
| CSD031 | 8.7.3 | Describe How Data partitioning will be achieved | DEC |  |
| CSD032 | 10.8 | Grid System Productisation needs to be defined & costed | DEC | ClosedError: Reference source not found |
| CSD033 | 5.8 | MCI 800 Gateway, i/f spec, training overhead, equipment specification, dimensioning requirements, | DEC | ClosedError: Reference source not found |
| CSD034 | 2.2, 3.2, 3.10, 4.3.5, 5.5.6, 5.5.5, 8.5, 9.1, 9.3, 9.4 | To review outstanding revisions as supplied by BT | DEC | ClosedError: Reference source not found |
| CSD035 | 4.3.2 | ISDN backup for CPE routers (BT to provide specification then Digital to confirm configuration) | BT |  |
| CSD036 | 6.1.2 | Update section (and corresponding section in Interfaces) to allow for modified AXE-10 C7 connection design | DEC |  |
| CSD037 | 7.5 | Overload performance query | DEC | ClosedError: Reference source not found |

# APPENDIX THREE - SIGN-OFF SHEET

| Name and Job Title | Company | Signature | Date |
| --- | --- | --- | --- |
| Mike McCabe Project Manager | Digital |  |  |
| Paul Buchan BT Account Manager | Digital |  |  |
| Graham Cobb Solution Architect | Digital |  |  |
| Pat Kelly Director, Product Management | GeoTel |  |  |
| Jerry Gechter Manager, Carrier Relationships | GeoTel |  |  |
| John Nevins Project Manager | BT |  |  |
| Dave Guard Chief Designer | BT |  |  |
| Roger Sutton Implementation Manager | BT |  |  |
| Andy Reason NBC Product Launch | BT |  |  |
| John Smith Telemarketing Product Line | BT |  |  |
| Glen Gilroy IVV&T | BT |  |  |

# APPENDIX FOUR - LOCATION OF CSD COPIES

| Document No. | Holder |
| --- | --- |
| 1 | John Nevins (BT) |
| 2 | Peter Donovan (BT) |
| 3 | John Smith (BT) |
| 4 | Glen Gilroy (BT) |
| 5 | Roger Sutton (BT) |
| 6 | Dave Guard (BT) |
| 7 | Andy Reason (BT) |
| 8 | Jerry Gechter (GeoTel) |
| 9 | Steve Webber (GeoTel) |
| 10 | Paul Buchan (Digital) |
| 11 | Mike McCabe (Digital) |
| 12 | John Abram (Digital) |
| 13 | Graham Cobb (Digital) |
| 14 | Unnamed for further distribution |

Digital would request that BT and GeoTel track “unnamed distribution” and furnish Digital with a list of who, apart from the named individuals above, receives a copy of the CSD.

The number of Network and System Management AWS PCs was increased to 15, from 3, as a result of the discussions during the Requirements Workshop.

The number of signalling terminations has been reduced as a result of the discussions during the Requirements Workshop.

In this context, “SCP” means “ICR site”

Closed by John Nevins in his comments on draft AX03

No action is required during the CSD phase. This item refers to a later project deliverable

Specified in SRS

Closed in AX04