To:	UsersFrom:	The AuthorSubj:	Simulator UsageHThis memorandum documents the PDP-8, PDP-11, and Nova simulators.  TheseEsimulators are freeware; commercial use is prohibited, and support isGnot available.  The best way to fix problems or add features is to readIand modify the sources yourself.  Alternately, you can send Internet mailDto dsmaint@pa.dec.com; but responses are NOT guaranteed.  [Note: the0Nova simulator is not included in this release.]DThe simulators use a common command interface.  The memorandum firstHdescribes the common features of the command interface and then provides1information on each of the individual simulators.$1. Compiling And Running A SimulatorEThe simulators have been tested on VAX/VMS, Alpha/VMS, and Alpha/OSF.CPorting to other environments will require changes to the operating#system dependent code in scp_tty.c.;To compile the simulators on Alpha/OSF, use these commands: 	% cc pdp8*.c scp*.c -lm -o pdp8	% cc pdp11*.c scp*.c -o pdp11 	% cc nova*.c scp*.c -lm -o novaBTo start the simulator, simply type its name.  The simulator takesBone optional argument, a startup command file.  If specified, thisCfile should contain a series of non-interactive simulator commands,Gone per line.  These command can be used to set up standard parameters,for example, disk sizes.	% pdp8 <startup file>(cr)	or	% pdp11 <startup file>(cr)	or	% nova <startup file>(cr)CThe simulator types out its name and version, executes the commands<in the startup file, if any, and then prompts for input with	sim>

2. Simulator ConventionsIA simulator consists of a series of devices, the first of which is alwaysGthe CPU.  A device consists of named registers and one or more numberedEunits.  Registers correspond to device state, units to device addressEspaces.  Thus, the CPU device might have registers like PC, ION, etc,Aand a unit corresponding to main memory; a disk device might haveEregisters like BUSY, DONE, etc, and units corresponding to individualIdisk drives.  Except for main memory, device address spaces are simulatedFas disk files in the host file system.  The SHOW CONFIGURATION command%displays the simulator configuration.IA simulator keeps time in terms of arbitrary units, usually one time unitGper instruction executed.  Simulated events (such as completion of I/O)Hare scheduled at some number of time units in the future.  The simulatorGexecutes synchronously, invoking event processors when simulated eventsGare scheduled to occur.  Even asynchronous events, like keyboard input,Hare handled by polling at synchronous intervals.  The SHOW QUEUE command#displays the simulator event queue.

3. Commands3.1 Loading ProgramsDThe LOAD command (abbreviation LO) loads a file in binary paper-tapeloader format:	sim> load <filename>(cr)3.2 Saving and Restoring StateAThe SAVE command (abbreviation SA) save the complete state of theCsimulator to a file.  This includes the contents of main memory and2all registers, and the I/O connections of devices:	sim> save <filename>(cr)CThe RESTORE command (abbreviation REST, alternately GET) restores a!previously saved simulator state:	sim> restore <filename>(cr)?Note: SAVE file format compresses zeroes to minimize file size.3.3 Resetting DevicesAThe RESET command (abbreviation RE) resets a device or the entire$simulator to a predefined condition:&	sim> RESET(cr)			-- reset all devices)	sim> RESET ALL(cr)		-- reset all devices3	sim> RESET <device>(cr)		-- reset specified device@Typically, RESET stops any in-progress I/O operation, clears anyCinterrupt request, and returns the device to a quiescent state.  It5does not clear main memory or affect I/O connections.(3.4 Connecting and Disconnecting DevicesFExcept for main memory, simulated unit address spaces are simulated asGdisk files in the host file file system.  Before using a simulated unitDthe user must specify the file to be used by a particular unit.  TheCATTACH (abbreviation AT) command associates a unit and a disk file:1	sim> ATTACH <device><unit number> <filename>(cr)EIf the file does not exist, it is created, and an appropriate messageis printed.FThe DETACH (abbreviation DET) command breaks the association between a$unit and a file and closes the file:*	sim> DETACH ALL(cr)			-- detach all units?	sim> DETACH <device><unit number>(cr)	-- detach specified unit2The EXIT command performs an automatic DETACH ALL. 3.5 Examining and Changing State4There are four commands to examine and change state:'	EXAMINE (abbreviated E) examines state&	DEPOSIT (abbreviated D) changes state>	IEXAMINE (interactive examine, abbreviated IE) examines state0		and allows the user to interactively change it?	IDEPOSIT (interactive deposit, abbreviated ID) allows the user		to interactively change stateAll four commands take the form;	command {-switches} {<device>{<unit number>}} <state list>DDeposit must also include a deposit value at the end of the command.EThe examine and deposit commands can work on the address space of anyGunit.  If no device is specified, the CPU (main memory) is selected; ifDa device but no unit is specified, unit 0 of the specified device isEselected.  The "state list" consists of one or more of the following,separated by commas:!	register		the specified register<	register1-register2	all the registers starting at register1!				up to and including register2!	address			the specified location;	address1-address2	all locations starting at address1 up to				and including address2$	STATE			all registers in the device 	ALL			all locations in the unit	Examples:)	sim> ex 1000-1100			-- examine 1100:1100%	sim> de PC 1040				-- set PC to 10400	sim> ie 40-50				-- interactively examine 40:500	sim> ex rx0 50060			-- examine 50060, RX unit 0)	sim> de all 0				-- set main memory to 0GNote: to terminate an interactive command, simply type a bad value (eg,XXX) when input is requested.3.6 Running ProgramsJThe RUN command (abbreviated RU) resets all devices, deposits its argumentE(if given) in the PC, and starts execution.  If no argument is given,#execution starts at the current PC.JThe GO command does NOT reset devices, deposits its argument (if given) inKthe PC, and starts execution.  If no argument is given, execution starts atthe current PC.DThe CONT command (abbreviated CO) does NOT reset devices and resumesexecution at the current PC.HThe STEP command (abbreviated S) resumes execution at the current PC forDthe number of instructions given by its argument.  If no argument is&supplied, one instruction is executed.EThe BOOT command (abbreviated BO) bootstraps the random access deviceBand unit given by its argument.  If no unit is supplied, unit 0 isBbootstrapped.  The specified unit must be attached to a disk file.3.7 Stopping ProgramsGPrograms run until the simulator detects an error or stop condition, or'until the user forces a stop condition.(3.7.1 Simulator Detected Stop Conditions4These simulator-detected conditions stop simulation:Ho HALT instruction.  If a HALT instruction is decoded, simulation stops.Co Breakpoint.  The IBKPT register provides a single virtual addressD  breakpoint.  If the PC matches the contents of the IBKPT register,A  simulation stops.  The breakpoint is automatically disabled for!  the next instruction execution.Ko I/O error.  If an I/O error occurs during simulation of an I/O operation,I  and the device stop-on-I/O-error flag is set, simulation usually stops.Io Processor condition.  Certain processor conditions can stop simulation;5  these are described with the individual simulators.$3.7.2 User Specified Stop ConditionsITyping the interrupt character stops simulation.  The interrupt characterFis defined by the WRU (where are you) register and is initially set to	005 (^E).3.8 Setting Device ParametersJThe SET command (abbreviated SE) changes the status of a device parameter:	sim> SET <unit> <parameter>CThe parameters are simulator and device specific.  Disk drives, forEexample, can usually be set ONLINE or OFFLINE, write ENABLED or writeBLOCKED; if a device supports multiple drive types, the SET command&can be used to specify the drive type.$3.8 Displaying Parameters and StatusHThe SHOW CONFIGURATION command shows the simulator configuration and the$status of all simulated I/O devices.EThe SHOW <device> command shows the status of the named simulated I/Odevice.KThe SHOW QUEUE command shows the state of the simulator event queue.  TimesHare in "simulation units", typically one unit per instruction execution,(relative to the current simulation time.BThe SHOW TIME command shows the number of time units elapsed sincethe last RUN command.3.9 Exiting The Simulator<EXIT (synonym QUIT) returns control to the operating system.

4. PDP-8 Features-The PDP-8 simulator is configured as follows:	device	simulates	name(s)$	CPU	PDP-8/E CPU with 32KW of memory*	-	KE8-E extended arithmetic element (EAE)0	-	KM8-E memory management and timeshare control&	PTR,PTP	PC8-E paper tape reader/punch	TTI,TTO	KL8-E console terminal	LPT	LE8-E line printer9	CLK	DK8-E line frequency clock (also PDP-8/A compatible)9	RK	RK8-E/RK05 cartridge disk controller with four drives6	RF	RF08 fixed head disk controller with four platters5	RX	RX8-E/RX01 floppy disk controller with two drivesIThe PDP-8 simulator implements one unique stop condition: if an undefinedJinstruction (uinimplemented IOT or OPR) is decoded, and register STOP_INSTis set, the simulator halts.4.1 CPUMThe only CPU options are the presense of the EAE and the size of main memory;Gthe memory extension and time-share control is always included, even ifmemory size is 4K.	SET CPU EAE		enable EAE	SET CPU NOEAE		disable EAE!	SET CPU 4K		set memory size = 4K!	SET CPU 8K		set memory size = 8K#	SET CPU 12K		set memory size = 12K#	SET CPU 16K		set memory size = 16K#	SET CPU 20K		set memory size = 20K#	SET CPU 24K		set memory size = 24K#	SET CPU 28K		set memory size = 28K#	SET CPU 32K		set memory size = 32KHIf memory size is being reduced, and the memory being truncated containsJnon-zero data, the simulator asks for confirmation.  Data in the truncated7portion of memory is lost.  Initial memory size is 32K.GCPU registers include the visible state of the processor as well as the+control registers for the interrupt system.	name		size	comments4	PC		15	program counter, including IF as high 3 bits	AC		12	accumulator	MQ		12	multiplier-quotient

L		1	link	SR		12	front panel switches	IF		3	instruction field	DF		3	data field	IB		3	instruction field buffer	SF		7	save field	UF		1	user mode flag	UB		1	user mode buffer	SC		5	EAE shift counter	GTF		1	EAE greater than flag!	EMODE		1	EAE mode (0 = A, 1 = B)	ION		1	interrupt enable+	ION_DELAY	1	interrupt enable delay for ION+	CIF_DELAY	1	interrupt enable delay for CIF 	PWR_INT		1	power fail interrupt(	UF_INT		1	user mode violation interrupt 	INT		14	interrupt pending flags	DONE		14	device done flags)	ENABLE		14	device interrupt enable flags2	OLDPC		15	PC prior to last JMP, JMS, or interrupt*	STOP_INST	1	stop on undefined instruction1	BREAK		16	breakpoint address (177777 to disable)	WRU		8	interrupt character94.2 Programmed I/O Devices (PTR, PTP, TTI, TTO, CLK, LPT)EThe paper tape reader (PTR), paper tape punch (PTP), and line printerC(LPT) read data from or write data to disk files.  The POS registerHspecifies the number of the next data item to be read or written.  Thus,Aby changing POS, the user can backspace or advance these devices.?The programmed I/O devices typically implement these registers:	name		size	comments 	BUF		8	last data item processed	INT		1	interrupt pending flag	DONE		1	device done flag 	ENABLE		1	interrupt enable flag/	TIME		24	time from I/O initiation to interrupt$				(for keyboard, polling interval)-	POS		32	position in the input or output file	STOP_IOE	1	stop on I/O error5For the serial devices, error handling is as follows:&	type	error	     STOP_IOE	processed as,	in,out	not attached	1	report error and stop				0	out of tape or paper'	in	end of file	1	report error and stop				0	out of tape or paper,	in,out	OS I/O error	x	report error and stop 4.3 RK8E/RK05BRK8E options include the ability to place units online or offline,write enabled or write locked:$	SET RKn OFFLINE		set unit n offline"	SET RKn ONLINE		set unit n online(	SET RKn LOCKED		set unit n write locked*	SET RKn ENABLED		set unit n write enabled$The RK8E implements these registers:	name		size	comments	STAT		12	status	CAR		12	current memory address	DAR		12	disk address	CMD		12	disk command	BUSY		1	control busy flag	INT		1	interrupt pending flag"	STIME		24	seek time, per cylinder	RTIME		24	rotational delay	STOP_IOE	1	stop on I/O errorError handling is as follows:!	error	     STOP_IOE	processed as%	not attached	1	report error and stop			0	disk not ready*	end of file	x	assume rest of disk is zero%	OS I/O error	x	report error and stop 4.4 RX8E/RX01LRX8E options include the ability to set units write enabled or write locked:(	SET RXn LOCKED		set unit n write locked*	SET RXn ENABLED		set unit n write enabled$The RX8E implements these registers:	name		size	comments	RXCS		12	status	RXDB		12	data buffer	RXES		8	error status	RXTA		8	current track	RXSA		8	current sector	STAPTR		3	controller state	BUFPTR		3	buffer pointer	INT		1	interrupt pending flag	DONE		1	device done flag 	ENABLE		1	interrupt enable flag	TR		1	transfer ready flag	ERR		1	error flag"	CTIME		24	command completion time	STIME		24	seek time, per track	XTIME		24	transfer ready delay	STOP_IOE	1	stop on I/O errorError handling is as follows:!	error	     STOP_IOE	processed as%	not attached	1	report error and stop			0	disk not readyERX01 data files are buffered in memory; therefore, end of file and OSI/O errors cannot occur.4.5 RF08$The RF08 implements these registers:	name		size	comments	STAT		12	status	DAR		20	current disk address	WLK		32	write lock switches	INT		1	interrupt pending flag	DONE		1	device done flag$	TIME		24	rotational delay, per word	BURST		1	burst flag	STOP_IOE	1	stop on I/O errorJThe RF08 is a three-cycle data break device.  If BURST = 0, word transfersGare scheduled individually; if BURST = 1, the entire transfer occurs ina single data break.Error handling is as follows:!	error	     STOP_IOE	processed as%	not attached	1	report error and stop			0	disk not readyERF08 data files are buffered in memory; therefore, end of file and OSI/O errors cannot occur.

5. PDP-11 Features.The PDP-11 simulator is configured as follows:	device	simulates	name(s)"	CPU	J-11 CPU with 256KB of memory"	-	FP-11 floating point unit (FPA)&	PTR,PTP	PC-11 paper tape reader/punch	TTI,TTO	DL-11 console terminal	LPT	LP-11 line printer	CLK	line frequency clock9	RK	RK11/RK05 cartridge disk controller with eight drives<	RL	RLV12/RL01(2) cartridge disk controller with four drives5	RX	RX-11/RX01 floppy disk controller with two drives?The PDP-11 simulator implements several unique stop conditions:D	- abort during exception vector fetch, and register STOP_VEC is setB	- abort during exception stack push, and register STOP_SPA is set>	- trap condition 'n' occurs, and register STOP_TRAP<n> is set8	- wait state entered, and no I/O operations outstanding$	  (ie, no interrupt can ever occur)5.1 CPUEMemory size is a compile time parameter in pdp11_defs.h; the standardmemory size is 248KB.IThese switches are recognized when examining or depositing in CPU memory:"	-v			interpret address as virtual*	-d			if mem mgt enabled, force data space+	-k			if mem mgt enabled, force kernel mode/	-s			if mem mgt enabled, force supervisor mode)	-u			if mem mgt enabled, force user mode-	-p			if mem mgt enabled, force previous modeGCPU registers include the visible state of the processor as well as the+control registers for the interrupt system.	name		size	comments	PC		16	program counter&	R0..R5		16	R0..R5, first register set(	R10..R15	16	R0..R5, second register set	KSP		16	kernel stack pointer!	SSP		16	supervisor stack pointer	USP		16	user stack pointer	PSW		16	processor status word#	  CM		2	  current mode, PSW<15:14>$	  PM		2	  previous mode, PSW<13:12> 	  RS		2	  register set, PSW<11>.	  IPL		3	  interrupt priority level, PSW<7:5>	  T		1	  trace bit, PSW<4>	  N		1	  negative flag, PSW<3>	  Z		1	  zero flag, PSW<2>	  V		1	  overflow flag, PSW<1>	  C		1	  carry flag, PSW<0>	SR		16	front panel switches	DR		16	front panel display!	MEMERR		16	memory error register	CCR		16	cache control register	MAINT		16	maintenance register	HITMISS		16	hit/miss register	CPUERR		16	CPU error register'	PIRQ		16	programmed interrupt requests)	FAC0H..FAC5H	32	FAC0..FAC5, high 32 bits(	FAC0L..FAC5L	32	FAC0..FAC5, low 32 bits	FPS		16	floating point status#	FEA		16	floating exception address	FEC		4	floating exception code-	MMR0..3		16	memory management registers 0..3	{K/S/U}{I/D}{PAR/PDR}{0..7}!			16	memory management registers 	INT		32	interrupt pending flags	TRAP		18	trap pending flags	WAIT		0	wait state flag%	WAIT_ENABLE	0	wait state enable flag!	STOP_TRAPS	18	stop on trap flags4	STOP_VECA	1	stop on read abort in trap or interrupt9	STOP_SPA	1	stop on stack push abort in trap or interrupt2	OLDPC		16	PC prior to last JMP, JMS, or interrupt,	BREAK		16	breakpoint address (1 to disable)	WRU		8	interrupt character95.2 Programmed I/O Devices (PTR, PTP, TTI, TTO, CLK, LPT)EThe paper tape reader (PTR), paper tape punch (PTP), and line printerC(LPT) read data from or write data to disk files.  The POS registerHspecifies the number of the next data item to be read or written.  Thus,Aby changing POS, the user can backspace or advance these devices.?The programmed I/O devices typically implement these registers:	name		size	comments 	BUF		8	last data item processed 	CSR		16	control/status register	INT		1	interrupt pending flag	ERR		1	error flag (CSR<15>)"	DONE		1	device done flag (CSR<7>)%	IE		1	interrupt enable flag (CSR<6>)/	TIME		24	time from I/O initiation to interrupt$				(for keyboard, polling interval)-	POS		32	position in the input or output file	STOP_IOE	1	stop on I/O error5For the serial devices, error handling is as follows:&	type	error	     STOP_IOE	processed as,	in,out	not attached	1	report error and stop				0	out of tape or paper'	in	end of file	1	report error and stop				0	out of tape or paper,	in,out	OS I/O error	x	report error and stop 5.3 RK11/RK05BRK11 options include the ability to place units online or offline,write enabled or write locked:$	SET RKn OFFLINE		set unit n offline"	SET RKn ONLINE		set unit n online(	SET RKn LOCKED		set unit n write locked*	SET RKn ENABLED		set unit n write enabled$The RK11 implements these registers:	name		size	comments	RKCS		16	control/status	RKDA		16	disk address	RKBA		16	memory address	RKWC		16	word count	RKDS		16	drive status	RKER		16	error status	INTQ		9	interrupt queue&	DRVN		3	number of last selected drive	INT		1	interrupt pending flag	ERR		1	error flag (CSR<15>)"	DONE		1	device done flag (CSR<7>)%	IE		1	interrupt enable flag (CSR<6>)	INT		1	interrupt pending flag"	STIME		24	seek time, per cylinder	RTIME		24	rotational delay	STOP_IOE	1	stop on I/O errorError handling is as follows:!	error	     STOP_IOE	processed as%	not attached	1	report error and stop			0	disk not ready*	end of file	x	assume rest of disk is zero%	OS I/O error	x	report error and stop 5.4 RX11/RX01MRX11 options include the ability to make units write enabled or write locked:(	SET RXn LOCKED		set unit n write locked*	SET RXn ENABLED		set unit n write enabled$The RX11 implements these registers:	name		size	comments	RXCS		12	status	RXDB		8	data buffer	RXES		8	error status	RXERR		8	error code	RXTA		8	current track	RXSA		8	current sector	STAPTR		3	controller state	BUFPTR		3	buffer pointer	INT		1	interrupt pending flag	ERR		1	error flag (CSR<15>)#	TR		1	transfer ready flag (CSR<7>)%	IE		1	interrupt enable flag (CSR<6>)"	DONE		1	device done flag (CSR<5>)"	CTIME		24	command completion time	STIME		24	seek time, per track	XTIME		24	transfer ready delay	STOP_IOE	1	stop on I/O errorError handling is as follows:!	error	     STOP_IOE	processed as%	not attached	1	report error and stop			0	disk not readyERX01 data files are buffered in memory; therefore, end of file and OSI/O errors cannot occur.5.5 RL11(V12)/RL01,RL02LRL11 options include the ability to set units write enabled or write locked,5and to set the drive size to RL01, RL02, or autosize:(	SET RLn LOCKED		set unit n write locked*	SET RLn ENABLED		set unit n write enabled	SET RLn RL01		set size to RL01	SET RLn RL02		set size to RL027	SET RLn AUTOSIZE	set size based on file size at attachHThe size options can be used only when a unit is not attached to a file.$The RL11 implements these registers:	name		size	comments	RLCS		16	control/status	RLDA		16	disk address	RLBA		16	memory address*	RLBAE		6	memory address extension (RLV12)+	RLMP..RLMP2	16	multipurpose register queue	INT		1	interrupt pending flag	ERR		1	error flag (CSR<15>)"	DONE		1	device done flag (CSR<7>)%	IE		1	interrupt enable flag (CSR<6>)	INT		1	interrupt pending flag"	STIME		24	seek time, per cylinder	RTIME		24	rotational delay	STOP_IOE	1	stop on I/O errorError handling is as follows:!	error	     STOP_IOE	processed as%	not attached	1	report error and stop			0	disk not ready*	end of file	x	assume rest of disk is zero%	OS I/O error	x	report error and stop

6. Nova Features,The Nova simulator is configured as follows:	device	simulates	name(s)!	CPU	NOVA CPU with 32KW of memory	-	hardware multiply/divide 	PTR,PTP	paper tape reader/punch	TTI,TTO	console terminal	LPT	line printer	CLK	real-time clock1	DKP	moving head disk controller with four drives;The Nova simulator implements these unique stop conditions:9	- reference to undefined I/O device, and STOP_DEV is set:	- more than INDMAX indirect addresses are detected during	  an interrupt:	- more than INDMAX indirect addresses are detected during$	  memory reference address decoding6.1 CPUIThe only CPU options are the presense of the multiply/divide instructionsand the size of main memory.$	SET CPU MDV		enable multiply/divide'	SET CPU NOMDV		disable multiply/divide!	SET CPU 4K		set memory size = 4K!	SET CPU 8K		set memory size = 8K#	SET CPU 12K		set memory size = 12K#	SET CPU 16K		set memory size = 16K#	SET CPU 20K		set memory size = 20K#	SET CPU 24K		set memory size = 24K#	SET CPU 28K		set memory size = 28K#	SET CPU 32K		set memory size = 32KHIf memory size is being reduced, and the memory being truncated containsJnon-zero data, the simulator asks for confirmation.  Data in the truncated7portion of memory is lost.  Initial memory size is 32K.GCPU registers include the visible state of the processor as well as the+control registers for the interrupt system.	name		size	comments	PC		15	program counter	AC0..AC3	16	accumulators 0..3	C		1	carry	SR		16	front panel switches	PI		16	priority interrupt mask	ION		1	interrupt enable+	ION_DELAY	1	interrupt enable delay for ION	PWR		1	power fail interrupt 	INT		15	interrupt pending flags	BUSY		15	device busy flags	DONE		15	device done flags+	DISABLE		15	device interrupt disable flags!	STOP_DEV	1	stop on undefined IOT.	INDMAX		15	maximum number of nested indirects2	OLDPC		15	PC prior to last JMP, JMS, or interrupt1	BREAK		16	breakpoint address (177777 to disable)	WRU		8	interrupt character96.2 Programmed I/O Devices (PTR, PTP, TTI, TTO, CLK, LPT)EThe paper tape reader (PTR), paper tape punch (PTP), and line printerC(LPT) read data from or write data to disk files.  The POS registerHspecifies the number of the next data item to be read or written.  Thus,Aby changing POS, the user can backspace or advance these devices.?The programmed I/O devices typically implement these registers:	name		size	comments 	BUF		8	last data item processed	INT		1	interrupt pending flag	BUSY		1	device busy flag	DONE		1	device done flag"	DISABLE		1	interrupt disable flag/	TIME		24	time from I/O initiation to interrupt$				(for keyboard, polling interval)-	POS		32	position in the input or output file	STOP_IOE	1	stop on I/O error5For the serial devices, error handling is as follows:&	type	error	     STOP_IOE	processed as,	in,out	not attached	1	report error and stop				0	out of tape or paper'	in	end of file	1	report error and stop				0	out of tape or paper,	in,out	OS I/O error	x	report error and stop6.3 Moving Head Disk (DKP)EMoving head disk options include the ability to place units online orHoffline, write enabled or write locked, and to select the type of drive:$	SET DKPn OFFLINE	set unit n offline#	SET DKPn ONLINE		set unit n online)	SET DKPn LOCKED		set unit n write locked*	SET DKPn ENABLED	set unit n write enabled+	SET DKPn FLOPPY		set unit n to floppy disk&	SET DKPn D31		set unit n to Diablo 31+	SET DKPn D44		set unit n to Diablo 44/6030)	SET DKPn C111		set unit n to Century 111)	SET DKPn C114		set unit n to Century 114IAll drives have 256 16b words per sector.  The other disk parameters are:/	drive	    cylinders surfaces	sectors	size (MW)	floppy		77	1	8	  .158	D31		203	2	12	 1.247	D44		408	4	12	 5.014	C111		203	10	6	 3.118	C114		203	20	12	12.472;The moving head disk controller implements these registers:	name		size	comments"	FCCY		16	flags, command, cylinder&	USSC		16	unit, surface, sector, count	STAT		16	status	MA		16	memory address	INT		1	interrupt pending flag	BUSY		1	device busy flag	DONE		1	device done flag"	DISABLE		1	interrupt disable flag"	STIME		24	seek time, per cylinder	RTIME		24	rotational delay	STOP_IOE	1	stop on I/O errorError handling is as follows:	error		processed as	not attached	disk not ready(	end of file	assume rest of disk is zero#	OS I/O error	report error and stop6.4 Fixed Head Disk (DSK):The fixed head disk controller implements these registers:	name		size	comments	STAT		16	status	DA		16	disk address	MA		16	memory address	INT		1	interrupt pending flag	BUSY		1	device busy flag	DONE		1	device done flag	DISABLE		1	device disable flag	WLK		8	write lock switches&	TIME		24	rotational delay, per sector	STOP_IOE	1	stop on I/O errorError handling is as follows:!	error	     STOP_IOE	processed as%	not attached	1	report error and stop			0	disk not readyIFixed head disk data files are buffered in memory; therefore, end of fileand OS I/O errors cannot occur.rint flush endProduct: [%s]QMS-PS 1700version: [%s]getidle: abort sent(\n%%[ status: idle ]%%] QMSBUG\n) print flushGetidle >>> %sGetidle <<< %sversion (23.0) eq { statusdict /Patch1Installed known not { (%%[ patch needed ]%%

) print } if } if statusdict begin (%%[ start page #=) print pagecount pstack pop flush end

!PATCHSyncprinter >>> %sSyncprinter <<< %suserdict /%s known {(%%%%[ resource