Intel® Modular Server System MFYSY25

P2.3.1 Unified Firmware Update

Release Notes & Installation Instructions

Copyright © 2007-2008 Intel Corporation

All Rights Reserved

June 18, 2008

RELEASE NOTES FOR: P2.3.1

## RELEASE PACKAGE CONTENTS

This section enumerates the contents of the release package.

- BIOS: SB5000.86B.10.00.0043.041620081415

- BMC: 1.30.3

- Management Module GUI opération Code: 2.4.100.20080610.11578

- Remote KVM Java Applet – 1.40

- EBF: 1.24

- Storage

  - IOP FW/SW: 2.91.0140.50

  - IOP SW: 2.91.0140.50

  - Expander FW: 1.03.0140.02

  - Redboot image: 1.01.0140.08

- Switch Boot Code: 1.0.0.6

- Switch FW: 1.0.0.27

- Fan Controller Op code – 01.1

- Fan Controller Boot Block – 01.2

- MIB files

## GENERAL INFORMATION

This release does not support rollback to previous versions of the Unified Firmware Update (UFU) package.

This release supports system configurations with two SCM for Microsoft® Windows 2003 R2 SP2 and Microsoft® Windows 2008. Dual SCM operation is not supported in this release for LINUX operating systems.

This release supports system configurations with only one SCM in Slot SCM1 for all supported Operating Systems.

## DEPENDENCIES

The system must be at firmware level P2.2.2 or P2.2.3 prior to starting this update.

The system should have only one SCM installed during the firmware update. After the firmware update has been completed the second SCM can be installed in SCM2 slot. After installation of the second SCM, the system will then start an update for the firmware of the second SCM and expander on the interposer. Please see instructions for 2nd SCM installation and firmware update process in the Install Second SCM in SCM2 Slot section below. The update process will take approximately 15 – 20 minutes for the second SCM and should not be interrupted.

## DEFECTS FIXED IN THIS RELEASE

| Area | Description |
| --- | --- |
| CMM GUI | ESM up time in CMM GUI is incorrect. |
| CMM GUI | Browse is not enabled when Redirect CDROMImage is selected |
| CMM GUI | RKVM F1 brings up local console help and does not pass F1 to remote compute module server |
| CMM GUI | Storage Pool view disappears |
| CMM GUI | IPMI requests/responses to/from the compute module out of synch |
| CMM GUI | Actions will complete if user clicks OK without typing “CONFIRM” |
| CMM GUI | Popup error when user does not select a replaced drive for the rebuild of a Storage Pool |
| CMM GUI | GUI error message needs clear instructions for user when a rebuild is attempted without an available spare drive for rebuild |
| BMC | BMC does not log events to compute module SEL if the CMM is nonoperational |
| BMC | Intermittent system fan boost due to readings from compute modules |
| Switch Module | Ethernet switch port flapping on slots without compute modules |
| Switch Module | ESM GUI switch uptime does not match that of CMM GUI |

## NEW FEATURES/FUNCTIONS IMPLEMENTED IN THIS RELEASE

- Dual SCM Controller - This release provides support for redundant SCM controllers with Failover / Failback functionality for Microsoft® Windows 2003 and Microsoft® Windows 2008 only. A future release will add support for Redhat® Enterprise LINUX 5 U 1 and Suse® Enterprise LINUX 10 U1.  Note that there are BKMs for each OS installation with the dual SCM controller configuration, as well as MPIO drivers. Please refer to the BKMs for proper installation of the operating system and drivers.

## KNOWN ISSUES

- Reset BIOS default settings – Certain settings and features such as Remote USB (essential for proper Remote KVM operation) may not be functional until BIOS Defaults are loaded. To ensure proper operation, reset BIOS defaults as soon as possible after updating BIOS. BIOS defaults are set by pressing F9 (Reset To Defaults) while in BIOS Setup.

- GUI inconsistencies with physical state - The GUI can be inconsistent with the state of the system because this is a web application. If you suspect an inconsistency, just reload the page (the page automatically refreshes once a minute). Some HW inventory changes (such as compute module insertion) can take up to 60 seconds to be reflected in the GUI unless you push the refresh button. The application may also appear inconsistent with the real state of the system if you have connectivity issues with the CMM.

- Complete System Diagnostics – Under synthetic stress test it is possible although rare for the SCM to reset if the user requests an All Subsystems diagnostics also called Complete System diagnostics.  Prior to running the All Subsystems diagnostics feature the user should first power off all server modules within the chassis to guarantee that this issue is not encountered.

- MIB Files:

- MIB files provide read only access to system information.  System management and configuration are handled via the Modular Server Control GUI.  Read/Write access will be available in future release.

- Remote applications:

- When starting rKVM with a Linux host, you must change the mouse mode to relative mode. If the remote mouse does not respond to local mouse movements, try switching to absolute mode and then immediately back to relative mode.

- Due to the way Linux handles the mouse (relative vs. absolute) there are instances when the remote mouse on a Linux host will be slow or appear to stop functioning. Many times a stuck mouse can be awakened by toggling between relative and absolute mouse modes via the menu or the keyboard by pressing Alt-s.

- If the local and remote mouse pointers do not synchronize (the remote mouse moves but is offset from the local mouse), try toggling the Synchronize mouse option. If that does not work, check the mouse acceleration parameters on the server with the command “xset q”. Set the mouse acceleration settings in Remote KVM in the Options menu to match those on the server. By default, the Remote KVM application will use an acceleration value of 2 and an acceleration threshold value of 4. These values are what the Red Hat Enterprise Linux Server 5 (RHEL5) and SuSE Linux Enterprise Server (SLES) operating systems expect, unless they have been modified after installation of the OS.

- If the Internet connection is lost during an existing RKVM session, the session will report video data lost. The user must close the message, and restart the session.

- Lost Keyboard/Mouse usage in SLES can sometimes be recovered by toggling full screen on and off.

- Remote media is best used with one server at a time. If you map a CD to six compute modules, the compute modules will drop the CD mapping, due to latency issues.

- The RKVM client version included in this release is 1.40. Check the version after installing the new release by launching the RKVM application and then using Help->About in the RKVM client window menus. If the version is something other than 1.40, close all RKVM client windows and delete the application from the client machine’s Java browser cache. (Run ‘javaws –viewer’, select the application titled ‘Networked KVM applet’ and press the delete key.)

- Remote Floppy is not supported during a RKVM session.

- Pressing F1 during a RKVM session will bring up the local KVM console help not the remote system application help.

## INSTALLATION INSTRUCTIONS

## Preparing For Update:

- Download the current Unified Firmware Update package MFSYS25_UFU_P2_3_1.zip for the MFSYS25 system (including the MFS5000SI Compute Module).

  - Copy the CMM_Core.zip file to a folder labeled P2_3_1 on a formatted flash drive. CMM_Core.zip is the update package and is loaded as a zip file by the CMM Firmware Update tool. Do not modify the file name, otherwise it will not be recognized by the Update tool.

- If you have not already done so, connect the appropriate power cords to the Modular Server System power supplies and wait for the chassis management module to fully boot: approximately 3-4 minutes, when the CMM is fully booted the system cooling fans will slow down.

- Prior to loading and using the P2.3.1 unified software update – all server modules in the system must be powered down.

- Only 1 SCM in SCM1 slot can be installed in the chassis. The second SCM can not be installed in the chassis until after the firmware update has been completed.

- Ensure that no Storage Pool or Virtual Drive build or rebuild operation is in progress.  If a build, rebuild, or migration operation is in progress, allow the operation to complete before attempting to update the firmware via the Unified Firmware Update package.

## Unified Firmware Update from P2.2.2 or P2.2.3 to P2.3.1:

- Launch a browser from a client system connected to the CMM

- Login into the GUI and browse to the Firmware tab (bottom selection in the left navigation pane.

- Ensure all servers are powered down, then click on the browse button and browse to the folder P2_3_1 you previously created, select the CMM_Core.zip file and the update will automatically begin.

- The upload process will take several minutes – a progress bar is displayed.

- After the file has been uploaded, the screen will display “A new Firmware File has been uploaded with build version 2.4.100.20080610.11578.

- A box below this message labeled Actions will state – This update can not be started until the following actions have been completed: Reset Storage Control Module 1 into Safe Mode.  Click on the link.

- It will then go to a screen that says Reset Controller SCM1 with a box checked that says “Come back in safe mode after reset.” Type CONFIRM and hit Apply.  The screen will then display “Storage Controller resetting. This may take up to 4 minutes.”

- It will then go to a screen that says “Reset of controller SCM1 to Safe Mode successful”. Hit OK. The screen will then return to the Firmware Update page, and click on the “Start Update” button.

- A dialogue box will appear – select the “Reboot CMM & Start Update” button. (fans will speed up and then return to normal as the CMM restarts). The CMM will take several minutes to boot prior to displaying the Login screen.

- Login to the GUI when prompted.

- Browse to the Firmware tab and scroll down to the Ethernet Switch module 1 line. The Ethernet Switch module line will indicate that the Ethernet Switch module firmware is updating to version 1.0.0.27. The update will take 5 minutes to complete.

- Scroll down to the Storage Controller line item. The Storage Controller 1 line item in the Firmware tab will indicate that the Storage Controller Module is updating to version 2.91.140.50. The update will take 5 - 10 minutes. Do not disturb the system while the update is in progress. While the update is still in process the Firmware tab will indicate that the Storage Controller Module is at 2.91.140.50. Do not interrupt the system while the system fans are at high speed. Wait until the system fans have returned to normal speed before verifying that all updates have been completed.

- Do NOT move the SCM to slot 2 – it should remain in Slot 1 only for all future use.

- Continue with step 15 below under the Verify Updates Completed section

## Verify Updates Completed:

- Login to the GUI and browse to the Firmware tab (bottom selection in the left navigation pane.

- At the top of the Firmware page – verify the Current Build Version is  2.4.100.20080610.11578.

- Verify the Ethernet Switch module firmware line under each installed Ethernet Switch Module shows firmware level 1.0.0.27, boot 1.0.0.6.

- Verify the Storage Control Module firmware level is 2.91.140.50.

- Insure that all Chassis fans are running at normal speed prior to installing the Second SCM in SCM2 slot.

## Install Second SCM in SCM2 Slot:

- Login to the GUI and browse to the Firmware tab (bottom selection in the left navigation pane.

- Install the second SCM in SCM2 slot.

- After the second SCM has booted, the firmware update page will either indicate “Firmware update to this component will begin soon” or “Update to version 2.91.140.50 queued”.

- The second SCM line on the firmware tab will then indicate “updating to 2.91.140.50. The firmware update will take approximately 5 - 10 minutes. During this time the chassis fans will be at high speed and the chassis fault LED will illuminate.

- The Firmware tab will then indicate that the second SCM has been updated to 2.91.140.50, however, the fans will remain at high speed and the chassis fault LED will remain illuminated. The Firmware tab will then indicate “updating expander”. Do not disturb the expander update process which will take approximately 10 minutes.

- At the end of the expander update, a warning message at the top of the screen will indicate “Controller Fail-back in progress. Storage Management functions may be temporarily unavailable.

- The chassis fault LED will stop illuminating and the warning message at the top of the screen will disappear.

- Browse to the event log. The event log should now display “Storage Controller Operational” for Storage Controller 2.

Legal Information

Information in this document is provided in connection with Intel Products and for the purpose of supporting Intel developed server boards and systems.

No license, express or implied, by estoppel or otherwise, to any intellectual property rights is granted by this document. Except as provided in Intel’s Terms and Conditions of Sale for such products, Intel assumes no liability whatsoever, and Intel disclaims any express or implied warranty, relating to sale and/or use of Intel products including liability or warranties relating to fitness for a particular purpose, merchantability, or infringement of any patent, copyright or other intellectual property right. Intel Corporation may have patents or pending patent applications, trademarks, copyrights, or other intellectual property rights that relate to the presented subject matter.

The furnishing of documents and other materials and information does not provide any license, express or implied, by estoppel or otherwise, to any such patents, trademarks, copyrights, or other intellectual property rights. Intel products are not intended for use in medical, life saving, or life sustaining applications. Intel may make changes to specifications and product descriptions at any time, without notice.

Intel is a registered trademark of Intel Corporation.

*Other names and brands are the property of their respective owners.

Copyright © 2007-2008 Intel Corporation.