TSOUND  v1.2

Copyright (C) 1991 PC Enterprises 1-908-280-0025

Permission is granted to distribute TSOUND.COM and TSOUND.DOC

free of charge provided the two programs are distributed

together and no portion of either program and/or the

contained messages have been altered.

Dedicated exclusively to the support of the IBM PCjr since

1984.	PCjr owners can receive a FREE PCjr products catalog by

calling 1-800-922-PCJR

TSOUND.COM is a PCjr utility for controlling the PCjr's

enhanced sound capabilities as well as PC Enterprises' Tandy

Sound Modification which adds Tandy four voice sound.	For

more information about these system board enhancements, call

either of the two phone numbers listed above.

HISTORY OF PC, PCjr, AND TANDY 1000 SOUND SUBSYSTEMS:

To understand what TSOUND.COM can do, you should first

understand more about your PCjr's sound subsystem and how the

PCE Tandy Sound modification adds to those capabilities.

The original IBM PC produced sound through a small speaker

mounted on the system board.  It could play a wide variety of

tones by using a programmable timer chip to modulate the

speaker with digital information, but only at a fixed volume.

Using this design, the PC can play only one tone at a time

and only at one volume level.	PC compatible sound is

therefore primarily used to produce beeps for signaling

certain events.

The PC also has BIOS support to emulate an on-off switch

between the timer and system board speaker.  In this document

file this feature is referred to as the system board speaker

switch.  In most cases, this feature gives software the

ability to turn off the system board sound, however because

the hardware does not actually include a physical switch, the

system board speaker can still beep under certain conditions

such as during an error.

When the PCjr was designed, it was obvious to IBM that the

PC's single voice beeper alone would not impress the home

computer market.  As a result, the PCjr received a special

sound chip which is known as the Complex Sound Generator

(CSG).  This chip can play three tones and one noise

simultaneously.  Each voice of the CSG is completely

independent, each having its own frequency and volume

control.  Nevertheless, even with these new capabilities, it

was still very important to remain 100% compatible with the

PC's sound subsystem since many PC programs were already

coded to use it.

When the PCjr is first powered on, the PC compatible sound

subsystem is automatically selected.  This means the system

board speaker is turned on.  In addition, the PCjr's enhanced

sound subsystem, which always produces sound through the

external speaker, is also set up to produce PC compatible

beeper sound.	When a PC compatible program produces sound,

it will be heard through both speakers.

Software designed to use the CSG can then select the CSG when

necessary, by using a multiplexor which is part of the PCjr's

sound subsystem.  A multiplexor is something like an

electronic switching box that allows one of four inputs to be

selectively connected to the PCjr's external speaker.

The PCjr's sound system, through use of the multiplexor, can

allow the sound from either its programmable timer chip, CSG,

cassette I/O, or external input to be heard through the

external speaker.

If software knows it's running on a PCjr and was designed to

use the CSG, all the software has to do is make the

multiplexor select the CSG instead of the programmable timer

and then send the music data to the CSG.  Since the

programmable timer gets selected automatically by the PCjr

when the computer is powered on, software designed to use the

PC's sound subsystem and not the CSG, works fine.  The

software just sends the PC compatible sound data to the

programmable timer chip without requiring the multiplexor to

change its default setting.  This way the PCjr has the best

of both worlds, PC compatibility and enhanced sound.

When Tandy came out with their 1000 computer, it was clear

that they had based their designs on the PCjr.  They did,

however, improve the sound system.  While retaining the

beeper and the CSG designs, they added a feature which

allows software to select both, the PC compatible beeper and

the CSG sound, at the same time.

The PCE Tandy Sound Modification eliminates this difference.

Once this modification is installed the PCjr, like the Tandy,

it can play four tones concurrently through the external

speaker by using the multiplexor to select an input that has

the timer chip and CSG together on one channel.  Since all

Tandy 1000s were shipped with a diskette drive, there was no

need for cassette recorders, so cassette support was dropped

in favor of four voice sound.	When the Tandy Sound

Modification is in use on the PCjr, cassette support is

disabled.

USING THE TSOUND.COM UTILITY:

Running TSOUND.COM without any parameters will report the

current settings for the sound subsystem.  It will report the

current state of the system board speaker switch and

multiplexor.  It will also report whether the PCE Tandy Sound

Modification has been installed.

Running TSOUND.COM with the /? option will display a help

screen that shows all the options of the TSOUND utility.

The /T option will cause the single voice beeper and all

three voices of the CSG to play tones, testing and

demonstrating the current settings of the computer's sound

subsystem.  Whether the sounds being played can be heard

through the external speaker or system board speaker is

determined by the state of the multiplexor and the state of

the system board speaker switch.

If you have the PC Enterprises' Tandy Sound Modification and

are about to run a Tandy program that supports Tandy four

voice sound, run the TSOUND utility with the /4 option.  This

will permit the PCjr to produce up to four sounds through the

external speaker (This option allows Tandy software to

produce one, two, three, or four voice sound).

There is usually no need for selecting the External Input

option (/E) for other than experimentation or for turning off

the external speaker.	At this time, this input is selected

only by programs that support the IBM PCjr Speech Attachment.

Future PCE sound products will most likely make use of this

input also.

There is also no need to select the Cassette (/C) option for

other than experimentation.  Programs that use cassette

drives will automatically select this channel to store and

retrieve data from cassette as necessary.

The IBM PCjr sound subsystem:

/�

������������/ �����������������������������������< �

�  System board speaker switch			 \�

�				   System board speaker

�	����������������������ͻ

Programmable Timer ������������Ķ		      O�

�I		      u�		 /�

CSG ���������������������������Ķn    Multiplexor     t�		/ �

�p		      p����������������<  �

Cassette I/O ������������������Ķu		      u�		\ �

�t		      t�		 \�

External input ����������������Ķ		       �       External speaker

����������������������ͼ

Note: These diagrams may not be able to be duplicated on some printers.

To print a copy of this document enter the following command at the DOS prompt:

COPY TSOUND.DOC PRN: