If the game runs perfectly  with smooth animations,  there is no need for

you to read this section.

This game  has been designed  to run perfectly  from a CD drive  that can

sustain a 150K bytes per second transfer rate.

The data transfer rate is measured during the Installation of the game.

If the measured rate is inferior to the specifications

of your CD drive, check the following points:

1. Do you use a portable SCSI interface plugged in the printer port?

These boxes  are  very convenient but usually reduce the transfer rate

to about 75K bytes.

2. Check  the installation of the drive,  especially the interrupt number

(see below), the port number.

IF THE ANIMATIONS PAUSE FROM TIME TO TIME:

First check the installation of your CD drive.

1. Interrupt number:

Some CD drives come with a default installation on Interrupt 3,  which is

not so good  if you have two serial ports on your PC.  In that case,  the

drive may work but the data transfers will be inconsistent.

The basic principle about Interrupt numbers is that all connected devices

must use a different number.

Example:  You have a sound board and a separate CD interface card in your

PC:  Choose  "interrupt 7"  for the soundboard, "interrupt 5"  for the CD

card. This is perfect  if you don't have any other cards  that use either

of these two interrupts.

Problems may arise if you have lots of cards in your PC, each one needing

a different interrupt number.  The best way to sort this out is to have a

program  that  lets  you  choose  a configuration  when  you boot  the PC

(i.e.  MCONFIG  from McAfee  is a good  shareware program  of this type).

This way, you can choose to install the Devices you need without fiddling

continuously with your CONFIG.SYS file.

2. Additional buffers:

There are two kinds of additional buffers used when reading a CD-ROM:

a) Buffers for the CD device.

b) Buffers for MSCDEX.

a) Buffers for the CD device.

The CD device is installed  by a line  in your CONFIG.SYS file. With some

CD drives, you may specify the number of additional buffers.

Check with your CD drive manual,  but don't worry  if you don't have this

feature.  It is rather a good sign  as it means  that  your CD drive  has

an internal buffer which is large enough.

The number of additional buffers is usually specified by a "/M:xx" option

in the line installing the device in your CONFIG.SYS file.

Try  changing  this option  and see  if there is any  improvement  in the

smoothness of the animations.

Be aware  that these buffers  reduce  the amount of memory  available for

other programs.

b) Buffers for MSCDEX.

MSCDEX  is a program  usually installed by a line  in  your AUTOEXEC.BAT.

It allows  DOS  to communicate  with your CD device.  You can specify the

number of buffers with the "/M:xx" option.

These  buffers  are  used  to memorize  the  contents  of  the  different

directories  of  your  CD-ROM.

If you have EMS memory,  add the "/E" option  so that these buffers don't

eat into your main memory.

3. Installation options of this game.

Choose  manual configuration  in the INSTALL program of this game to view

the options currently selected.

"Extra Memory" options:

The game  will run  much faster if you have extra memory (EMS or XMS)

If you have  something  between 350k  and 1M bytes of extra memory to

offer, then this is perfect.

"Animations" options:

This game uses a very special technique to display animations as smoothly

as possible even when the reading of the CD-ROM is "irregular".

If animations stops a little sometimes,  try the option  "Try every trick

to be very smooth". A more advanced but risky technique will be used.

If the animations lock, select "Standard smooth animations" option for

Animations.

NOTES ON THE TRANSFER RATE MEASUREMENT:

The "TEST CD DRIVE SPEED" option has been included in the INSTALL program

of this game  mainly to help you  to fine tune  the installation  of your

CD-ROM drive.

It measures:

1. The  data transfer speed,  calculated  as  the CD-ROM  is continuously

read.  This is  the most  important  specification  of a CD-ROM drive,

even more important than the average access time.

2. The regularity of the data transfer during an animation.

100% means  that the CD-ROM drive  always needs approximately the same

time  to read  each frame.  This is excellent  as small irregularities

will be corrected by the game.

But  if  you come out  with  less  than 100%,  you  should  check  the

installation of your CD drive again to try to improve this.

If you run  the test  several times,  you may not always  have exactly

the same result, especially if it's below 100%.

A  disk  cache  buffer  larger than 4Mb  might give false results  if you

use the "TEST CD DRIVE SPEED" option more than once.

Note that Smartdrive 4.0 ((c) Microsoft) doesn't cache the CD-ROMs.

Some  caddyless  drives  seem  to loose  the  grip  on the disk  when the

motor  is  stopped   after  several  minutes  of  inactivity   (typically

between 5 and 15 minutes of inactivity).  In that case,  the disk doesn't

spin correctly when the motor is restarted and you can hear an unpleasant

clicking noise coming from the CD-ROM drive.  There is no other solutions

then  than to open the drawer  and  close it again.  This problem  is not

a malfunction of the game.