To use ROGUE, you'll have to have an MS-DOS machine with some kind of

terminal emulation available either through the console driver or through

firmware.  For IBM-PCs and compatibles, you can use ANSI.SYS, or its faster

cousins NANSI.SYS or FANSI.SYS.  The termcap file included with this archive

already has an entry called 'ibmpc-mono' which will work with ANSI.SYS.

It also has an entry for the HP 150, which is what I used to port this version

of ROGUE.

If you don't have either of these options, you can create your own termcap.

A termcap consists of a series of lines with a terminal name at the beginning

and the terminal characteristics on the rest of the line.  Each terminal

characteristic is a two character mnemonic followed by the information for

that terminal.  Here is the hp150 termcap as an example:

# HP 150 termcap

hp150:\

:li#24:co#80:cl=\EH\EJ:cm=\E&a%dr%dC:do=^J:se=\E&d@:so=\E&dB:\

:te=\E&jB:ti=\E&j@:up=\EA:

You can put comments in the termcap file on lines beginning with #.

Since most termcap entries are longer than one 80 column line, you can continue

the entry onto another line by placing a \ at the end of the preceding line.

Make sure not to place a backslash on the line with the last terminal

characteristic entry.

You may enter non-printing characters in these ways:

\E is ESC (ASCII 27)

\n is LF (ASCII 11)

\r is CR (ASCII 13)

\b is BS (ASCII 8, backspace)

\t is TAB (ASCII 9)

\nnn means the character with octal code nnn

^X is that control character (make sure it's in upper case)

all other characters have their standard meanings

Here is a description of the termcap terminal characteristics that ROGUE

recognizes.  You must follow each with the control sequence that performs

the indicated action on your terminal or the requested information.

cl=	clear screen

cm=	cursor move

You can encode the row and column to go to in

these ways:

%d output row and column as ASCII digits, i.e. 39

%2 output row and column as two-digit ASCII number

%3 output row and column as three-digit ASCII number

%. output row and column as ASCII character, i.e.

column 65 would be encoded as ASCII 'A'

%+x add character x to row or column number, then

output like "%.".

%i increment row and column numbers by 1 (given in

addition to normal specifiers above, i.e.

%i%d,%d)

%r reverse row and column (normal order is

row, column; given in addition to

normal specifiers above, i.e. %r%d,%d)

Examples:

To move to row 5, column 60 on an HP 150, you

send \E&a5r60C.  The general cm string for the

HP 150 is "\E&a%dr%dC".

To move to row 5, column 60 on a VT-52, you

send \EY\005\074 (using the conventions for

non-printable characters given above).  The

general cm string is "\EY%.%.".

The ADM-3A encodes the row and column in single

characters by adding the value 32 (SPACE) to

each.  Its cm string would be "\E=%+ %+ ".

%i is used if the upper left corner is

addressed as 1,1 instead of 0,0.

You must use the same encoding for both the

row and column.

do=	cursor down one line (no carriage return)

up=	cursor up one line

li#	number of lines on your screen

example: li#24

co#	number of columns on your screen

example: co#80

se=	end standout mode

so=	start standout mode (i.e. inverse or underlined text)

te=	terminal de-initialization string

ti=	terminal initialization string

ve=	return cursor to normal

vs=	make cursor more visible (if possible)