Equinox

One Equinox Way

Sunrise, Fl. 33351-6709

(305) 746-9000

Equinox P/N 650155

03/28/97

Release Notes

for

Equinox SuperSerial

SST-4I,  SST-8I,  SST-64I,    SST-128I,

SST-4E,  SST-8E,  SST-64E,    SST-128E,

SST-4P,  SST-8P,  SST-64P,    SST-128P,

SSM-12I, SSM-24I, SSM-12E,    SSM-24E,

SST-16I, SST-16P, SST-8I/RJ

Host Controllers

BSDI BSD/OS Driver

Version: 3.32

Equinox P/N 650155

Equinox, Equinox SuperSerial are trademarks of

Equinox.  BSD/OS are registered trademarks of

Berkeley Software Design, Incorporated.  All other brand and product

names are the trademarks of their respective holders.

Copyright (c) Equinox Systems, Inc 1995

All Rights Reserved

Information in this document is subject to change without notice

and does not represent a commitment on the part of Equinox.

Equinox provides this document "as is", without warranty of any

kind, either expressed or implied, including, but not limited to,

the particular purpose.  Equinox may make improvements and/or

changes in this manual or in the product(s) and/or the program(s)

described in this manual at any time.

This product could include technical inaccuracies or typographical

errors.  Changes are periodically made to the information herein;

these changes may be incorporated in new editions of the publication.

RESTRICTED RIGHTS:

For non-U. S. Government use:

These programs are supplied under a license.  They may be used,

disclosed, and/or copied only as permitted under such license

agreement.  Any copy must contain the above copyright notice and

this restricted rights notice.  Use, copying, and/or disclosure

of the programs is strictly prohibited unless otherwise provided

in the license agreement.

For U.S. Government use:

Use, duplication, or disclosure by the Government is subject to

restrictions as set forth in sub-paragraph (c)(1)(ii) of the

Rights in Technical Data and Computer Software clause of DFARS

52.227-7013.

1.0  Introduction

1.1  Overview

Equinox provides support for BSD/OS applications

through a universal driver conforming to the BSD/OS Driver

specification. Copy the driver files and rebuild the kernel

to install this driver. The user

should consult the Hardware Manual and Device Driver

Software Installation & Reference Manual (Unix PN 560084/B)

for SST SuperSerial Technology prior to installing the hardware.

The Equinox BSD/OS driver follows and supports the same

features as described in the Device Driver Software Installation &

Reference Manual (PN 560084/B for SVR3.2 & SCO UNIX & XENIX

Operating Systems) except as noted in this document.

1.2  Compatibility

The Equinox SuperSerial BSD/OS driver supports the Equinox

SuperSerial SST-4I, SST-8I, SST-64I, SST-128I, SST-4E,

SST-8E, SST-64E, SST-128E, SST-4P, SST-8P, SST-64P, SST-128P

SSM-12I, SSM-24I, SSM-12E, SSM-24E, SST-16I, SST-16P, and SST-8I/RJ

boards and therefore can be used on ISA, EISA, or PCI servers.

The Equinox based Micro Channel host boards are not supported.

The driver supports up to a total of 8 Equinox SuperSerial boards

of any type, for a total number of 1024 channels (ports). However,

a current BSD/OS limitation prevents more than 364 channels from

being open simultaneously, including any open consoles.

2.0  Installation

To install the Equinox SuperSerial BSD/OS driver, the following must

be done: (Note: you must be root, superuser )

1)  Mount the diskette "mount -o ro /dev/fd0 /a"

2)  Install the software "installsw"

3)  Answer the installation questions

2.1  SST-4I, SST-8I, SST-64I, SST-128I, SSM-12I, SSM-24I, SST-16I,

SST-8I/RJ Installation

Equinox SuperSerial SST-4I, SST-8I, SST-64I, SST-128I, SSM-12I,

SSM-24I, SST-16I, or SST-8I/RJ boards may be installed in either

ISA or EISA platforms.

For each Equinox SST ISA based board a configuration statement

will be added to your equivalent of the file /sys/i386/conf/LOCAL

eqnx0 at isa? port 0x220 iomem 0xd0000

Note: The number following eqnx, "0" increases with each defined board.

The port hex number "0x220" is defined by the dip switch on the board.

The hex address "0xd0000" is a 16k memory location to map the

board(s). The iomem address must be the same for all Equinox ISA

boards. The other numbers must be different for each board.

When the driver is loaded the memory location specified is checked to see

if it is unused. If it is the board(s) are mapped.

If the memory address for iomem is 0x00000, then an Equinox driver hole

finder will run. The locations between 640K - 1Meg are examined and

when an unused location is found the board(s) are mapped to this new

address.  A console message will be displayed stating that the memory

location specified has changed.

eqnx: Note Memory address (%x) changed to (%x)

2.2  SST-4E, SST-8E, SST-64E, SST-128E, SSM-12E, or SSM-24E Installation

Install the Equinox SuperSerial SST-4E, SST-8E, SST-64E, SST-128E,

SSM-12E or SSM-24E boards only on an EISA platform.

Run the EISA configuration utility prior to loading the driver.

This defines the Equinox board type and memory

address. The Equinox ".cfg" files (found on a disk supplied

with the board) should be copied to the utility

disk which contains the EISA configuration software.

If the EISA configuration is not done or done incorrectly the

driver will display an error message for the board which was not

configured. The board can not be used and will be skipped if an

attempt is made to load it. Since all the configuration for the

EISA boards is in CMOS, the user need not supply any port

parameters.  Equinox SuperSerial EISA boards are loaded in slot

order, the lowest slot first.

For each Equinox SST EISA based board in page mode, a configuration

statement will be added to your equivalent of the file

/sys/i386/conf/LOCAL

eqnx0 at eisa?           iomem 0xd0000

Note: The number following eqnx, "0" increases with each defined board.

The hex address "0xd0000" is a 16k memory location to map the

board(s).

If the iomem address of 0x00000 is given, then the Equinox driver's hole

finder will map the board, otherwise the address given will be used.

For each Equinox SST EISA based board in "flat mode", a configuration

statement must be added to your equivalent of the file

/sys/i386/conf/LOCAL

eqnx0 at eisa?

Note: The number following eqnx, "0" increases with each defined board.

2.3  SST-4P, SST-8P, SST-64P, SST-128P, or SST-16P Installation

Install the Equinox SuperSerial SST-4P, SST-8P, SST-64P, SST-128P,

or SST-16P boards only on a PCI platform.

Note: BSD/OS 2.0 does not support PCI based boards. Use BSD/OS 2.1 and

higher for PCI support.

For each Equinox SST PCI based board, a configuration statement will

be added to your equivalent of the file /sys/i386/conf/LOCAL

eqnx0 at pci?

Note: The number following eqnx, "0" increases with each defined board.

3.0  General Information

Probe

-----

Each time the driver's probe routine is called it checks the supplied

configuration information against the board's information.

if a match is found the result is returned and later on the board will

be "ATTACHED". If no match was found the probe will result in an error.

Boards that have no associated probe configuration will be ignored.

Thus, if the configuration file has an ISA board at port 0x220 but

the board is strapped for 0x240, no match is found, and that board

will be unavailable.

All Equinox SST boards use the same major number "31".

Each board uses 2 unit numbers which are defined by the

eqnx# (specified in the configuration file). (#*2+1)

For example:

eqnx0 at isa? port 0x220 iomem 0xd0000

will result in unit 1 and 2

0 crw-rw-rw-  1 root  wheel  31, 1, 0 Sep 18 09:29 /dev/tty1a1

0 crw-rw-rw-  1 root  wheel  31, 1, 1 Sep 18 13:24 /dev/tty1a2

0 crw-rw-rw-  1 root  wheel  31, 1, 2 Sep 18 09:29 /dev/tty1a3

0 crw-rw-rw-  1 root  wheel  31, 1, 3 Sep 18 09:29 /dev/tty1a4

0 crw-rw-rw-  1 root  wheel  31, 1, 4 Sep 18 09:29 /dev/tty1a5

0 crw-rw-rw-  1 root  wheel  31, 1, 5 Sep 18 09:29 /dev/tty1a6

0 crw-rw-rw-  1 root  wheel  31, 1, 6 Sep 18 09:29 /dev/tty1a7

0 crw-rw-rw-  1 root  wheel  31, 1, 7 Sep 18 09:29 /dev/tty1a8

eqnx1 at isa? port 0x240 iomem 0xd0000

will result in unit 3  and 4

0 crw-------  1 root  tty    31, 3, 0 Sep 20 10:45 /dev/tty2a1

0 crwxrw-rw-  1 bart  tty    31, 3, 1 Sep 15 17:59 /dev/tty2a2

0 crw-rw-rw-  1 root  wheel  31, 3, 2 Sep 15 17:59 /dev/tty2a3

0 crw-rw-rw-  1 root  wheel  31, 3, 3 Sep 15 17:59 /dev/tty2a4

0 crw-rw-rw-  1 root  wheel  31, 3, 4 Sep 15 17:59 /dev/tty2a5

0 crw-------  1 root  tty    31, 3, 5 Sep 15 17:59 /dev/tty2a6

0 crw-rw-rw-  1 root  wheel  31, 3, 6 Sep 15 17:59 /dev/tty2a7

0 crw-rw-rw-  1 root  wheel  31, 3, 7 Sep 15 17:59 /dev/tty2a8

Individual devices "tty names" are defined by minor numbers

(see Equinox SST, Device Driver Software Installation & Reference Manual)

3.1    Baud Rates

In some environments only a limited number of baud rates are available.

The Equinox driver implements high baud rates in those environments

by mapping the high baud rates to low baud rates, making the low baud

rates unavailable for use. In BSD/OS baud rates are a pure number,

so mapping high baud rates to low baud rates is not need. Terminal

rates above 115,200, such as 230,400  need to be defined in /etc/gettytab

if not already defined.

Top baud rate depends on the board type:

sst-8x  (230,400)

sst-4x  (460,800)

sst-16x (460,800)

sst-64x or sst-128x (depends on the port module type:

pm-8(230,400), pm-16(115,200 )

ssm-12/24 (115,200)

3.2   Equinox support files

sstty and ssmkn can be found in "/usr/sbin"

The equinox directory where ssmkn "log" and "database" file

can be found in "/var/db/equinox"

3.3  Notes

1) If you experience problems with some applications which are

dependent on the line discipline, (such as tip), the port may

be in block mode "bypass mode". The mode can be changed by the

command "sstty +lo port-name","sstty -s port-name" will displayed

the mode:

Line Discipline                    Block Mode

-------------			    ---------

rmode = 0    (read)               rmode = 2

wmode = 0    (write)              wmode = 2

2) Don't forget to set rts and cts flow control if hardware flow is needed

"stty rts_iflow cts_oflow"

3)  PPP / SLIP

Hardware flow control should be defined (rts_iflow cts_oflow).

If using software flow control (XON/XOFF) make sure the port

has the setting of "ixon" "ixoff" defined.

4.0  Not Supported Features

The following are not supported:

Equinox SST Micro Channel based hardware

Transparent Print

Alternate Session

5.0  Messages

5.1 Sample Messages

The following  messages may occur during the installation and/or

operation of the  driver.

Equinox SuperSerial Technology Device Driver, BSDI Release 3.32

Copyright (c) 1996 Equinox Systems Inc. All Rights Reserved.

eqnx: Probe in bus(0) for iobase 0x240 maddr 0

eqnx: Testing board at I/O address 0x240, SSP(0), please wait.

eqnx1 at isa0 iobase 0x240 maddr 0xf0115000-0xf0118fff

eqnx: ATTACH of eqnx1 (SST-8I  ), Driver Enabled for SST board#(0), unit(1)

5.2 Additional Messages

The following  messages may occur during the installation and or

during the operation of the  driver.

eqnx: test_8bit port (%x) addr (%x)

eqnx: Testing board at I/O address 0x%x, SSP(%d), please wait.

eqnx: Testing board in Slot %d, SSP(%d), please wait.

eqnx: ISA holes at(%d) addr(%x) status (%x) MAPPED!

eqnx: Found a hole at %d

eqnx: ISA 8bit Address %x as 16 bit wide hole.

eqnx: Found (%d) SST boards

eqnx: Memory mapping PASSED for board %d addr(%x)

eqnx: Probe in bus(%x) for iobase 0x%x maddr %x

eqnx: Note Memory address (%x) changed to (%x)

eqnx: ATTACH of dev (%s) at %x\

eqnx: ATTACH of %s, Driver Enabled for SST board#(%d)

5.3 Error Messages

The following error messages may occur during the installation of the

driver.

eqnx: PRAM memory test failure %d for board with I/O address 0x%x.

eqnx: PRAM memory test failure %d for board in Slot %d.

eqnx: DRAM word memory test failure %d for board with I/O address 0x%x.

eqnx: DRAM word memory test failure %d for board in Slot %d.

eqnx: DRAM tags memory test failure %d for board with I/O address 0x%x.

eqnx: DRAM tags memory test failure %d for board in Slot %d.

Run EISA configuration utility for Equinox board in slot %d.

eqnx: Error EISA board in flat mode

eqnx:ISA: No 8/16 bit wide holes found.

eqnx: No Equinox SST EISA boards found

eqnx: No Equinox EISA board found in slot iobase 0x%x

eqnx: Invalid iobase 0x%x

eqnx: No Equinox board found at iobase 0x%x

eqnx: No Equinox boards found.

eqnx: Error, No iobase given, config ignored

eqnx: No SST device found for iobase %x

eqnx: ATTACH, fail to malloc memory for port structure

eqnx: ATTACH, fail to malloc memory for tty structure

EQNX: SSP %d not detected for board with I/O address%d.

6.0  Known Problems/Issues

1) software manual

2) ssdiag

When using the topology feature, LMX modules remain highlighted

when selecting another module.

7.0  Modifications

Version 3.33

1) Fixed a potential port hang that can result when OPOST and

TAB3 are enabled.

2) Incorporates updated versions of Modem Pool services that

configure the FIFOs on 16550 compatible modems. This provides

for more reliable operation.

3) Added support to detect a buffer overflow on SSP4 based boards.

A port could hang if flow control were enabled and exactly

enough data arrived to fill the buffer.

Version 3.32

1) NOTE: 3.31 was released only to select Beta sites.  All

modifications to 3.31 deemed sound are included in 3.32

2) Incorporates updated versions of Modem Pool services that

correctly configure some slower initializing Plug and Play

(PnP) modems.

3) Fixed an earlier regression that caused SIGINT to not be sent

upon receipt of a Break when BRKINT is configured.  Recognize

Breaks on Modem Pool Ports.

4) Correctly report parity and framing errors to the line discipline

when parity checking is enabled via termio.

Version 3.30

1) SST-16I, SST-16P, SST-8I/RJ, SST-4I now supported.

2) Changed SSP4 minor numbers from skipping to next 64

to being sequential.

3) Added setting and clearing of DTR and RTS output overload

control signals.

4) Added flush of modem pool I/O buffers when carrier drops.

5) Set hardware flow control on modem pool if CCTS_IFLOW or

CCTS_OFLOW are set.

6) Reset channel diagnostic command now clears I/O buffers

for non-modem pool channels.

7) Added a timer to prevent a close from sleeping forever if

output was in progress and the LMX was removed.

8) Fixed problem of modem pool channel not responding by adding

a check for completion of modem pool service calls before

processing receive events.

9) Fixed a conflict with the modem pool control signals.

10) Stabilized ssdiag topology screens to reduce extraneous

characters by repainting screen more frequently.