***********************************************************

*                                                         *

* EISALink document file                                  *

* [OCEAN - EISALink EISA 32-bit SONIC Network Controller] *

*                                                         *

***********************************************************

The following subdirectories and files are included in this

software package:

!PLX1001.CFG    -       EISALink configuration file

README          -       Novell NetWare 3.11 and ODI drivers

informations

NOVELL.22       -       Subdirectory contains

Novell NetWare 2.2 drivers

NOVELL.311      -       Subdirectory contains

Novell NetWare 386 v3.11 drivers

NDIS            -       Subdirectory contains

Microsoft LAN Manager's NDIS drivers

***********************************************************

*                                                         *

* Procedures for configuring EISALink using !PLX1001.CFG  *

* EISA configuration file                                 *

*                                                         *

***********************************************************

The EISALink EISA 32-bit SONIC Network controller fully supports

the EISA automatic configuration feature, and is operational with

most computers using the default settings.  The configuration

will need to be changed if conflicting port assignments or memory

allocation is encountered.

The EISALink configuration file, !PLX1001.CFG is included in this

floppy disk.  To configure, perform the following steps:

1. Copy the EISALink !PLX1001.CFG file onto your configuration

diskette or configuration directory of your harddisk

provided with your system computer.

2. Run your own configuration program came with your compter

system.

3. Follow the instructions on your screen to select the

"Configure Computer" option.  An "Auto Added" prompt will

advise you that the EISALink board has been installed into

a previously empty EISA slot.

4. Follow the instruction on your screen to select the

"OCEAN EISALink EISA 32-bit SONIC Network Controller" and

select the desired configuration.