FUNCTIONAL SPECIFICATION:

EXTENDED ISA

CONFIGURATION LANGUAGE

FOR ISA BOARDS

Version 1.00

January 4, 1989

BCPR SERVICES, INC. *

* Agent for dissemination of EISA Specification

1 Introduction

This document describes a subset of the Extended Industry Standard Architecture (EISA)

Board Configuration File Language, hereafter referred to as the CFG language. This

subset of the CFG language is capable of describing the configuration requirements of ISA

adapter boards.

The purpose of the CFG language is to provide adapter board configuration information to

a system configuration utility. An adapter board's configuration information includes a list

of system resources used by each of the board's possible configurations. System resources

include IRQs, DMA channels, I/O address space, and memory address space.

The configuration utility uses the information provided by each adapter board's CFG

language file to configure the system while avoiding system resource conflicts. After

configuration, the user may view initialization requirements, including switch and

jumper settings, for each adapter board.

2 Notational Conventions

Configuration language files (CFG files) consist of free-form ASCII text. Whitespace,

including spaces, tabs, carriage returns, and linefeeds, is ignored outside of quoted strings.

It is recommended that blank lines and indentation be used as necessary to increase the

readability of the file.

Within a given block, any statements that are required must be placed in front of those

statements that are indicated as optional.

2.1 Values and Addresses

A <value> or <address> may be given in hexadecimal, decimal, or binary. The radix, or base, is

specified by attaching one of the following characters to the end of the <value> or <address>:

H - Hexadecimal

D - Decimal

B - Binary

Upper or lower case is accepted. The radix character must be placed immediately after the

value, with no blanks in between. Example: 1F00h. If no radix is specified, decimal is

assumed.

Hexadecimal numbers beginning with a letter must have a leading zero. Example: 0C000h

Slot-specific EISA port addresses must be prefixed by a 0Z. The Z represents the 4-bit slot

specific portion of these addresses. Example: 0Z400h

Binary <values> are specified left to right, most significant bit to least significant bit.

Example:

XXXXXXXX

76543210

Page 1

A <value> or <address> may also be given in units by attaching one of the following unit

abbreviations to the value. Upper or lower case is accepted.

K - Kilobytes

M - Megabytes

Units may only be used with decimal numbers. A radix may not be specified in addition to

a unit. The unit and value may be attached or separated by a blank. Example: 512K or

512 K.

Fractions are not accepted.

A null <value> can be indicated with empty braces: {}

2.2 Ranges

A <range> is defined with a hyphen: <value - value>

A <range> may be either ascending or descending. Ascending <ranges> are specified: <low value

- high value>. Descending <ranges> are specified: <high value - low value>.

Blanks may or may not be used within a <range>. Example: 300h-30fh or 300h - 30fh.

2.3 Lists

Lists consist of items such as <values>, <addresses>, text fields, etc., that are delimited with

spaces and vertical bars. Spaces indicate AND while vertical bars indicate OR.

Example: (x and y) or (y and z) would be denoted: x y | y z

A <rangelist> is defined as a list of either <ranges> or <values>. Example: 1-3 | 4-6. This

example is the equivalent of: 1 2 3 | 4 5 6, which can also be considered a <rangelist>.

<Ranges> and single <values> may not be mixed in a list. Example: 1-3 4 5 6 is illegal

syntax.

Ascending and descending <ranges> may not be mixed in a list. Example: 1-3 | 6-4 is illegal

syntax.

<Switchlists> and <jumperlists> are lists of switch and jumper numbers, respectively. These lists

are delimited only with blanks. Vertical bars may not be used. <Ranges> of switch or jumper

numbers may also be used.

2.4 Text

Many statements in the configuration file language contain text fields. These text fields are

free form and are enclosed in quotes. Maximum lengths are given for each instance.

Free form text may contain embedded tabs, denoted by \t, and embedded linefeeds,

denoted by \n. Quotes and backslashes may be placed in the text using \" and \\

respectively.

Embedded tabs are expanded to the next tab stop. Tab length is eight characters (tab stops

are located at 1, 9, 17, etc.).

Page 2

Whitespace, defined as ASCII characters 09h - 0Dh and 0X20h, is ignored at the beginning

and end of text fields when multiple lines are required.

Example: KEYWORD = "This multiple line text field will be

displayed without the multiple spaces,

tabs, carriage returns, and linefeeds

found at the beginning and end of each

line."

An empty text field is indicated with double quotes: ""

2.5 Comments

;

;The semicolon will be used to place comments in the CFG file.

;Comments on each line must be preceded by a semicolon.

;These comments will not be displayed to the user.

;

2.6 Symbols Used in Syntax Description

The following special symbols are used in this description of the configuration file syntax:

[]              The item or statement is optional.

x | y           Either x or y is allowed.

Italicized words represent variables.   ;Note: <> used here in place of italics.

Upper case characters and words are keynames and must be entered as shown.

3 Configuration File Outline

Board Identification Block

Initialization Information Block(s)

Function Statement Block(s)

The Board Identification Block is used to identify and characterize the board. This block

must come first in the CFG file.

Initialization Information Blocks define the programmable ports, switches, jumpers, and

software used to configure the board. All Initialization Information Blocks must be located

after the Board Identification Block and before the first Function Statement Block.

Function Statement Blocks are used to define each function of the board. A Function

Statement Block lists the different alternatives, or choices, for configuring a function. Each

choice lists the global resources associated with its configuration alternative, in addition to

the values needed to initialize that configuration. The initialization values may refer to any

programmable I/O ports, switches, jumpers, or software defined in the Initialization

Information Blocks.

Page 3

4 Board Identification Block

BOARD

ID = "AAAXXYY"

NAME = "<name>"

MFR = "<name>"

CATEGORY = "<board category>"

[SLOT = ISA8 | ISA16 | ISA8OR16 | EISA | EMB(n)]

[LENGTH = <value>]

[SKIRT = YES | NO]

[READID = YES | NO]

[LANGUAGE = EN | FR | GR | IT | SP]

[COMMENTS = "<information>"]

[HELP = "<information>"]

The BOARD statement appears at the beginning of each CFG file. This statement, along

with the other required statements, must appear before the optional statements contained

in brackets [].

ID is a required statement containing the 7 character board id. This id is used to identify

the board and to name the CFG file. The id consists of a 3 character vendor id (AAA), a 2

character hex product id (XX), and a 2 character hex revision number (YY). The id must

contain 7 characters and must be placed in quotes.

NAME is a required statement used to identify the product. Vendor and product name

should be included. Revision and part numbers may also be included. A maximum length

of 60 characters is recommended.

MFR is a required statement used to specify the board's manufacturer. A maximum length

of 30 characters is recommended. This statement will be uppercased for display.

CATEGORY is a required statement used to identify the board by category. The <board

category> must consist of three characters enclosed in quotes. These three characters will be

uppercased. The categories are:

COM     Communications Board

MEM     Memory Expansion Board

MFC     Multi-function Board

MSD     Mass Storage Device

NET     Network Board

NPX     Numeric Coprocessor Board

PAR     Parallel Port Board

PTR     Pointing Device

SYS     System Board

VID     Video Adapter Board

OTH     Other

Page 4

SLOT specifies the board's expansion slot type. This statement is optional. The default is

ISA16.

ISA8 indicates that the board is 8-bit ISA.

ISA16 indicates that the board is 16-bit ISA.

ISA8OR16 indicates that the board is 16-bit ISA but will also operate in an 8-bit

slot.

EISA indicates that the board requires an EISA slot.

EMB(n) specifies an embedded slot. Embedded slots are defined under Advanced

Topics in section 7.4.

LENGTH is an optional statement that specifies the length of the board in millimeters. A

unit abbreviation should not be used. Fractions are not accepted. Example: LENGTH =

330 (for a full length board). If omitted, board length will not be used as a criteria for

placement within the system.

SKIRT specifies whether or not the board has a lower extension that prevents it from

fitting into larger size expansion slots. This statement is optional. The default is NO.

READID is an optional statement that specifies whether or not the board has a readable

id. The default is NO.

LANGUAGE is an optional statement that specifies the language used in the CFG file.

The default is English. The language choices are:

EN      English

FR      French

GR      German

IT      Italian

SP      Spanish

COMMENTS is an optional text field containing additional information about the board.

This text field may contain a maximum of 600 characters and will be displayed in a window

at least 40 columns wide when the board is selected.

HELP is an optional text field containing information that will be displayed to the user if

he requests help while configuring the board. This text field may contain a maximum of

600 characters and will be displayed in a window at least 40 columns wide.

5 Initialization Information Block

There are four types of Initialization Information Blocks: Programmable I/O Port

Identification Blocks, Switch Identification Blocks, Jumper Identification Blocks, and

Software Identification Blocks. A configuration file may contain several of each type of

these blocks. Each type of Initialization Information Block contains an index used to

differentiate it from others of the same type. These indices will be used by INIT

statements, discussed later, to reference the blocks. Each type of block should be

numbered independently of the other types. Each type of block should also be numbered

beginning with 1.

Page 5

5.1 Programmable I/O Port Identification Block

IOPORT(i) = <address>

[SIZE = BYTE | WORD | DWORD]

[INITVAL = <value>]

In the statement, IOPORT(i) = <address>, i is an index used to differentiate this

Programmable I/O Port Identification Block from others. <Address> is the address of the

programmable port. Slot-specific EISA port addresses must be prefixed with a 0Z.

SIZE is an optional statement that gives the addressability of the port. The default is

BYTE.

INITVAL is an optional statement used to specify the source of a port's bit values. The

<value> is binary only: it is not necessary to specify the radix. A bit position marked with a 1

or 0 indicates that the bit is reserved and should be set to that value. A bit position marked

with an 'x' indicates that the bit will be set by the configuration program. A bit position

marked with an 'r' indicates that the bit value should be read from the port. Each of the

port's bits must be defined by one of these characters. If this statement is omitted, all bit

values that are not set by the configuration program will be read from the port.

Example:

IOPORT(1) = 0ZC81h

SIZE = BYTE

INITVAL = rr10xxxx

There may be instances in which an expansion board requires a single port to receive

multiple initialization values in sequence. This can be accomplished by defining the same

port multiple times. A separate I/O Port Initialization Block would be included for the

port for each sequential initialization value it needs to receive. These separate IOPORT

statements would have different indices so that they can be individually referenced. During

power-up, the initialization values are written to the ports in the order in which they are

defined in the configuration file. In this way, the single port can receive the sequence of

initialization values it requires.

5.2 Switch Identification Block

SWITCH(i) = n

NAME = "switch name or description"

STYPE = DIP | ROTARY | SLIDE

[VERTICAL = YES | NO]

[REVERSE = YES | NO]

[LABEL = LOC(<switchlist>) <textlist>]

[INITVAL = LOC(<switchlist>) <valuelist>]

[FACTORY = LOC(<switchlist>) <valuelist>]

[COMMENTS = "<information>"]

[HELP = "<information>"]

In the statement, SWITCH(i) = n, i is an index used to differentiate this Switch

Identification Block from others. Switches should be numbered beginning with 1. For dip

switches, n gives the number of switches in the set. For rotary and slide switches, n gives

the number of positions on the switch. The maximum value for n is 8 for rotary switches

and 16 for slide and dip switches.

Page 6

Several statements in the Switch Identification Block use the LOC(<switchlist>) clause to

reference individual switches within the block. A <switchlist> is defined as a list of switch

numbers, delimited with blanks. This list may be either ascending or descending, but not

mixed. Example: LOC(1 3 5) may be used, but LOC (1 5 3) is illegal syntax. A range may

also be used in place of <switchlist>. Example: LOC(1-4) or LOC(4-1).

A dip switch <value> can be 0, indicating OFF, 1, indicating ON, or X, indicating a "don't

care". A slide or rotary switch value consists of the position number the switch is set to.

Switch <values> do not require radix identifiers: binary is assumed for dip switches and

decimal is assumed for rotary and slide switches.

A <list> of dip switch <values> may not be delimited. Example: LOC(1-6) xx1101

NAME is a required statement giving either the manufacturer's name or a description for

this switch. This is important as a reference for the user. A maximum of 20 characters is

recommended for the NAME statement.

STYPE is a required statement used to indicate the type of switch. A DIP switch is defined

as a set of switches, each having an ON and OFF position. A ROTARY switch is defined

as a round switch that may be set to one of n positions. A SLIDE switch is defined as a

straight line switch that may be set to one of n positions. All switches are numbered

beginning with 1.

VERTICAL is an optional statement that indicates that the switch is positioned

vertically on the board. The default is NO. See section 5.3.1, Switch and Jumper

Examples.

REVERSE is an optional statement that indicates that the switch is numbered 1234...

instead of the usual ...4321. Switches numbered in ascending order from left to right or top

to bottom are considered to be reversed. The default is NO. See section 5.3.1, Switch and

Jumper Examples.

LABEL is an optional list of text fields used to specify labels for individual switches. These

switches are listed in LOC(<switchlist>). Each label may have a maximum length of 10

characters. Example: LABEL = LOC(1-3) "SW1" "SW2" "SW3". If the LABEL statement

is omitted, the default numbering is ..4321 for normal switches and 1234.. for REVERSE

switches.

INITVAL is an optional statement used to specify the value of reserved switches in a set.

LOC(<switchlist>) specifies the switches being defined and <valuelist> gives the values of these

switches. If omitted, all switches are assumed to be either determined by the configuration

program or "don't cares". Example: INITVAL = LOC(1 2) 10

FACTORY is an optional statement indicating the default value of the switches when the

board was shipped. LOC(<switchlist>) specifies the switches being defined and <valuelist> gives

the values of these switches. Example: FACTORY = LOC(1-8) 11111111

COMMENTS is an optional text field containing additional information that will assist the

user in configuring this particular switch. This field may contain a maximum of 600

characters and will be displayed in a window at least 40 columns wide.

HELP is an optional text field containing information that will be displayed to the user if

he requests help while configuring the switch. This text field may contain a maximum of

600 characters and will be displayed in a window at least 40 columns wide.

Page 7

5.3 Jumper Identification Block

JUMPER(i) = n

NAME = "jumper name or description"

JTYPE = INLINE | PAIRED | TRIPOLE

[VERTICAL = YES | NO]

[REVERSE = YES | NO]

[LABEL = LOC(<jumperlist>) <textlist>]

[INITVAL = LOC(<jumperlist>) <valuelist>]

[FACTORY = LOC(<jumperlist>) <valuelist>]

[COMMENTS = "<information>"]

[HELP = "<information>"]

In the statement, JUMPER(i) = n, i is an index used to differentiate this Jumper

Identification Block from others. Jumpers should be numbered beginning with 1. N gives

the number of jumpers positions in the set. The maximum value for n is 16.

Several statements in the Jumper Identification Block use the LOC(<jumperlist>) clause to

reference individual jumpers within the block. A <jumperlist> is defined as a list of jumper

numbers, delimited with blanks. This list may be either ascending or descending, but not

mixed. Example: LOC(1 3 5) may be used, but LOC(1 5 3) is illegal syntax. A range may

also be used in place of <jumperlist>. Example: LOC(1-4) or LOC(4-1). Paired and tripole

jumpers are specified by their numbers, while inline jumpers are specified by the two posts

that they connect. A caret is used to show this connection. Example: LOC(1^2). Ranges

may not be used for inline jumper <jumperlists>.

Jumper <values> differ for each type of jumper and are explained in section 5.3.2, Notation

for Jumpers.

A <list> of jumper <values> may not be delimited. Example: LOC(1-3) 101

NAME is a required statement giving either the manufacturer's name or a description for

this jumper set. This is important as a reference for the user. A maximum of 20 characters

is recommended for the NAME statement.

JTYPE is a required statement used to indicate the type of jumper. INLINE jumpers are

arranged in a line, to be connected from one post to the next. PAIRED jumpers are

arranged as a series of double posts, to be connected across. TRIPOLE jumpers are

arranged as a series of triple posts, so that each jumper has two possible settings. Examples

of each type of jumper are given in section 5.3.2, Notation for Jumpers.

VERTICAL is an optional statement that indicates whether or not the jumper block is

positioned vertically on the board. The default is NO. See section 5.3.1, Switch and

Jumper Examples.

REVERSE is an optional statement that indicates that the jumper is numbered 1234...

instead of the usual ...4321. Jumpers numbered in ascending order from left to right or top

to bottom are considered to be reversed. The default is NO. See section 5.3.1, Switch and

Jumper Examples.

LABEL is an optional list of text fields used to specify labels for individual jumpers. These

jumpers are listed in LOC(<jumperlist>). These labels may be placed between two

jumpers, numbered i and j, by specifying the jumper location as i^j. Each label may have a

maximum length of 10 characters. Example: LABEL = LOC(1-3) "J1" "J2" "J3". If the

LABEL statement is omitted, the default numbering is ..4321 for normal jumpers and

1234.. for REVERSE jumpers.

Page 8

INITVAL is an optional statement used to specify the settings of reserved jumpers in a set.

LOC(<jumperlist>) specifies the jumpers being defined and <valuelist> gives the settings of these

jumpers. If omitted, all jumper settings are assumed to be either determined by the

configuration program or "don't cares". Example: INITVAL = LOC(1^2 3^4) 11

FACTORY is an optional statement indicating the default settings of the jumpers when the

board was shipped. LOC(<jumperlist>) specifies the jumpers being defined and <valuelist> gives

the settings of these jumpers. Example: FACTORY = LOC(1-6) 110000

COMMENTS is an optional text field contained additional information that will assist the

user in configuring this particular jumper set. This field may contain a maximum of 600

characters and will be displayed in a window at least 40 columns wide.

HELP is an optional text field containing information that will be displayed to the user if

he requests help while configuring the jumper. This text field may contain a maximum of

600 characters and will be displayed in a window at least 40 columns wide.

Page 9

5.3.1 Switch / Jumper Examples

The following examples assume the following adapter board orientation: edge connector

down, board bracket on the right.

Adapter Board Orientation:

{Board Outline}

Normal Switch:                  Reverse Switch:

{Horizontal                     {Horizontal

Rectangle}                      Rectangle}

6 5 4 3 2 1                     1 2 3 4 5 6

Normal Vertical Switch:         Reverse Vertical Switch:

6                               1

5                               2

4  {Vertical                    3  {Vertical

3   Rectangle}                  4   Rectangle}

2                               5

1                               6

Paired Jumper (Normal):

o o o o o o

o o o o o o

6 5 4 3 2 1

Tripole Jumper (Reverse):       Tripole Jumper (Reverse, Vertical):

o o o o o o                     1  o o o

o o o o o o                     2  o o o

o o o o o o                     3  o o o

1 2 3 4 5 6                     4  o o o

Inline Jumper (Reverse):

o o o o o o

1 2 3 4 5 6

Page 10

5.3.2 Notation for Jumpers

Paired Jumpers:

Values for paired jumpers can be given in the same way that they are given

for switches, with a 1 or 0 for each position. A 1 indicates that the jumper is

present; a 0 indicates it is not. X's may also be used to indicate "don't care".

Tripole Jumpers:

Values for tripole jumpers can also be expressed in the standard way, with

some clarification of the meaning of 1s and 0s. A 1 indicates that the jumper

is in the upper position for horizontal jumpers or the right position for

vertical jumpers. A 0 indicates that the jumper is in the lower position for

horizontal jumpers or the left position for vertical jumpers. An upper or

lower case N indicates that the jumper is not present. X's may also be used

to indicate "don't care".

Inline Jumpers:

The standard notation is inadequate for describing inline jumpers. These

jumper positions are represented using carets. For example, the first jumper

position in an inline jumper block would be represented by 1^2. A 1

indicates that the jumper is present, a 0 indicates that the jumper is not

present. X's may also be used to indicate "don't care".

Inline Jumper Example:

JUMPER(1) = 7

NAME = "J101"

JTYPE = INLINE

VERTICAL = YES

REVERSE = YES

LABEL = LOC(1^2 3^4 5^6 7^8) "IRQ2" "IRQ3" "IRQ4" "IRQ5"

FACTORY = LOC(3^4) 1

This example would appear on the screen as follows. Note that the number

of jumper positions (7) is not equal to the number of posts.

o  1

IRQ2

o  2

o  3

IRQ3    |

o  4

o  5

IRQ4

o  6

o  7

IRQ5

o  8

J101

Page 11

5.4 Software Identification Block

SOFTWARE(i) = "software description"

In the SOFTWARE(i) statement, i is an index used to differentiate this Software

Identification Block from others.

At configuration time, the software description comments are displayed to the user.

Often, these comments will simply inform the user of a device driver that must be

called from the CONFIG.SYS file, or a program that must be run under DOS to

initialize an expansion board. On ISA systems, these programs often require

parameters to inform them which interrupt, DMA channel, or other global resource

was selected for their expansion board. By placing an INIT = SOFTWARE(i)

statement after the resource declaration, the parameter associated with the selected

resource may be appended to the software description comment and displayed to

the user. An example of this usage is shown in section 6.4.3, INIT Statement for

Software.

Software drivers on EISA machines will be able to determine their resource

parameters by querying a ROM BIOS call and this feature will not be necessary.

6 Function Statement Block

FUNCTION = "<name>"

[TYPE = "<function type">]

[COMMENTS = "<information>"]

[HELP = "<information>"]

CHOICE = "<name>"

resource description block

.

.

[CHOICE = "<name>"]

[resource description block]

The FUNCTION statement names a function of the board. A maximum of 40 characters is

recommended for the function name. The information that follows details the choices of

resources used to implement the FUNCTION and the steps used to initialize the

FUNCTION.

TYPE is an optional statement used by a board manufacturer to identify a function or

device on a board. This information will be available to software drivers through a ROM

BIOS call. Characters in the TYPE string will be uppercased, and the TYPE string must be

placed in quotes. Types are discussed in more detail, and some predefined types are listed

in section 6.3, Types and Subtypes.

COMMENTS is an optional text field containing additional information about the

function. This text field may contain a maximum of 600 characters and will be displayed in

a window at least 60 columns wide when the function is selected.

HELP is an optional text field containing information that will be displayed to the user if

he requests help while configuring the function. This text field may contain a maximum of

600 characters and will be displayed in a window at least 40 columns wide.

Page 12

CHOICE statements are used to place different resource statements and their

corresponding initialization information into logical groups. Each FUNCTION must have

at least one CHOICE statement, and may have as many as necessary. A group of resource

and initialization statements is called a resource description block, and is described in the

following section. Each of a function's choices will have a different resource description

block. For example, if the function were "Serial Port", its choices might be "COM1",

"COM2", and "Disable Port". Each choice requires a different set of resources and

different initialization values. A maximum of 30 characters is recommended for the name.

6.1 Resource Description Block

Resource description blocks contain statements identifying the system resources used by a

particular configuration choice and the steps required to initialize this configuration. A

resource description block may contain the following statements:

[SUBTYPE = "choice type"]

[DISABLE = YES | NO]

[HELP = "<information>"]

[Resource Statements]

[INIT Statements]

SUBTYPE is an optional statement used in conjunction with the FUNCTION level TYPE

declaration to provide information to software drivers through a ROM BIOS call.

Characters in the SUBTYPE string will be uppercased, and the SUBTYPE string must be

placed in quotes. Types and subtypes are discussed in more detail, and some predefined

types are listed in section 6.3, Types and Subtypes.

The DISABLE statement is used to inform the configuration program that, if the

corresponding CHOICE is selected, the function or device will be disabled. When resource

conflicts occur, the configuration program will select this choice only as a last resort. This

statement is optional. The default is NO.

HELP is an optional text field containing information that will be displayed to the user if

he requests help with the CHOICE. This text field may contain a maximum of 600

characters and will be displayed in a window 40 columns wide.

Resource statements list the system resources that will be used if the corresponding

CHOICE is selected. System resources include DMA channels, I/O ports, IRQs, and

memory address space. The statements used to identify these resources are discussed

separately.

INIT statements provide initialization information for each possible configuration of

resources. These statements are discussed separately.

Page 13

6.2 Resource Statements

6.2.1 DMA Statement

DMA = <list>

[SHARE = YES | NO]

[SIZE = BYTE | WORD | DWORD]

[TIMING = DEFAULT | TYPEA | TYPEB | TYPEC]

SHARE is an optional statement that indicates whether the function is willing to share this

DMA channel. The default is NO.

SIZE is an optional statement that specifies whether the DMA channel uses 8-bit, 16-bit, or

32-bit data transfer. There is no default value. When provided, this information will be

used for verification purposes only.

TIMING is an optional statement that specifies the timing by the DMA channel. If

omitted, DEFAULT timing is assumed.

Example:        DMA = 2 | 4

This example indicates that DMA channel 2 or 4 may be used. The selected

channel may not be shared, and default timing will be used.

6.2.2 PORT Statement

PORT = <rangelist> [STEP = <value>][COUNT = <value>]

[SHARE = YES | NO]

[SIZE = BYTE | WORD | DWORD]

If a range is given in the PORT statement, the STEP field may be used to indicate the

number of ports requested and the increment used in searching for available I/O space. If

the number of ports requested is different from the increment value, the COUNT field may

be used to supply the number of ports. The COUNT field may not be used without the

STEP field. If the STEP and COUNT fields are not supplied it is assumed that every port

in the range is being requested.

SHARE is an optional statement that indicates whether the function is willing to share the

requested port. The default is NO.

SIZE is an optional statement that is used to indicate the size of the port. There is no

default value. When provided, this information will be used for verification purposes only.

Slot-specific EISA port addresses must be prefixed with 0Z.

Page 14

Examples:

PORT = 300h - 30Fh

This example indicates that the entire range of 16 port addresses from 300h to 30Fh

will be used.

PORT = 300h - 30Fh STEP = 4

This example indicates that any of the following ranges may be used: 300h-303h,

304h-307h, 308h-30Bh, 30Ch-30Fh.

PORT = 300h - 310h STEP = 4

This example is acceptable, although it calls for 17 ports taken 4 at a time. The

extra port address is ignored, and the example becomes identical to the previous

one.

PORT = 300h - 30Fh STEP = 4 COUNT = 2

This example indicates that any of the following ranges may be used: 300h-301h,

304h-305h, 308h-309h, 30Ch-30Dh.

6.2.3 IRQ Statement

IRQ = <list>

[SHARE = YES | NO]

[TRIGGER = LEVEL | EDGE]

SHARE is an optional statement that indicates whether the function is willing to share this

interrupt. The default is NO.

TRIGGER is an optional statement that specifies whether the interrupt is level or edge

triggered. The default is edge triggered.

Example:        IRQ = 3 | 4 | 5

SHARE = YES

TRIGGER = LEVEL

This example indicates that IRQ 3, 4, or 5 may be used as a shared, level triggered,

interrupt.

6.2.4 MEMORY Statement

MEMORY = <rangelist> [STEP = <value>]

[ADDRESS = <rangelist> [STEP = <value>]]

[WRITABLE = YES | NO]

[MEMTYPE = SYS | EXP | OTHER]

[SIZE = BYTE | WORD | DWORD]

[CACHE = YES | NO]

[SHARE = YES | NO]

The MEMORY statement is used to define a range of memory on the board. Memory

expansion boards can usually contain several different amounts of memory. Each of these

values can be listed, or a range can be given. If a range is given, the STEP field is required

to give the smallest increment by which memory can be added to the board.

Page 15

ADDRESS specifies the address where the memory will be located. If a single address is

given, the memory must be located there. If a list of addresses is given, the memory may be

located at any of these addresses. If an address range is given, the memory will be located

within that range, and the STEP field is required to give the increment used in searching

for an available slot. The ADDRESS statement should only be used for memory that

resides in the system's 4 GB physical address space.

WRITABLE is an optional statement that indicates whether the memory may be written

to. The default is YES.

MEMTYPE is an optional statement that specifies whether the memory is SYStem,

EXPanded, or OTHER. System memory includes memory in the 4 GB physical address

space that is managed by the operating system. Expanded memory is memory that does not

use physical address space and is managed by an expanded memory manager. Expanded

memory should not have an ADDRESS statement. OTHER includes memory that is not

managed by the operating system, such as expanded memory page frames, memory mapped

I/O, and bank-switched memory. OTHER memory should have an ADDRESS statement

only if it resides in the physical address space. The default for the MEMTYPE statement is

SYS.

SIZE is an optional statement used to identify the memory as 8-bit, 16-bit, or 32-bit. There

is no default. When provided, this information will be used for verification purposes only.

CACHE is an optional statement that indicates whether or not this memory is cachable.

The default is NO.

SHARE is an optional statement that indicates whether this memory address space may be

shared with another adapter board. The default is NO.

Examples:

MEMORY = 128K

ADDRESS = 0E0000h

WRITABLE = NO

MEMTYPE = OTHER

This example describes a 128K system ROM located at address 0E0000h.

MEMORY = 1M - 4M STEP = 512K

ADDRESS = 1M - 16M STEP = 256K

WRITABLE = YES

MEMTYPE = SYS

This example describes a memory expansion board that can add from 1 to 4 MB of

extended memory. Memory can be added to this board in 512K increments. This

memory must be located in the address space above 1 MB and below 16 MB. It's

starting address can be on any 256K boundary within this range.

Page 16

6.3 Types and Subtypes

Types and subtypes are used to identify devices installed in the system, special

characteristics of these devices, and the way in which these devices are configured. This

information is stored in CMOS memory, where it can be used by operating systems and

generic device drivers to identify the devices installed in a system at initialization time.

Type strings are composed of multiple fields, separated by commas. Fields are arranged so

that they describe the device function or characteristics with increasing level of detail. The

first field in the string should describe the general functional category, such as

communication or video, for the FUNCTION being described. Subsequent fields in the

type strings should more specifically classify the FUNCTION by describing its

characteristics in more detail. For example, a standard VGA expansion board would have

the type string "VID,VGA". VID categorizes it as a video board, and VGA defines it as a

specific standard. Further additions could be made to the type string to describe more

specific characteristics or enhancements.

Subtype strings are used to add information to the type string regarding the device's

configuration as determined at system configuration time. The subtype strings for each

possible configuration of the device are specified under the corresponding CHOICE. The

configuration utility automatically appends to the type string the content of the subtype

string that corresponds to the CHOICE selected for that FUNCTION at system

configuration time. Subtype strings are separated from the type string by a semicolon.

Semicolons are also used to delimit multiple subtype strings.

Page 17

The following examples list some predefined types.

Device Type                     Device Description

COM,ASY                         Standard ISA compatible serial controller

COM,ASY,FIFO                    NS16550A based serial ctrl. with FIFO

COM,SYN                         Synchronous communications card

COM,ASY,SYN,SDLC,NSC,HDLC       Multi-function communications card

supporting SDLC, bisync, and HDLC

protocols

PAR                             ISA compatible parallel port controller

PAR,BID                         ISA parallel port with bidirectional mode

PTR,8042                        Standard 8042 pointing device

NPX,287                         Intel 287 numeric coprocessor

NPX,387                         Intel 387 numeric coprocessor

NPX,387SX                       Intel 387 numeric coprocessor, SX version

NPX,W1167                       Weitek 1167 numeric coprocessor

NPX,W3167                       Weitek 3167 numeric coprocessor

KEY,nnn,COU=xx                  Standard keyboard

nnn = number of keys

(083, 084, 101, or 103)

xx = keyboard country code

(US, UK, FR, GR, IT, SP, LA, SV,

SU, NL, DK, NO, PO, SF, SG, CF,

or BE)

VID,MDA                         Standard mono adapter

VID,MDA,HERC                    Standard Hercules mono adapter

VID,CGA                         Standard CGA adapter

VID,CGA,ATT400                  CGA with AT&T 640x400 superset

VID,EGA                         Standard EGA adapter

VID,VGA                         Standard VGA adapter

MSD,DSKCTL(1)                   Primary standard AT-compatible disk ctl.

MSD,FPYCTL(1)                   Primary standard 765 floppy controller

MSD,TAPCTL(1)                   Primary tape controller

OTH,JOY                         Miscellaneous entry (OTHer) identifying

a joystick card

Page 18

The use of TYPEs and SUBTYPEs is shown in the following example:

FUNCTION = "Serial Port"

TYPE = "COM,ASY"

CHOICE = "Configure as COM1"

SUBTYPE = "COM(1)"

LINK

PORT = 3F8h-3FFh

IRQ = 4

INIT = IOPORT(1) XXXXXXX0

CHOICE = "Configure as COM2"

SUBTYPE = "COM(2)"

LINK

PORT = 2F8h-2FFh

IRQ = 3

INIT = IOPORT(1) XXXXXXX1

In the above example, either "COM(1)" or "COM(2)" would be appended to "COM,ASY"

depending upon which CHOICE was selected by the configuration program or the user.

Page 19

6.4 INIT Statements

INIT statements provide the values needed to initialize programmable boards, or the

switch settings used on switch-programmable boards. INIT statements may also be used to

provide the user with information needed to properly install software drivers. INIT

statements may follow any system resource statement, including DMA, IRQ, PORT,

MEMORY, and ADDRESS. These resource statements may have multiple INIT

statements if necessary. Also, multiple resource statements may use the same INIT

statement. The relationship between INIT statements and their corresponding resource

statements is defined by the type of group into which these statements are placed.

Resource and INIT statement groups are discussed in section 6.5.

6.4.1 INIT Statement for Programmable I/O Ports

INIT = IOPORT(i) <list>

IOPORT(i) indicates which port is to be used. <List> gives a list of values to be output

to the port for each resource option. These values must be in binary and may

contain Xs to mask off bits not used for that resource.

Example:

IRQ = 3 | 4

INIT = IOPORT(1) XXXXXX01 | XXXXXX10

In this example, bits 1 and 0 would be set to 01 for IRQ3 or 10 for IRQ4.

A shorthand form of the INIT statement for I/O ports is provided for boards that must

initialize a large number of ports. This form does not require a corresponding IOPORT

statement, as the port address is contained within the INIT statement. The syntax for the

shorthand form is as follows:

INIT = IOPORT(<address>) <list>

<Address> specifies the address of the I/O port. Slot specific addresses must be

prefixed with a 0Z.

<List> gives a list of values to be output to the port for each resource option. The

values are binary only: it is not necessary to specify the radix. A bit position

marked with an 'r' indicates that the bit value should be read from the port. The

length of the values in the list must be the same as the data width of the port: 8, 16,

or 32 bits.

Example:

DMA = 2 | 4

INIT = IOPORT(0Z380h) XXXXXX01 | XXXXXX10

In this example, bits 1 and 0 would be set to 01 for DMA channel 2 or 10 for DMA channel

4.

Page 20

6.4.2 INIT Statement for Switches and Jumpers

INIT = SWITCH(i) LOC(<switchlist>) <list>

INIT = JUMPER(i) LOC(<jumperlist>) <list>

SWITCH(i) or JUMPER(i) indicates which switch/jumper block is to be used.

LOC(<switchlist>) or LOC(<jumperlist>) lists the individual switches or jumpers used to

configure the current resource.

A <switchlist> is defined as a list of switch numbers, delimited with blanks. This list

may be either ascending or descending, but not mixed. Example: LOC(1 3 5) may

be used, but LOC(1 5 3) is illegal syntax. A range may also be used in place of

<switchlist>. Example: LOC(1-4) or LOC(4-1).

A <jumperlist> is defined as a list of jumper numbers, delimited with blanks. This list

may be either ascending or descending, but not mixed. Example: LOC(1 3 5) may

be used, but LOC(1 5 3) is illegal syntax. A range may also be used in place of

<jumperlist>. Example: LOC(1-4) or LOC(4-1). Paired and tripole jumpers are

specified by their numbers, while inline jumpers are specified by the two or more

posts that they connect. A caret is used to show this connection. Example:

LOC(1^2). Ranges may not be used for inline jumper <jumperlists>.

<List> gives a list of the settings needed for each resource option. A 1 represents a

switch in the ON position, a 0 represents a switch in the OFF position, and Xs may

be used as don't cares. Settings for jumpers depend on the jumper type, and are

discussed in section 5.3.2 entitled Notation for Jumpers.

Example:

PORT = 300h - 30Fh STEP 4

INIT = SWITCH(1) LOC(1 2) 00 | 01 | 10 | 11

This example shows a PORT statement that has four possible configurations. Dip switch

locations 1 and 2 of SWITCH(1) are used to select a configuration, as shown in the

following table:

Port Range      Switch #1       Switch #2

300h-303h       OFF             OFF

304h-307h       OFF             ON

308h-30BH       ON              OFF

30Ch-30Fh       ON              ON

6.4.3 INIT Statement for Software

INIT = SOFTWARE(i) <list>

SOFTWARE(n) indicates which SOFTWARE statement will be displayed to the

user during system configuration. <List> gives a list of parameters to be passed to the

SOFTWARE statement for each resource option. Each parameter should be text

enclosed in quotes. The SOFTWARE statement will display a block of comments

followed by the parameters associated with the resources chosen.

Page 21

Example:

BOARD

ID = ...

NAME = "Network Adapter"

.

.

SOFTWARE(1) =

"The Network Adapter is initialized using the program \n

NET.EXE. The following command should be placed in your \n

AUTOEXEC.BAT file: NET /I=x /D=y where "

.

.

IRQ = 3 | 4 | 5

INIT = SWITCH(1) LOC(1 2) 00 | 01 | 10

INIT = SOFTWARE(1) "x is 3" | "x is 4" | "x is 5"

DMA = 2 | 5

INIT = SWITCH(1) LOC(3) 0 | 1

INIT = SOFTWARE(1) "y is 2" | "y is 5"

.

.

In the above example, the IRQ and DMA statements are followed by two INITs. The first

INIT is used to inform the user of the proper switch setting for the chosen value. The next

INIT statement provides the proper parameter to the SOFTWARE statement. This

parameter is displayed after the SOFTWARE statement. For example, if IRQ3 and DMA

channel 2 were chosen above, the SOFTWARE statement would be displayed followed by:

x is 3

y is 2

6.5 Resource and INIT Statement Groupings

Resource statements and INIT statements are divided into groups within a configuration

file. Each resource description block within each CHOICE may have one or more of these

groups. A group is defined as a set of resources and INIT statements that are all directly

related. Resource statements that are initialized independently of each other would fall

into separate groups.

The start of a grouping is marked by a keyword indicating the group's type. The group ends

when either another group or the end of the resource description block is encountered.

There are three types of groups in which resource and INIT statements can be placed.

These groups, linked, combined, and free-form, are discussed separately.

All resource and INIT statements must be defined within a group.

Page 22

6.5.1 Linked Groupings

In a linked group, each statement must have the same number of options. These options

have a one-to-one correspondence with options in the other statements. If the first option

is chosen in one statement, then by definition the first option is chosen in each of the other

statements. Linked groups have the following syntax:

LINK

resource and/or INIT statements

Example:

LINK

IRQ = 3 | 4

DMA = 2 | 5

INIT = IOPORT(1) xxxxxx10 | xxxxxx01

In the above example, the first initialization value is used if IRQ 3 and DMA 2 are chosen,

and the second is used if IRQ 4 and DMA 5 are chosen.

6.5.2 Combined Groupings

In a combined group, every combination of options encompassing all resource statements

are provided with an initialization value in the group's INIT statement. This type of group

is used primarily with MEMORY and ADDRESS statements. Combined groups have the

following syntax:

COMBINE

resource and/or INIT statements

Example:

COMBINE

MEMORY = 1M | 2M

ADDRESS = 1M | 2M

INIT = IOPORT(2) 00xxxxxx | 01xxxxxx | 10xxxxxx | 11xxxxxx

In the above example, the first initialization value is used for 1M of memory starting at 1M.

The second is used for 1M of memory starting at 2M. The third is used for 2M of memory

starting at 1M. Finally, the fourth initialization value is used for 2M of memory starting at

the address 2M.

The INIT statement for a combined group should have a number of values equal to the

product of all the options in the preceding resource statements. The combinations

always proceed with the first resource statement taking the most significant position and

the last resource taking the least significant position.

Page 23

When MEMORY and ADDRESS statements are used in a combined group, certain illegal

combinations are automatically recognized by the configuration program. These

combinations result from the use of ranges in the ADDRESS statement. An example of

this situation follows:

COMBINE

MEMORY = 1M | 2M

ADDRESS = 1M - 4M STEP = 1M

INIT = IOPORT(1) XXXXX000 - XXXXX100

This example is shown in the following table:

Memory Amount           Starting Address        Port Value

1M                      1M              XXXXX000

1M                      2M              XXXXX001

1M                      3M              XXXXX010

2M                      1M              XXXXX011

2M                      2M              XXXXX100

2M                      3M              --------

In the above example, 1M of memory may be placed at addresses 1M, 2M, or 3M, but not

at 4M because this violates the ADDRESS statement that states that the memory must be

contained in the range 1M to 4M. Similarly, 2M of memory may be placed at addresses 1M

or 2M only. Initialization values are only provided for the five legal memory and address

combinations.

6.5.3 Free-Form Groupings

In a free-form group, all resource and INIT statements are considered to be independent of

each other. No correspondence at all between the resource and INIT statements is

required. Free-form groups have the following syntax:

FREE

resource and/or INIT statements

Example:

FREE

IRQ = 2 | 3 | 4 | 5

DMA = 4

INIT = IOPORT(2) xxxx1010b

In the above example, IRQ 2, 3, 4, or 5 may be selected, with no initialization required. It

is assumed that any initialization for this statement will be done at a later time by an

independent program. This example also states that DMA channel 4 will be used, and that

the initialization given in the INIT statement should always be performed. Each of these

statements is independent and completely unrelated to the others.

Page 24

7 Advanced Topics

The following advanced topics are not implemented by the ISA Configuration Utility, but a

brief introduction to them is included for completeness.

7.1 Subchoices

There are many examples of existing ISA switch programmable boards in which the

number of possible configurations is very large. Memory expansion boards that allow you

to allocate memory as conventional, extended, or expanded are especially notorious for

this. In these cases, it is desirable to let the configuration utility do most of the work rather

than forcing the user to select from a long and confusing list of configuration choices.

Subchoices may be used to insulate the user from part of this decision making process.

The syntax for the SUBCHOICE statement is shown below:

CHOICE = "<name>"

global resource description block

SUBCHOICE

subchoice resource description block

.

.

SUBCHOICE

subchoice resource description block

A choice may have as many subchoices as necessary.

The resources listed before the subchoices in the global resource description block are

allocated regardless of the subchoice selected.

Each subchoice contains the resource and INIT statements required for that configuration.

Subchoices are mutually exclusive. Only one subchoice will be selected per choice by the

configuration program.

Subchoices are provided as a means of simplifying very complex configuration scenarios for

the user's benefit. Consequently, they should only be used in complex situations, such as

handling certain memory configurations. Separate CHOICE statements should be used

whenever the user has a choice to make, for example, choosing between COM1 and COM2

for a serial port. If the user does not wish to make this decision, the configuration program

is capable of doing so automatically. Subchoices are configuration alternatives that are not

seen by the user. He may still scroll through the possible configurations, including those

present in subchoices, but a list of these alternatives is never presented to him, as is the

case with separate, named, choices.

Page 25

The following example presents a situation in which the use of subchoices is warranted.

Suppose that the configuration of a memory expansion board is directly dependent upon

the amount of base memory that currently exists in the system. This may be accomplished

using separate CHOICE statements that explicitly state the amount of base memory for

each case:

CHOICE = "Add Memory, 256K Already Installed"

CHOICE = "Add Memory, 384K Already Installed"

CHOICE = "Add Memory, 512K Already Installed"

CHOICE = "Add Memory, 640K Already Installed"

This is a viable solution to the problem, although it requires the user to know how much

base memory was previously installed. A solution using subchoices would appear to the

user as follows:

CHOICE = "Add Memory"

This choice would be followed by four subchoices that the user would not see. The first

subchoice would attempt to add memory at 256K. If memory already existed there,

the configuration utility would try the second subchoice, that would attempt to add memory

starting at 384K. This progression would continue until a subchoice that did not conflict

with previously allocated resources was found. This solution, although equivalent to the

first, would be significantly less confusing for a beginning user.

In general, subchoices should not be used when a small number of choices will suffice.

They should be reserved for cases in which the language is insufficient or it is desirable to

insulate the user from making a decision that is too technical.

7.2 TOTALMEM Statement

TOTALMEM is a statement that is placed under a CHOICE statement and used to specify

the total amount of memory on a memory expansion board. This statement is required by

memory expansion boards that can allocate portions of their memory as conventional,

extended, or expanded. This would require multiple memory statements in a single

CHOICE, and the user would select the desired amount of each type of memory. The

TOTALMEM statement allows the configuration program to verify that the total amount

of memory selected in multiple memory statements does not exceed the amount of memory

installed on the board. This statement also allows the configuration program to select the

proper SUBCHOICE, when SUBCHOICES are used.

The syntax for the TOTALMEM statement is:

TOTALMEM = <rangelist> [STEP = <value>]

For a given CHOICE, the amounts of memory that could exist on the board must be given

in the TOTALMEM statement. Each of these values can be listed, or a range can be given.

If a range is given, the STEP field is required to give the smallest increment by which

memory can be added to the board.

Page 26

Examples:

TOTALMEM = 1024K | 1536K | 2048K

TOTALMEM = 1M - 2M STEP = 512K

The above examples are equivalent.

A TOTALMEM statement must be given for each CHOICE within a FUNCTION that

provides expansion memory and allows the user to divide that memory as conventional,

extended, and/or expanded memory. The TOTALMEM statement should be placed

before the resource and INIT statement groupings for that CHOICE.

The TOTALMEM statement should encompass all system and expanded memory provided

by the expansion board (MEMTYPE = SYS or EXP). Memory that is classified as

OTHER is not included in the TOTALMEM statement.

7.3 Embedded Slots

Embedded slots are used to contain resource information for devices that reside on

another board, typically the system board. Embedded devices decode slot-specific I/O, and

may or may not have readable id's. Embedded devices must use slots that do not physically

exist on the system board. If the system board provides n expansion slots, the range of

usable embedded slots is n+1 thru 15, because slot-specific I/O is defined for slots 0 thru

15.

Embedded slots are defined using standard configuration files. An embedded slot

configuration file must contain the statement SLOT = EMB(n), where n is the number of

the embedded slot being used. This statement is placed in the Board Identification Block

in place of the standard SLOT statement.

7.4 Virtual Slots

Virtual slots allow peripherals to be added to installed boards such as disk controllers or

serial ports, and allow functions or devices which reside on the system board to appear

separately. Virtual slots may also contain resource information for software drivers, such

as extended memory managers, that require resources. Virtual slots do not use slot-specific

I/O, and do not have readable id's.

Virtual slots are defined using standard configuration files. A virtual slot configuration file

must have the statement, SLOT = VIR, in its Board Identification Block, in place of the

standard SLOT statement.

7.5 System Board Configuration Files

The system board is identified in the board identification block as embedded slot #0, using

the statement:

SLOT = EMB(0)

Page 27

7.5.1 System Board Identification Block

System board configuration files must supply additional information to the configuration

utility. This information includes the amount of non-volatile memory available, the

number of expansion slots on the system board, and the size and type of each expansion

slot. This information is provided in the System Board Identification Block, that may be

placed anywhere after the Board Identification Block.

SYSTEM

EISACMOS = <value>

SLOT(1) = ISA8 | ISA 16 | EISA

LENGTH = <value>

[SKIRT = YES | NO]

.

.

SLOT(i) = ISA8 | ISA16 | EISA

LENGTH = <value>

[SKIRT = YES | NO]

The SYSTEM statement appears at the beginning of the System Board Identification

Block.

EISACMOS is a required statement listing the amount of non-volatile memory available

for EISA slot-specific use.

The SLOT(i) and LENGTH statements identify the number, type, and length of expansion

slots available on the system board. The SLOT statements specify the slot types as ISA8,

or 8-bit ISA, ISA16 for 16-bit ISA, or EISA. The LENGTH statements specify the lengths

of the longest expansion boards that can be accommodated by each slot. Lengths must be

stated in millimeters. Fractions may not be used.

The optional SKIRT statements indicate whether or not each slot on the system board can

accommodate an expansion board that contains a lower extension, or skirt. The default is

YES, expansion boards with skirts can be accommodated.

7.6 CONNECTION Statement

Associated with each FUNCTION statement in the CFG file is an optional statement of

the following syntax:

CONNECTION = "location"

where the "location" text is replaced by a short string describing the location of the

connector. So, for example, a serial/parallel board might have the CONNECTION value

for the serial port set to "Top" or "Upper", and the CONNECTION value for the parallel

port set to "Bottom" or "Lower". There are no predefined types for this field.

This information will be displayed to the user on request.

Page 28

7.7 Variable Ports

There are cases in which the address of an IOPORT that needs to receive an initialization

value is dependent upon a choice made by the configuration program or the user. Variable

ports can be used in these situations. The syntax is as follows:

IOPORT(n) = PORTVAR(i)

Instead of specifying the address in the Programmable I/O Port Identification Block, a

reference is made to a PORTVAR. PORTVARs should be numbered independently; their

indices have no relationship with those of the IOPORT statements. The PORTVAR is

then defined later under a CHOICE statement.

PORTVAR(i) = <address>

The INIT statements that reference the IOPORT are unchanged.

Example:

IOPORT(1) = 0Z280h

INITVAL = 111100XX

IOPORT(2) = PORTVAR(1)     ;Variable Port

INITVAL = XXXX10xx

FUNCTION = "Serial Port"

SUBFUNCTION = "Port Number"

CHOICE = "COM1"

PORTVAR(1) = 3F9h

LINK

PORT = 3F0h - 3FFh

INIT = IOPORT(1) XXXXXX01

CHOICE = "COM2"

PORTVAR(1) = 2F9h

LINK

PORT = 2F0h - 2FFh

INIT = IOPORT(1) XXXXXX10

SUBFUNCTION = "Baud Rate"

CHOICE = "1200"

FREE

INIT = IOPORT(2) 1001XX00

CHOICE = "2400"

FREE

INIT = IOPORT(2) 1001XX01

.

.

In the above example, the I/O port used to initialize the baud rate is dependent upon

whether the serial port is selected as COM1 or COM2.

Page 29

#