National Instruments GPIB Pop-up Monitor, Version 1.0, 6/18/90.

DESCRIPTION:

The Pop-up Monitor window displays the current state of the GPIB

in the form of simulated LEDs and switches.  The user can directly

control each signal on the GPIB by pressing the key indicated

between the switch and LED.  If the GPIB signal is asserted then

the LED is on, if the  GPIB signal is not asserted the LED is off.

LEDs and switches appear as follows:

switch on: �߳  switch off: �ܳ   LED on: �  LED off: �

BOARDS:

The Pop-up Monitor currently supports three interface boards:

National Instruments AT-GPIB, National Instruments GPIB-410,

and National Instruments GPIB-PCIII.

MEMORY REQUIREMENTS:

The Pop-up Monitor installs on any IBM PC or PC compatible, with a

suggested minimum of 128K bytes of RAM.  The Pop-up monitor software

takes up approximately 15K.

INSTALLATION:

At the DOS prompt, simply type "mon" followed by any required flags.

Example:     A>mon

FLAGS:

Allows the user to change configuration of the Monitor window at time

of installation.  Flags are always preceded with a minus sign (-). Any

flags that do not contain a minus sign are ignored.

-m configures Monitor window for a monochrome screen.

-b### to change board address to ### where ### is any hex number.

-3 configures Monitor window for PC-III (default is AT-GPIB).

-4 configures Monitor window for GPIB-410 (default is AT-GPIB).

-h,-? prints built-in help screen without installing Pop-up Monitor.

Examples:

MON -m -b2C0 would install a monochrome Pop-up Monitor for the

National Instruments AT-GPIB board at address 2C0 hex.

MON -4 would install the Pop-up Monitor for National Instruments

GPIB-410 board at default address 300 hex.

MON -h would print a short help screen to aid in installation.

DEFAULT SETTING:

The following examples show the settings.

A>mon        --installs a color Pop-up Monitor (or monochrome in the

absence of a color graphics adapter) for the AT-GPIB

board at address 2C0 hex.

A>mon -4     --installs a color Pop-up Monitor (or monochrome in the

absence of a color graphics adapter) for the GPIB-410

board at address 300 hex.

UN-INSTALLATION:

Once the Pop-up Monitor code has been successfully installed, a system

reboot is necessary to remove it from memory.

GETTING HELP:

Installation:   Both "mon -h" or "mon -?" will fill the screen with a short

help screen primarily designed as an installation aid.  Executing mon -h or

mon -? will NOT install the Monitor code in memory.  Consequently, these

commands may be executed at any time, before or after the Monitor has been

installed.

On Line Help:   Press the F1 key and the help menu will appear.  Highlighted

prompt is explained inside the box.   Change the highlighted prompt by use

of the left and right cursor keys (keys 4 and 6 on the number pad).

If both the left and right cursor keys beep when pressed, then NUMLOCK

is active, so press the NUMLOCK key and try again.  User need not hit F1

to activate help screen, but instead hit either the left or right cursor

key and the help screen will appear. The Pop-up Monitor will not respond

differently when the help screen is present.

CHANGING SOFTWARE CONFIGURATION:

User may change which board the software is configured to by getting the

help screen active, positioning the highlighted prompt on BOARD and hitting

[ALT-A].

Example:

User installed Pop-up Monitor with no flags,

A>mon

then remembered that PC contained GPIB-410 board.  Instead

of rebooting and starting over, user hits [ALT]-[RIGHT SHIFT]

to get monitor window, then hits F1 to get help screen,

and positions highlighted prompt on BOARD.   User then

presses the 'A' key while holding down the ALT key at

the same time.  This places the user in the CHANGE CONFIGURATION MENU.

In the CHANGE CONFIGURATION MENU, user has three options, to change

the board base address, change board type, or change monitor type

(color or monochrome).

CHANGING BASE ADDRESS:

User may change the board base address by placing the highlighted prompt

on BASE and pressing the up and down arrow cursor keys. Pressing the up-arrow

cursor key causes the base address to increase, the down-arror cursor key

causes the base address to decrease.  The new base address is not changed

until user escapes from CHANGE CONFIGURATION MENU by pressing escape [ESC].

Key Pad

#

# � #       to move prompt: # � #               #

#         to increase or lower base address:  �

#

CHANGING BOARD TYPE:

User may change the board type my placing the highlighted prompt on BOARD in

the CHANGE CONFIGURATION MENU.  Pressing the up or down arrow keys toggles the

board type.

Note: NEVER remove or install an interface board while the computer is ON.

Key Pad

#

# � #       to move prompt: # � #          #

#         to toggle between board types: �

#

CHANGING TO/FROM MONOCHROME:

Sometimes the LEDs of the Pop-Up Monitor are hard to read, especially on

a monochrome monitor.  There are two ways to correct this situation.  First,

one may install the Monitor with the -m flag.  Second, from the CHANGE

CONFIGURATION MENU, position the cursor on MONITOR, then press on the up

or down arrow cursor keys.  This will change monochrome to/from color.

Key Pad

#

# � #       to move prompt: # � #               #

#         to toggle between monitor settings: �

#