# Windows 2000 installation procedure for FIVA MPC-101 (#2)

### Install Display Adapter Device Driver and Change Display Settings

Casio San Jose

Ted Inoue

Casio FIVA MPC-101 is optimized to run Windows 98. Casio will not be responsible for any damage or data loss as your result of installing Windows 2000 setup. Below are general guidelines and comments if you attempt to install another OS other than Windows 98. Casio will not provide any technical support for any issues which may occur as a result of installing another OS.

## Install Display Adapter device driver

- Prepare gx drivers floppy disk which uncompressed gxdriver.zip is included.

- Open Control Panel and choose “Add/Remove Hardware” icon

- Double click “Add/Remove Hardware” icon

- Press “NEXT>” button

5.  Choose “Add/troubleshoot a device” and press “NEXT>” button, then Windows 2000 starts detecting a new Hardware.

1

2

6.  Choose “Add a new device” and press “NEXT>” button.

1

2

7.  Choose “No. I want to select the hardware from a list” and press “NEXT>” button

1

2

8.  Choose “Display adapters” and press “NEXT>” button.

9.  Press “Have Disk…” button

10.  Press “Browse…” button

1

2

3

11.  Choose “31/2 Floppy [A:]” and press “Open” button

1

2

12.  Click “GXVIDEO1” folder and press “Open” button

1

2

13.  Click gx file and press “Open” button

14.  Confirm the selected folder and press “OK” button

15.  Confirm “National Semiconductor Corporation GX” displayed and press “NEXT>” button.

16.  Press “NEXT>” button to install video device driver

17.  Press “Yes” button to continue the installation

18.  Start Installing video drivers from the floppy disk to hard disk

- Press “Finish” button to complete.

20.  Press “Yes” button to restart

## Keep the following process to change screen size VGA to 800x600

21.  Right Click on the Desktop screen

22.  Select “Properties”

23.  Choose “Settings” tab

24.  Change screen size to “800 by 600 pixels” and change Colors to “High Color (16bit)”.

25.  Press “Apply” button

26.  Press “OK” button to apply 800 by 600 screen

27.  Press “Yes” button to keep this setting.

Change Display Settings   [ Display Acceleration and Icon Color ]

28.  Press “Advanced..” button

29.  Choose “Troubleshooting” tab

30.  Change slide bar of Hardware acceleration from “Full” to “None” and press “OK” button

31.  Choose “Effects” tab

1

2

- Check off “Show icons using all possible colors” and Press “OK” button.