How to install Windows NT to Fiva

Casio San Jose

Ted Inoue

Casio FIVA MPC-101 is optimized to run Windows 98. Casio will not be responsible for any damage or data loss as your result of installing Windows 2000 setup. Below are general guidelines and comments if you attempt to install another OS other than Windows 98. Casio will not provide any technical support for any issues which may occur as a result of installing another OS.

## Comments

- Regarding FIVA Tablet MPC-501, the device driver of touch panel has not been released for Windows NT at this moment (End of March 2000). So another OS can not be installed in FIVA Tablet.

- Mobile Cockpit does not work on Windows NT. You do not have any power management.

- USB does not work on Windows NT

- FIVA can not be turned off by shut down operation, so you have to stick the reset hole to turn off.

- There may be many limitations or non-operated functions.

## Preparation

- Floppy diskette drive MPC-140FDDE, Port Replicator MPC-160RPE and Casio CD-ROM drive MPC-141CDDE or Panasonic KXL-708A CD-ROM drive are necessary to setup.

2.  Uncompressed Fp0, Fp1,Fp2 and Fp3 zip files, and make 4 floppy diskettes.

## FDISK and FORMAT by FAT16

- First FIVA hard disk should be performed with Win95 Fdisk and format.

- Insert Fp0 floppy disk into FDD and power on FIVA

- Enter “Fdisk” and press “CR key”

- Select “3” to delete all current partitions. Enter “4” for NON-DOS partition. Delete all partitions with the same operation.

- Select “1” and “CR key” to create new partition and enter “CR key” again to create the primary DOS partition. Then Enter “CR key” again to create maximum size. It will be assigned with 2047MB.

- Then FIVA ask you to turn off, but you do not need to turn off yet. Continuously create the second partition.

- Enter “Fdisk” and press “CR key”.

- Select “1” and press “CR key” then enter “2” and press “CR key” to create the extended DOS partition. When the second partition appears, enter “CR key”. When “ESC key” is pressed, Fdisk automatically tries to create the logical Dos drive D: with all extended partition. So press “CR key” and “ESC key” to exit.

- Please check the partition you created by pressing “4”.

- After Fdisk is finished, you need to restart FIVA by operating “ALT + CTL + DEL” key..

- After power on, enter “format c:” and press “CR key”. Then enter “y” and press “CR key”.

- Then enter “Format D:” and press “CR key”. Then enter “y” and press “CR key”.

- Please check the partition you created by pressing “4”. System should be assigned with FAT16.

- Reset FIVA

## Setup Windows NT

- Replace Fp0 floppy diskette with Fp1 floppy diskette.

- Insert Windows NT CD-ROM into CD-ROM drive.

- Power on FIVA.

- Setup asks you to enter any key to continue, so press any key.

- Setup copies all installation files of Windows NT to the hard drive. 3290 files are copied. It takes about 2 hours.

- Setup needs to reset lock of drive C:, so enter “y” and press “CR key”

- Windows NT asks you to press ENTER key to restart, so press ENTER key

- Windows NT asks you to press some keys if Windows NT setup starts or not, so press ENTER key to start setup.

- Windows NT asks you if there are additional mass storage disk, so press ENTER key not to have any other disk except displayed hard disk.

- Windows NT asks you if you agree the displayed licensing agreement, press F8 key if you agree.

- Windows NT asks you if hardware/software configurations are correct, press ENTER key.

- Windows NT asks you if you want to install Windows NT to the highlighted partition, press ENTER key.

- Windows NT asks you if you convert file system to NTFS, press ENTER key to leave the current file system if you want,

- Windows NT asks you if the installed folder WINNT is OK, press ENTER key.

- Windows NT asks you if Windows NT check up your hardware, press ENTER.

- Windows NT asks you to press ENTER key for restart, press ENTER key.

- Windows NT Setup Wizard starts. Click “NEXT>” key or Press ENTER key.

- Windows NT Setup Wizard ask you which type you prefer, Typical, Portable, Compact or Custom, Choose Typical if you do not want other types.

- Enter name and organization

- Enter CD key ( it is on the backside of CD-ROM case)

- Enter Computer name for your Local Are Network.

- Enter administrator password.

- Choose if Emergency Floppy Disk is necessary.

- Choose Windows NT components.

- Click “NEXT>” button to install the Windows NT Networking system

- Windows NT Setup Wizard asks you if this FIVA need to connect to your LAN. Even if you choose “Do not connect this computer to a network at this time”, you can preset after Windows NT setup is finished.

- Click “NEXT>” button to finish Windows NT Setup Wizard.

- Click “Close” button if date/time setting tab appears.

- Click “OK” button for detected display adapter.

- Click “OK” button to close the Display Properties tab.

- When Windows NT starts, FIVA displays with 640 x 480 VGA screen and has no audio sound. So we install Video device driver and Audio device driver below.

- Insert a floppy diskette "Fp2" including FIVA Video & Audio device drivers.

- <Change screen size from 640 x 480 to 800 x 600>

Open “Control Panel” and double click “Display” icon

Choose “Settings” tab

Click “Display Type…” button

Click “Change…” button on Adapter Type

Click “Have Disk…” button

Click “Browse…” button

Click “NT40certi112” folder and click “Open” button

Click “gx” file and click “Open” button

Click “OK” button to confirm the folder.

“Cyrix Corporation GX” is displayed, so click OK button.

Click “Yes” button to proceed to install new video device driver

Click “OK” button, and click “Close” button and click “Close” button again.

FIVA asks you to restart this computer, click “Yes” button after floppy diskette is removed.

When FIVA restarts, double click “Display” icon to change the screen size..

Change screen size from 640 x 480 to 800 x 600 on “Desktop area”

Change no. of color from 256 colors to 65536 colors on “Color Palette”

Leave the setting of “Refresh Frequency” with 60 Herzs.

Click “Apply” button then a warning message appears.

Click “OK” button to apply the setting

Click “OK” button to close “Display Properties”.

- <Add Audio Device Driver>

1.  Insert “Fp2” floppy diskette into FDD.

2.  Open “Control Panel” and double click "Multimedia" icon.

3.  Choose “Devices” tab and press “Add…“ button.

4.  Choose “Unlisted or Update Driver” and press “OK” button

5.  Press “Browse…” button and choose “a:” on Drives.

6.  Double click “nt4aud12” and click “OK” button.

7.  Confirm the selected folder then click “ OK” button

8.  Confirm to display “XpressAudio [TM] Driver” then click “OK” button

9.  XpressAudio Base I/O address is displayed with “220”, then click “Continue” button.

10. XpressAudio Configuration is displayed, change from “330” to “Disable” for MPU401 I/O

Address.

11. “System Setting Change” dialog is displayed, click “Restart Now” button after the floppy diskette

Fp2 is removed from FDD.

12. After FIVA restarts, you can hear sound of Windows NT startup theme.

54.  <Add CD-ROM Device Driver>

1.  Insert “Fp3” floppy diskette including CDD drivers into FDD.

2.  Open “Control Panel” and double click "SCSI Adapters" icon.

3.  Choose “Drivers” tag and click “Add…” button

4.  Press “Browse…” button

5.  Double click “Nt” folder.

6.  Confirm the selected folder and click “OK” button.

7.  “PCMCIA ATA Card Host Adapter for NT3.51, 4.0” is displayed and click “OK” button

8.  “A:\” is displayed then enter “A:\nt” and click “Continue” button

9.  “System Setting Change” dialog is displayed, click “Yes” button to restart FIVA after floppy

diskette Fp3 is removed from FDD

10. FIVA display the warning “At least one service or driver failed during system startup……..”

please ignore this warning message if you do not connect CD-ROM drive.

- Install Service pack 4. Perhaps this service pack is included in Windows NT box.

- <Install PCMCIA ethernet card device driver>

Confirm IRQ and I/O Port address by "Windows NT diagnostic" in Administrative Tools"

IRQ 5 already is used by Audio. If you need IRQ, use IRQ 7.

Open Control Panel and double click "Network" icon.

A warning dialog “Windows NT Networking is not installed. Do you want to install it Now”			appears. Click “Yes” button to start Wizard.

56.  Windows NT 4.0 supply the Internet Explorer 2.0, however if you access Microsoft Web site to

download Service pack 5. or 6, you need IE 3.0 or later.

I installed IE3.0 from CD-ROM disk which Windows 95 CD-ROM supplied.

57.  Install IE 4.0 or 5.0

58.  Install Service pack 5.0

END