This file briefly describes each line contained in the CONFIG.SYS file that is

preloaded on the Thinkpad 701C/701CS. Please note: a backcup of the orginal

CONFIG.SYS file has been placed in the C:\THINKPAD directory as CONFIG.BAK.

DEVICE=C:\DOS\HIMEM.SYS

HIMEM.SYS is an extended memory manger provided with PC DOS.  Please refer

to the PC DOS User's Guide for more information.

DOS=HIGH,UMB

This statement performs two basic function:  First, it loads significant

portions of PC DOS into the high memory area.  Second, it allows PC DOS to

load programs and device drivers into upper memory blocks.  Both of these

functions free up conventional memory below 640K.  See chapter 10 in the PC

DOS 6.3 User's Guide for more information on the High Memory Area and Upper

Memory Blocks.

FILES=30

This statement sets the number of files that PC DOS can access at one time

to 30.

BUFFERS=10

This statement tells PC DOS how many disk buffers to allocate.  Each buffer

requires approximately 532 bytes of memory.  Please refer to the PC DOS

User's Guide for more information.

STACKS=9,256

This statement allocates 9 stacks of 256 bytes each for programs to use when

hardware interrupts occur.

DEVICE=C:\DOS\EMM386.EXE NOEMS I=B000-B7FF X=CC00-CFFF

EMM386 is a device driver that provides support for Expanded Memory and

Upper Memory Blocks.  The NOEMS parameter dictates that no page frame will

be allocated thus providing no Expanded Memory support.  The I=B000-B7FF

parameter specifies that this memory range can be used as UMBs.  The

following statement must be present in the C:\WINDOWS\SYSTEM.INI file under

the [386Enh] section in order for this to work:

device=c:\dos\monoumb.386

The X=CC00-CFFF parameter tells EMM386 not to use memory in this range for

UMBs.  This memory is set aside for use by PCMCIA adapters that you may

install.  Other unused memory space in the C000-EFFF range is automatically

allocated for use as UMBs by EMM386.  Please refer to the PC DOS User's

Guide for more information.

DEVICEHIGH=C:\THINKPAD\IBMDSS01.SYS /D

This statement loads the PCMCIA Socket Services into upper memory.  The /D

parameter allows warm docking when used with a Dock II docking station.  If

you are not using a Dock II then removing the /D parameter will reduce the

amount of upper memory this driver uses by approximately 1K. Please refer

to the READ.ME file in the C:\THINKPAD directory for more information.

DEVICEHIGH=C:\THINKPAD\IBMDOSCS.SYS

This statement loads the PCMCIA Card Services into upper memory.

Please refer to the READ.ME file in the C:\THINKPAD directory for more

information.

DEVICEHIGH=C:\THINKPAD\DICRMU01.SYS /MA=CC00-CFFF

This statement loads the PCMCIA Resource Map utility into upper memory.  The

/MA parameter tells Card Services what locations in upper memory can be used

by PCMCIA cards, and must match or be a subset of an excluded memory range

on the EMM386 line. Please refer to the READ.ME file in the C:\THINKPAD

directory for more information.

DEVICEHIGH=C:\THINKPAD\$ICPMDOS.SYS

This statement loads the PCMCIA power management utility into upper memory.

Please refer to the READ.ME file in the C:\THINKPAD directory for more

information.

DEVICEHIGH=C:\DOS\POWER.EXE

This statement loads the PC DOS Advanced Power Management driver into upper

memory.  This driver allows APM aware operating systems to directly control

various system functions to reduce power consumption.  With this driver

loaded you must have your system type set as "MS DOS System with APM" under

windows setup in order for suspend and resume to work correctly under

windows.

DEVICEHIGH=C:\THINKPAD\AUTODRV.SYS C:\THINKPAD\AUTODRV.INI

This statement loads the PCMCIA super client driver into upper memory.

Please refer to the READ.ME file in the C:\THINKPAD directory for more

information.

REM DEVICEHIGH=C:\THINKPAD\DISKDRV.SYS

This statement when un-REMed will load the PCMCIA disk driver into upper

memory.  This driver is used for PCMCIA hard disks and other ATA compatible

PCMCIA devices (like SRAM cards).  If the /D parameter is specified on the

socket services line then four (4) drive letters will be allocated to the

ATA devices.  If the /D parameter is not specified on the socket services

line then two (2) drive letters will be allocated for the ATA devices.

These drive letters will be allocated regardless of whether or not any ATA

devices are installed in the system. Please refer to the READ.ME file in

the C:\THINKPAD directory for more information.

REM DEVICEHIGH=C:\DOS\SETVER.EXE

This statement when un-REMed will load SETVER into upper memory.  SETVER is

a utility that will allow you to run some applications that require a

different version of DOS.  Please refer to the PC DOS User's Guide for more

information.