DISKIO - Fixed Disk Benchmark, Version 1.14

(C) 1994-1998 Kai Uwe Rommel

Dhrystone 2.1 C benchmark routines (C) 1988 Reinhold P. Weicker

This is a simple fixed disk and CD-ROM drive benchmark program for

OS/2. It measures (approximately) the following numbers:

Hard disks:

- drive cache/bus transfer rate (introduced with version 1.6)

- data transfer rate (on inner and outer tracks)

- CPU usage of full speed disk I/O (introduced with version 1.9)

- average latency time

- average data access time

CD-ROM drives (introduced with version 1.10):

- data transfer rate

- CPU usage of full speed data reads

- average data access time

Concurrent hard disk and CD-ROM drive benchmark (introduced

with version 1.10):

- hard disk data transfer rate

- CD-ROM drive data transfer rate

- CPU usage for concurrent full speed I/O on both devices

It also measures CPU speed without disk I/O using the Dhrystone CPU

benchmark which is needed as calibration for determining the CPU load

caused by disk I/O.

Of course all values are influenced by other activity on the system,

so stop all other software if you want reliable results. The results

will also differ slightly between repeated runs.

The benchmark program needs about 105 seconds to complete on a typical

system with one hard disk and one CD-ROM drive. On system with more

drives it needs about 5 seconds to calibrate, 60 seconds per hard

disk, 30 seconds per CD-ROM drive and 10 seconds for the concurrency

benchmark.

The individual benchmarks are run on all hard disks and CD-ROM drives

in the system, if started without arguments. The concurrency benchmark

is run using the first hard disk and first CD-ROM drive in the system.

To run it only for a specific drives, use command line arguments.

Enter "diskio -?" to get a syntax description for the command line

arguments.

The CPU load caused by the disk I/O of the benchmark routines is

"measured" by first calculating the Dhrystone benchmark result without

any disk I/O and then the number of Dhrystone's the CPU could run in

the background during full speed read operations on the devices. The

lower the percentage number, the less CPU time is (ab)used by the disk

interface. This depends on the quality and technology of the disk

interface (i.e. if it uses PIO or DMA, IDE or SCSI etc.), but also on

the speed of the host CPU and the speed of the devices.

A few hard disk sample results, numbers listed are the data transfer

rate on track 0 (k/sec), latency time (ms), access time (ms),

cache/bus speed (k/sec), CPU usage (last two numbers only if result

was available):

- Pentium PCI/SCSI systems:

Quantum Atlas + NCR 53c810 + PPro-200:		6932, 0.6, 14, 8060, 15%

Quantum Atlas + NCR 53c810 + P-100:		6914, 0.8, 14, 7515, 18%

Quantum Atlas + AHA-2940 + P-133:		6800, 1.3, 13, 6900, 20%

HP C3323 + NCR 53c810 + P-100:			4569, 1.2, 18, 7016, 21%

IBM DPES-31080 + NCR 53c810 + P-100:		4186, 1.1, 18, 7008, 15%

DEC DSP-3160 + NCR 53c810 + P-120:		3930, 2, 16, 6370, 15%

DEC DSP-3133L + NCR 53c810 + P-100:		4393, 1.5, 17, 6124, 22%

- Pentium PCI/EIDE (Intel Triton/Triton-II) systems:

Quantum LPS-540A + P-100:			2177, 1.3, 19, 2227, 92%

Quantum LPS-540A + P-100 (TRIOS2.ADD):		3532, 1.5, 20, 6083, 9%

Quantum Fireball 1280A + P-100:			6188, 0.7, 16, 6294, 89%

- 486 PCI/SCSI systems:

HP C3323 + NCR 53c810 + 486-100:		3945, 2, 18, 5045, 54%

IBM DPES-31080 + NCR 53c810 + 486-100:		4239, 1.3, 18, 6613, 46%

DEC DSP-3160 + NCR 53c810 + 486-66:		3930, 2, 16

DEC DSP-3053L + NCR 53c810 + 486-66:		4130, 3, 16

Quantum LPS-340S + NCR 53c810 + 486-66:		1900, 5, 23

Maxtor 7345S + NCR 53c810 + 486-66:		1881, 10, 17

- 486 PCI/IDE systems:

Quantum LPS-540A + IBM1S506.ADD + 486-100:	1796, 1.6, 20, 1820, 94%

- 486 EISA/SCSI systems:

DEC DSP-3210 + AHA-2740 + 486-50:		3650, 2, 17

DEC DSP-3160 + AHA-1742 + 486-66:		3050, 3, 18

- 486 ISA systems:

Quantum LPS-340S + AHA-1542CF + 486-50:		1508, 7, 26

Quantum LPS-540A + IDE + 486-66:		1492, 4, 21

And a few CD-ROM sample results, listing transfer rate (k/sec),

access time (ms) and CPU usage (%):

- Pentium PCI/SCSI systems:

Toshiba 3701 (6.6x) + NCR 53c810 + PPro-200:		990, 160, 14%

Plextor 8x + NCR 53c810 + P-100:			1222, 134, 29%

Plextor 12x + NCR 53c810 + P-100:			1722, 123, 37%

- Pentium PCI/EIDE (Intel Triton/Triton-II) systems:

Sony CDU-311 (8x) + P-100:				1209, 175, 49%

Concurrency benchmark sample results, listing hard disk and CD-ROM

transfer rate (k/sec) and CPU usage (%):

- Pentium PCI/SCSI systems:

Quantum Atlas + Toshiba 3701 (6.6x) + NCR 53c810 + PPro-200:	5121, 861, 21%

Quantum Atlas + Plextor 8x + NCR 53c810 + P-100:		4975, 1206, 38%

- Pentium PCI/EIDE (Intel Triton/Triton-II) systems:

Quantum Fireball 1280A + Sony CDU-311 (8x) + P-100:		3565, 1165, 99%

There is no warranty. Use this software on your own risk. Due to the

complexity and variety of today's hardware and software which may be

used to run this program, I am not responsible for any damage or loss of

data caused by use of this software. It was tested very well and is

expected to work correctly, but nobody can actually guarantee this for

any circumstances. And because this software is free, you get what you

pay for ...

This program can be used freely for private or educational purposes.

If you want to use it for commercial purposes or want to integrate it

into another package or find any bugs or have suggestions about

further enhancement, please contact the author.

Author:   Kai Uwe Rommel

Muenchen, Germany

Fax:      +49 89 324 4524

E-Mail:   rommel@ars.de (preferred)

rommel@leo.org