About FRATE.DAT file

====================

A file named FRATE.DAT is created in the \IBMVESA\ directory during

the installation.  Please modify the FRATE.DAT file with an editor

to match the frequency capabilities of your monitor.

FRATE.DAT Data Format

---------------------

A semicolon in the first column identifies a comment line.

'FRATE' is the keyword to recognize the refresh rate( scan rate) frequency.

The syntax of the line is as follows:

FRATE=XXXX

������ 4th column is 800x600 mode frequency.

���     '0' : Reserved

���     '1' : 60Hz for both 16/256 color

���     '2' : 72Hz for both 16/256 color

���     '3' : 72Hz for 16 color/60Hz for 256 color

���     'X' : Not supported

���

������ 3rd column is 1024x768x16 mode frequency.

��      '0' : 43.5Hz Interlaced

��      '1' : 60Hz

��      '2' : Reserved

��      '3' : Reserved

��      'X' : Not supported

��

������ 2nd column is 1024x768x256 mode frequency.

�       '0' : 43.5Hz Interlaced

�       '1' : 60Hz

�       '2' : Reserved

�       '3' : Reserved

�       'X' : Not supported

�

������ 1st column is 640x480x256,640x400x256 and standard VGA mode

frequency.

'0' : 60Hz (set non-iso CRT clock range forcedly)

'1' : 75Hz (set iso CRT clock range forcedly)

'2' : 72Hz for 256 color/60Hz for VGA mode

'3' : 75Hz for 256 color/60Hz for VGA mode

Example-1

;S-VGA mode         F Rate         Line Rate      Video Clock

;640*480(256)       60Hz           31.469KHz      25.175MHz

;800*600(16/256)    Not supported

;1024*768(16/256)   Not supported

FRATE=0XXX

Example-2

;S-VGA mode         F Rate          Line Rate      Video Clock

;640*480(256)       60Hz            31.469KHz      25.175MHz

;800*600(16/256)    72Hz            48.077KHz      50.000MHz

;1024*768(16/256)   60Hz            48.363KHz      65.000MHz

FRATE=0112