Readme.txt - PMfax

(Text copy -- Read readme.htm with your internet browser)

Copyright (c) 1992-2000 by Keller Group Inc. All rights reserved.

www.kellergroup.com

Table of Contents

HARDWARE

- Modems

- Multiline Notes

- Serial cards and drivers

- Brooktrout Boards

- GammaFax Boards

SOFTWARE/APPLICATIONS

- Scanner & OCR Support

- Integration with Other Applications

QUICK START GUIDE FOR PMFAX OS/2

- Introduction

- Desktop Features

- Faxing from OS/2 Apps

- Faxing from DOS Apps

- Faxing from Windows Apps

- Solving Problems

- Why should I get the Pro version?

NEW FEATURES & DOCUMENTATION

- New in PMfax Lite

- New in PMfax Pro

- New in PMfax LAN

- Documentation for some newer features

---------------------------------------------------------------------------

HARDWARE

MODEM HARDWARE

PMfax supports a very wide variety of modem hardware. Most modern modems

support either the Class 1 or Class 2.0 fax command set, but PMfax also

supports some older fax command sets.

To configure PMfax to use your fax modem, do the following:

1. Open the PMfax settings notebook (on the Utilities menu).

2. On the Ports page, select the port used by your modem.

3. On the Modem Type page, select your modem's fax command set (Class 1

or Class 2.0).

When in doubt, first try Class 1. If you get a "modem error" message when

you try to send a fax, then try Class 2.0. Selecting the wrong values will

not hurt anything, so give it a try and see what works best for your

hardware.

Using an external modem is often easier than using an internal modem. Some

modems are "Winmodems" that only work when used with a Windows software

driver, so they won't work on other operating systems. Some "Plug and Play"

internal modems can be difficult to install on non-Windows systems because

their "switchless" configuration uses drivers or software that may require

Windows.

When it comes to fax and voice support, the best modem is not necessarily

the fanciest or the most expensive. Simple modems based on standard

Rockwell or Lucent chip sets provide very good performance for both fax and

voice. Modems that send data at high speed still send faxes at a maximum of

14.4 kb (for compatibility with the millions of fax machines - after all,

that's what makes faxing so useful), so a simple modem using a standard

chip set may be better than a fancy data modem using a proprietary chip

set.

Modems are very inexpensive these days, so you may want to test a few

different modems from several manufacturers. Modems use different command

subsystems for data, fax and voice activities, so the best data modems are

not necessarily the best for fax or voice functions.

Additional notes are below. Manufacturers are constantly changing their

modem hardware and firmware so it is difficult to reference specific model

numbers, but if your modem doesn't seem to work correctly these notes may

suggest a fix.

VOICE - USING PMFAX VOICE FEATURES WITH MODEM HARDWARE

Using a voice/fax modem based on the Rockwell or Lucent chip set is

recommended, but others are also supported. Some modems do not support

voice, so check your modem's package or documentation to determine if it is

a voice/fax modem.

Some modems do not follow our supported voice data standards and must be

run in "native" mode. These modems, when used in "wave" mode, will have

either poor sound quality or total noise. If you change to "native" mode,

you must attach your speaker/microphone/telephone directly to you modem and

rerecord your outgoing message and any other voice prompt files using your

modem to get them into your modem's native format.

When using voice features, we recommend loading our fmd.sys driver (and

then selecting an FMD port on the Ports page of the Settings notebook).

This driver is optimized to handle fax/voice issues, but the standard COM

ports will often work fine, too. On some systems, checking the "Use HW FIFO

(16550A)" checkbox on the Modem Type settings page is also required for

good voice quality.

Some examples of modems that have been tested or reported to work (many

others also work):

Voice with "Rockwell command set" and "wave format" (Multimedia support):

Cardinal - 14.4/28.8 Fax Modems with Voicemail/Speakerphone

Practical Peripherals - 336 Ext Voice Model No. 5638US

Reveal - Quad Office/MV500/Decathlon XL

Zoom - Voice VFP 14.4V

Shark - Multimedia Baby Tiger 288-R

Radicom - 33.6 voice

Motorola - 56k VoiceSurfer (External)

VOBIS/Elsa - Highscreen Bahn Boostar 56k future

Voice with "Rockwell command set" and "native" format:

Hayes - Optima 288 V.34/V.FC+FAX+VOICE

Voice with "U.S. Robotics command set" and "native" format:

USRobotics - Sportster internal Voice 28.8 w/Voicemail Model 1171

USRobotics - Sportster external Voice 28.8 w/Voicemail Model 1172

Voice with "U.S. Robotics command set" and "Wave format":

USRobotics - Sportster 56000 Voice V4.5.1 or later (ATI3 to check)

Voice with "ZyXEL command set" and "native" format:

ZyXEL - Omni 288s

Voice with "Cirrus Logic command set" and "wave format" (OS/2 Multimedia):

Jaton Communicator - (CL-MD3452 chipset with IS-101 voice)

Voice with Brooktrout adapter and "wave format" (OS/2 Multimedia):

Brooktrout - TR114 series

FAX - NOTES FOR CERTAIN MODEMS

For most Class 1 modems, using the 'Check TCF signal' on the Modem Type

page of the Settings notebook will provide enhanced speed downgrading. This

may improve the quality of received faxes. On a few modems, engaging this

switch will cause the modem not to initialize properly, so use this switch

with care. We don't recommend using this switch with a US Robotics

Sportster modem.

Most modems do NOT need any special settings. A few older modems that may

need special settings are listed below. If your model of modem is not one

of these, first try Class 1 or Class 2.0 without any special settings and

test your modem for faxing with the PMfax software.

DELL - Digitan (older Dell desktop units)

Class-2, XON Receive start, no DLE esc

DELL - Datarace (older Dell laptop units)

Class-1, Special command \J0\Q2

Dynalink - V1433VQE

Class 1, Check TCF Spec cmd \A3\N3%C3%E2S24=15S38=10

ELSA - 2460, MicroLink 14.4T/TM/TL

Class-2, Special command \Q1

14.4T with ROM 2.01 also needs Special command &K1&R7

Everex - EverFax 24/96 (COM1/2 only)

Class-2, Alt bit order, XON Receive start

Megahertz - 96/96 FAX/Modem (P296FMV)

Class-1, Special command \J0\Q1

Megahertz - 14400 Fax/Modem (XJ244)

Class-2, XON Receive start, No DLE esc

Megahertz - 14400 Fax/Modem (XJ1144)

Class-1, Special command &K4

MICC - 9642 Class-2 MicroCom user reported:

Class-1, Special command &C1

MultiTech - MultiFaxPC MT932, Multi Modem MT932EAF, II, V.32 Fax

Class-2, Alt bit order

Reveal - Quad Office/VM500/Decathlon XL

Class-1, Special command &C1

Supra - SupraFAXmodem V.32bis

Class-1 or Class-2 Silent Answer feature is supported

Special command +FAA=2 for Class 2 with Silent Answer,

and +FEA=2 for Class 1 with Silent Answer

USRobotics - Courier HST Dual Std w/Fax

Class-1, Special command &H2, Receive EOP delay

USRobotics - Sportster 14,400 or 28,800

Class-1, Receive EOP delay, Special command &H2&I0&R1

If sending to Pitney Bowes machines, also use S36=0

Do not use 'Check TCF signal' with this modem

USRobotics - Sportster Voice 28.8 with Personal Voicemail Model 1171/1172

Class-1, Receive EOP delay, Special command &H2&I0&R1S36=0

Do not use 'Check TCF signal' with this modem.

USRobotics - Sportster Voice 33.6

Class-2.0 Special command &H2&I0&R1S36=0

USRobotics - Sportster 56000 Voice (x2 modem)

Need V4.5.1 FLASH ROM or later from USR (ATI3 to check)

Class-1 (or try Class-2.0) Special command &H2&I0&R1S36=0

USRobotics - Courier 28800

Class-1, Special command &H2&I2, Receive EOP delay

USRobotics - V.terbo 21600 w/14400 Fax

Class-1, Receive EOP delay

USRobotics - Worldport 2496 V.42bis, 9696 MNP5 FAX

SendFax

USRobotics - Worldport 9696 V.42 w/Fax, 14400 V.42 w/Fax

Class-1, Special command &H2

Zoltrix - 33.6 fax/data/voice (TI RK Voice chipset - user tested)

Class-2.0 Spec cmd: &A3&B1&H1&R2&D2&C1X4S0=0S7=90

(#CID=1 for caller ID enabling)(not supported for voice)

Zoom - 56kbps FAX modem, Model 2819

Class-1, Skip TCF read

MULTILINE NOTES

If you have purchased additional line options for your program, you can

install multiple fax hardware devices and fax telephone lines, and then do

simultaneous fax sending and/or receiving on these lines. PMfax can support

up to 96 lines per CPU, and the LAN product has optional "multiple fax

server" support for even more capacity.

On a given CPU, the multiple fax hardware devices must be of the same type.

The same configuration page in the Settings notebook controls the

configuration for all the fax hardware devices. The Ports page in the

Settings notebook is used to configure the individual fax lines for

sending, receiving or both. The selected lines are automatically used for

simultaneous sending/receiving.

Fax modems - Either internal or external fax modems can be used, and while

the modems do not need to be identical, they must all use the same values

on the Modem Type page in the Settings notebook. Each fax modem must be on

a separate COM port in your system, and the COM ports should generally be

standard hardware chips (8250, 16450, 16550, or 16550A equivalents),

although the ports need not be at the conventional COM port address and IRQ

settings since you can specify these settings on the FMD.SYS line in your

CONFIG.SYS file. See the next section for a discussion of serial boards

(both "non-intelligent" and "intelligent") for adding COM ports. For large

port counts, we recommend using ports with 16550A UARTs and Class 2.0 fax

modems.

Brooktrout boards - If you have properly installed the OS/2 runtime

software from Brooktrout, the Brooktrout ports will appear on the Ports

page in the Settings notebook. It is possible to run up to 96 lines using

Brooktrout fax hardware. The boards require Brooktrout's OS/2 driver at a

proper version level for your fax software. You can download the needed

Brooktrout files from our web download area. See the installation notes for

Brooktrout hardware.

GammaLink boards - The GammaFax ports will appear on the Ports page in the

Settings notebook. The fax program interfaces with the GammaFax boards via

the GammaFax Dispatcher, so it can operate simultaneously with many other

GammaFax software products which also use the GammaFax Dispatcher. See the

installation notes for GammaFax hardware.

SERIAL CARDS AND COM DRIVERS (UARTS, INTELLIGENT PORTS)

The multiline versions of the fax software can support up to 96 fax lines,

but it can be difficult to find enough ports, interrupts or slots to attach

modems to your PC.

A serial port board might be the answer. Boards that provide real hardware

UART ports (compatible with standard PC COM ports) are supported, and the

fax software's optimized device driver (FMD.SYS) allows you to use

nonstandard port I/O addresses, nonstandard interrupts and shared

interrupts. Serial boards are commonly available in 4-, 8- and 16-port

versions for both ISA and MCA machines, and we've tested many products

including boards from DigiBoard, STB, GTEK, and Quatech. For large systems,

we generally recommend using boards with 16550A UARTs and Class 2.0 fax

modems.

16550A UART Support - The fax software can take advantage of 16550A UART

chips if they are used in your PC serial port, internal fax modem or serial

port board. The use of 16550A UARTs is not required, but they will decrease

the system load during fax sending and receiving with fax modem hardware

since the 16550A hardware buffering substantially reduces the interrupt

rate.

Intelligent Boards - "Intelligent" serial boards (and some types of

non-standard fax modems) do not provide physical UART ports, so they cannot

be used through the optimized FMD.SYS OS/2 device driver. Instead, they

provide a replacement for OS/2's COM.SYS device driver which makes their

ports accessible as COM devices, and the ports can only be accessed through

their proprietary COM.SYS replacement driver. If you wish to try using such

"intelligent" boards, simply select the COM ports on the ports settings

page. This has been tested with COM.SYS and SIO.SYS, and should work with

other COM.SYS replacement drivers. Because the performance of this

interface is dependent upon the characteristics of the COM.SYS-type driver

which is provided by other parties, we cannot provide warranty or support

for use of these "intelligent" boards. But this can be very useful since,

for example, SIO.SYS supports special serial ports like the Hayes ESP and

Telcor Tport which are not directly supported by our FMD.SYS driver. You

should test your specific configuration to see if it works in an acceptable

manner. We continue to recommend use of FMD.SYS or the standard OS/2

COM.SYS driver for all devices which provide actual UART ports.

BROOKTROUT HARDWARE

Brooktrout's OS/2 driver (version 3.7) is required for Brooktrout TR112,

TR114, and TruFax boards. This driver is available in our web download

area.

To our knowledge at the time of writing, Brooktrout has not released PCI

support for OS/2, so be sure to use ISA boards for the OS/2 platform.

To install the Brooktrout fax hardware, first follow the instructions in

Brooktrout's hardware guide to install your Brooktrout fax boards, then

follow these instructions to install the Brooktrout OS/2 driver software.

[The destination directory for the Brooktrout software is assumed to be

C:\BFAX for the following steps. Please change the commands as needed for

your installation directory.]

1) To create the install directory, type:

MKDIR C:\BFAX

2) Copy BFAX.SYS, FAXINIT.EXE, *.CFG, and *.PEX into C:\BFAX. If a

directory other than C:\BFAX is used, BTCALL.CFG will need to be edited and

all four paths changed from C:\BFAX to the proper directory.

3) Edit your C:\CONFIG.SYS to add an appropriate line like this one:

DEVICE=C:\BFAX\BFAX.SYS D1 I12 N24

This line is for DMA channel 1, interrupt 12, and 24 channels. These

settings should match the jumpers set on the boards. Modify the D1 or I12

as appropriate. The N24 (or other number) is only required for more than 16

channels. Please make sure any DEVICE= lines from older Brooktrout BFAX

drivers are removed from your CONFIG.SYS and that there are no device

conflicts with the DMA channel and interrupt number you have chosen.

4) Edit your C:\STARTUP.CMD to add the following line:

C:\BFAX\FAXINIT

(or put "RUN=C:\BFAX\FAXINIT.EXE" after the BFAX.SYS line in your

CONFIG.SYS).

5) Install the OS/2 fax software. With PMfax version 4, select Brooktrout

as the fax hardware. If you have a CD-based copy of PMfax v3 and use the

install.exe program, select "Brooktrout BFAX" as the fax hardware type when

running install. If you have a web-downloaded copy of PMfax v3, by sure to

replace the default fax.adp file in the web download with the version of

this file for Brooktrout hardware by getting the "Brooktrout fax adapter"

file from our web download area.

6) You will need to reboot for OS/2 to load the new BFAX.SYS driver.

7) On the Utilities/Settings/Brooktrout screen, specify the path:

C:\BFAX\BTCALL.CFG

Note that FAXINIT.EXE, BFAX.SYS, *.PEX, and *.CFG are from Brooktrout. We

will do our best to help out, but any real problems should be addressed to

Brooktrout.

We have noticed some things about Brooktrout's FAXINIT program. There is an

undocumented range of addresses that FAXINIT looks for. Here is what we

know about FAXINIT:

FAXINIT doesn't load firmware into the cards, it just finds them. After it

peeks around at the board addresses, it tells the BFAX.SYS driver where

they are. (The driver then tells TR114 cards what DMA to use.) This means

that FAXINIT needs to know where to look. FAXINIT will automagically find

cards at the following addresses:

140 144

210 214 218 21C

220 224 228 22C

230 234 238 23C

240 244 248 24C

250 254 258 25C

260 264 268 26C

270 274 278 27C

If your card address is not on this list, it WILL NOT be found by default.

If you choose non-standard addresses, you must create a configuration file

listing the port address of each card, even if some of the cards have

standard addresses. Addresses must be in hex format. For example, if you

have 3 cards, with non-standard addresses at 300, 350, and 354, list them

in your configuration file in the following way:

addr 300

addr 350

addr 354

or:

addr 300

addrs 350 2

To execute FAXINIT when you are using non-standard addresses, use the

following command where filename is the name of your configuration file:

faxinit -n filename

GAMMAFAX HARDWARE

PMfax supports used of the intelligent GammaFax boards, such as the CPi,

CP, CP MC, and XPi. Multiport boards also work, including the CP4-LSI and

the MLCP4 with DTI-124 T1 interface board. (XP and XP MC boards are NOT

supported since they are non-programmable and do not support API access.)

PMfax for OS/2 requires use of GammaLink's GammaFax OS/2 dispatcher

(version 5.3).

Install the GammaFax OS/2 software as directed in GammaLink's instructions.

It is recommended that the GammaFax software be installed in a directory

different than the fax program. For our examples that follow, we are

assuming that the GammaFax software is installed in the directory "C:\FAX".

Be sure to modify the pathnames if you use different directories.

Verify that the command script files (C:\FAX\GFQM.CMD and C:\QF.CMD)

correctly reflect the GammaFax software's installation directory. For

example, if the GammaFax OS/2 software is installed in C:\FAX, the first

line of both the files should be "SET GFAX=C:\FAX".

Verify that your CONFIG.SYS file includes the line "SET GFAX=C:\FAX". (You

will need to reboot your system so that the changes to your CONFIG.SYS file

will take effect.)

Edit the configuration file, C:\FAX\GFAX.$DC, to include the lines below,

in addition to or replacing the lines provided by the GammaFax installation

program. The gfx lines should be repeated for each GammaFax channel to be

controlled by the fax adapter and the value 1 should be replaced with the

logical channel number.

To setup each line:

gfxshutdown 1 3         *changed from default 0 for API direct control

To enable DTMF LAN routing for each line:

gfxdtmftimeout 1 6 15   *added to control DTMF interdigit and total t/o

gfxdigits 1 4 3072      *added for 4 DTMF digits and * or # termination

gfxdtmftone 1 440 500   *added to use 440 cps .5 second DTMF prompt

Please refer to the "GammaFax Reference Manual" for more information about

the configuration file (chapter 4 "Configuring the GammaFax System" in

version 5.x).

An example of a complete configuration file is included at the end of this

section.

Start C:\FAX\GFQM.CMD in an OS/2 window. If desired, you can add

C:\FAX\GFQM.CMD to your STARTUP.CMD or create a program object with the

Path and File Name set to C:\FAX\GFQM.CMD and place the object in your

startup folder.

Start the fax program and set the ports to be controlled using the Ports

page in the Settings notebook. Also in the Settings notebook, set the

proper phone line type (recognizable dial tone or not) and dialing style

(tone or pulse).

---------

This is an example of a 2-line multiline configuration file, and also

demonstrates the use of DTMF input for routing incoming faxes.

From what we understand, the CPi and XPi boards are like the CP and XP, but

with built-in DTMF reception - the CP board requires an optional daughter

board for DTMF input.

The lines in this example were rearranged so all that reference a channel

are grouped together. This does not affect the operation.

chassis 1

buffers 4

numchan 2

channel 1 0 GFAX1.01

country 1 1

init 1

load 1 C:\FAX\GFXCX.BIN

gfxshutdown 1 3         *changed from default 0 for API direct control

gfxdtmftimeout 1 6 15   *added to control DTMF interdigit and total t/o

gfxdigits 1 4 3072      *added for 4 DTMF digits and * or # termination

gfxdtmftone 1 440 500   *added to use 440 cps .5 second DTMF prompt

channel 2 1 GFAX1.02

country 2 1

init 2

load 2 C:\FAX\GFXCX.BIN

gfxshutdown 2 3         *changed from default 0 for API direct control

gfxdtmftimeout 2 6 15   *added to control DTMF interdigit and total t/o

gfxdigits 2 4 3072      *added for 4 DTMF digits and * or # termination

gfxdtmftone 2 440 500   *added to use 440 cps .5 second DTMF prompt

SOFTWARE/APPLICATIONS

SCANNER SUPPORT

Native scanner support software is available from several vendors, and most

of the scanner packages can easily be used to scan and fax documents with

our fax software. Because any scanning software can generally print the

scanned image to your printer, it can also print the scanned image to our

fax printer driver (FxPrint) to convert it into a fax document.

For the OS/2 platform, information on using some of the OS/2 scanner

packages with the fax software:

Applause and other scanning and OCR products [Solution Technology -

http://www.stiscan.com] - Image utilities for OS/2 - support for HP ScanJet

and Logitech scanners. Scanned images can be turned into fax documents by

printing to FxPrint, Clipboard cut/paste, or using Applause's Edit/Scale to

FAX command and then using File/Save As to produce a DCX-format file which

can be read with the fax program's Fax/Open file/DCX command.

CopyShop/2 [Trafalgar Business Systems - http://www.tbsny.com] - Utility

for "scan & fax", "scan & print" and "scan & view" using HP ScanJet

scanners. By selecting FxPrint as the printer for CopyShop/2, you get a

"single-button" way to scan a document and get the Fax Send dialog. For

multi-page faxes, it supports automatic document feeders and, for those

without auto feeders, it allows a series of single page scans to be treated

as a single fax job.

Galleria [Bitware Australia, information at

http://www.os2ss.com/select/applications/graphics/galleria.html] - Image

utility for OS/2 - support for HP ScanJet scanners. Scanned images can be

turned into fax documents by Clipboard cut/paste or printing to FxPrint.

INTEGRATION WITH OTHER APPLICATIONS

PMfax is designed to cooperate with other applications. Some examples:

SCANNING - As described above, some OS/2 scanner packages support "scan and

fax" with the fax software.

INTERNET E-MAIL FAXING - The software can send and receive fax documents

via e-mail by cooperating with various Internet E-mail products. When our

fax software and cooperating e-mail products are used for sending and

retrieving the e-mail, the fax document is transparently delivered to the

receiver and appears in their fax log as a received document. Advanced

"Internet Relay Systems" for Internet fax delivery are also supported. See

our web page on Internet Faxing for the latest information on this feature.

FAX MERGING - Some OS/2 word processors, including DeScribe v5.0 and IBM

Works (in the BonusPak), support fax merging within their mail merging

features. Other word processors (even Windows or DOS word processors) can

often support such automated faxing by embedding the >>TO printer driver

command in their document or by using word processor macros which pass the

>>TO command to the FxPrint printer driver in various ways as described in

the Reference Manual. You can also do a type of "fax merging" with the new

Edit/Cover sheet command in the fax software.

PHONE BOOK SHARING - The Personal Contact Management Utility (pLog) from

Oberon Software supports the fax phone books, so you can maintain a common

phone book for both fax and voice auto-dialing. The phone book files use a

simple, standard format (comma-separated value ASCII) that can be imported

and exported by most spreadsheet and database programs.

SERIAL PORT SHARING - The "shared mode" feature can often allow data

programs to make outbound calls while the fax software remains in receive

mode. But even if you are using "private mode", you can use the FxRcv.EXE

utility program in a command file to exit receive mode, call your data

program, then enable receive mode again. Some programs (TE/2 from Oberon

Software, Golden CommPass from Creative Systems Programming Corp, and

others) provides fields or scripts using the FxRcv commands to manage the

port sharing.

QUICK START GUIDE

This 32-bit fax software for OS/2 is fully enabled for the OS/2 Workplace

Shell, taking advantage of the latest OS/2 user interface standards.

It allows you to send and receive faxes of unlimited length and print faxes

to any OS/2 printer. The FxPrint printer driver works with the fax program

for creating and sending faxes by "printing" from your OS/2, DOS and

Windows applications running on OS/2. Features include drag-and-drop

printing and viewing, normal and fine fax resolution, portrait and

landscape orientation, multiple page lengths and printer emulation for

print capture (LPT3) from your DOS and Windows applications.

If you have the Lite version and you send or receive faxes on a regular

basis, you will want to upgrade to the Pro version. The Pro version has

many additional features, including a configurable toolbar, fax markup and

editing, drawing, import, export, automatic printing, gray-scale viewing

for enhanced on-screen readability, multiple phone books, enhanced log

selection and maintenance, and printer driver commands. You can also get

LAN versions with automatic routing, multichannel support for up to 96 fax

lines and programming interfaces for developing your own "fax enabled"

applications. To purchase the Pro version or program options, see the

information on the program's Help Product Information screen or contact

Keller Group.

READING THE QUICK REFERENCE GUIDE - A Quick Reference Guide page is

included in the program's fax log. Run the program, open the fax log, and

double-click on the Reference Guide to view the page. The program can print

any fax document to your OS/2 printer, so you can print a copy of the Quick

Reference Guide.

CONFIGURING THE PROGRAM - In the fax program's Settings notebook, set the

port used by your fax modem on the Ports page, your modem baud rate and

line type on the Modem page, and your fax modem values on the Modem Type

page. See the program's README.DOC file for recommended settings for your

modem. The Lite product supports fax modems which follow the Class 1, Class

2/2.0 and SendFax standards. The Pro version includes support for many

additional types of fax hardware.

TO GET HELP - Check the message line as you move the mouse over a toolbar

button or menu item, or press F1 for extensive on-line help information on

any menu item that you have highlighted. The Lite version includes the same

help system as the Pro product, so some of the described features are not

in the Lite version.

TRY THE TOOLBAR - The buttons in the toolbar provide "single-click" access

to the menu items. When you put the mouse over a toolbar button, the

message line tells you about the button. You can drop fax file objects on

some of the buttons to send, print or view the fax. In the Pro version, the

toolbar is configurable using drag-and-drop (drag the buttons, or drag menu

commands to the toolbar).

USE POP-UP MENUS - As with other Workplace Shell operations, a right-click

of the mouse will provide a pop-up menu. The contents of the pop-up menu

will vary based on what the mouse is on when you press the right mouse

button. Different pop-up menus will appear if you right-click on the main

program window, the status window, the fax log window, or the phone book

window.

TAKE ADVANTAGE OF THE DESKTOP FEATURES

Drag-and-drop printing - If you use an OS/2 application that supports

printing of its document file objects by dropping them on printer objects,

then you can drop them on the FxPrint printer object to create and send a

fax. Most modern OS/2 word processors now support this feature.

More drag-and-drop - Double-click on a fax file object to view the fax, or

drag and drop a fax file object to do many useful things. For example, you

can drop it on any printer object to print it, on the fax program object or

into the fax workspace to view it, on the Send toolbar button to send it,

on the Print toolbar button to print it, on any of the View or Orientation

toolbar buttons to view it in that size or orientation, or on the Log

toolbar button to add a copy of the fax to the fax log for later viewing or

sending.

Resizable windows - Drag the borders of the program, log and phone book

windows to suite your preferences, and the program will remember the sizes.

Fonts of your choice - Drag fonts from the OS/2 Font Palette and drop them

in many places, and the program will remember your font choices. Drop them

in the log, in the phone book, on the message line, or on the status

window.

PRINTING FAXES FROM OS/2 APPLICATIONS

The FxPrint printer driver is a true OS/2 printer driver, and you use it

just like any other OS/2 printer driver. The FxPrint printer driver turns

your print job into a fax document file, and an optional Send pop-up dialog

allows you to send the fax.

To create and send a fax from an OS/2 application, use the OS/2

application's Printer Setup command to select the FxPrint printer object,

then use the OS/2 application's Print command.

If you are using an OS/2 application that supports printing of its document

file objects by dropping them on a printer object, then you can drop them

on the FxPrint printer object to create and send a fax. Most modern OS/2

word processors now support this feature.

You can configure the FxPrint printer driver for various page lengths,

orientations, and other settings. The on-line help explains the various

options in detail.

Note that you can create several printer objects which use the FxPrint

printer driver, and each can have different "Job Properties" in its

settings notebook.

PRINTING FAXES FROM DOS APPLICATIONS

The FxPrint printer driver provides emulation for the IBM Proprinter X24

printer, so your DOS programs can create and send fax documents by printing

to the FxPrint printer object's LPT device.

By default, the FxPrint printer object is usually attached to LPT3. FxPrint

should be attached to an LPT device that is not used by any other printer

object. You can check and set the LPT device on the Output page of the

FxPrint printer object's settings notebook.

To create and send faxes from DOS programs running on OS/2, have your DOS

program use LPT3 for printing (or whatever LPT device you have set on the

Output page of the FxPrint printer object). If possible, tell your DOS

program that LPT3 is an IBM Proprinter X24 printer. This will allow the DOS

program to use the Proprinter's fonts and graphics. To produce a fax

document that is easier to read, tell your DOS program to use the

Proprinter's "Elite PS" font or a bold font.

If you want the results to be larger on the fax page, change the LPT

Emulation setting from "IBM Proprinter X24E" to "IBM PP X24E Enlarged" by

using the Job Properties button on the Printer driver page of the FxPrint

printer object's settings notebook.

If your DOS program can only produce high-quality printed output when using

a LaserJet printer, then you may want to upgrade to the Pro version and the

optional PCL5 emulation feature. This allows your DOS and Windows programs

to print to the fax printer driver as if it was a LaserJet III printer.

PRINTING FAXES FROM WINDOWS APPLICATIONS

As described in the previous section, the FxPrint printer driver provides

emulation for the IBM Proprinter X24 printer. Your Windows programs can

print to the Proprinter Windows printer driver, and that printer driver can

pass the data to the FxPrint printer object's LPT device to create a fax

document.

You must do a onetime installation/configuration of the Windows printer

driver. To do this:

1. Run a full-screen Windows session, such as by double-clicking on the

WIN-OS/2 Full Screen object in the Command Prompts folder.

2. Open the Windows Control Panel (in the Main Group), then open

Printers.

3. Press the Add button, select IBM Proprinter X24, then press the

Install button. You will be prompted to insert a printer driver

diskette for installing the Proprinter Windows printer driver. If you

are running OS/2 for Windows, this will be one of your Windows

installation diskettes. If you are running regular OS/2, this will be

one of your OS/2 installation diskettes.

4. Press the Connect button and set the printer driver's device to the

same LPT device used by the FxPrint printer driver (like LPT3.OS2).

5. Installing and using the ATM (Adobe) fonts is recommended (see your

OS/2 documentation).

To create and send faxes from Windows programs running on OS/2, use the

application's Printer setup command to select the Proprinter printer

driver, then use the application's Print command.

If you want the results to be larger on the fax page, change the LPT

Emulation setting from "IBM Proprinter X24E" to "IBM PP X24E Enlarged" by

using the Job Properties button on the Printer driver page of the FxPrint

printer object's settings notebook.

If you upgrade to the Pro version and the optional PCL5 emulation feature,

then your Windows programs print to the fax printer driver as if it was a

LaserJet III printer.

IF YOU HAVE PROBLEMS

Most fax problems are due to improper modem settings and are easily

corrected. Be sure to see the notes and suggestions about your fax hardware

at the beginning of this file.

Port selection - Be sure that the fax program is using the correct port by

checking the Ports page in the program's settings. If your fax hardware is

turned on and attached to the specified port, they you will hear it pick up

the phone line and dial when you tell the fax program to send a fax. If it

doesn't, try testing your fax modem with a data communications program to

make sure it is properly attached and working.

Modem settings - Look at the Modem Type page in the program's settings and

check the settings for your fax hardware. See the Fax Modems section of

this document for the recommended settings for your modem. If your modem is

not listed, check your modem manual to determine if your modem uses the

Class 1, Class 2, Class 2.0 or SendFax command set for faxing. If your

modem supports both Class 1 and Class 2/2.0 and you are having problems in

Class 2/2.0 mode, try changing it to Class 1. As noted in the modem

information at the start of this file, some modems also need additional

check boxes or special commands on the Modem Type page.

Finding the fax directory - If the fax program can't seem to find the fax

log, or if you have changed the location of your fax data directory (which

contains FAX.LOG, FAX.IDX, *.FAX and *.PBK files), then you must tell the

program where it is. To do this, start the fax program with the parameter

of

-L newdir

where newdir is the full pathname of the directory which contains the

FAX.LOG file. You only need to do this once, and the fax program will then

remember it.

Capturing details to track down problems - The fax program includes a

facility for capturing detailed information on its activities, including

all the commands and responses with your fax hardware. Technical support

will need this information to provide assistance. To report a problem,

first start the fax program with a parameter of -V, recreate your problem,

close the fax program, and e-mail a copy of the VOUT file (found in your

fax data directory) to technical support.

WANT MORE?

GET IT ALL WITH THE FULL RETAIL VERSIONS (PMfax Pro and PMfax LAN)

The Pro products add many features, including:

* Powerful fax editing, including the ability to sign your fax by

dropping your signature bitmap onto the fax, append other fax

documents by dropping them onto your fax, and type in any font by

dragging the font from the font palette.

* Extended selection in the fax log so you can print or delete multiple

faxes with a single command, a Clean command to remove "informational"

log entries like "busy" or "no answer", and other log maintenance and

fax archiving features.

* Multiple selection in the phone book, and support for multiple phone

books and distribution groups.

* A gray-scale feature for improved viewing of faxes with smaller fonts

and images.

* Options for automatically starting in receive mode and automatically

printing faxes when received or when sent.

* A utility program for changing receive mode from the command line or a

command file (for example, to turn off receive mode so your data

program can use the fax modem), or to tell the program to view or

print a fax.

* LAN versions, multiline versions, developer toolkits for doing fax

broadcasting or client-server programming, and more.

For information on upgrading to the full retail version or purchasing

program options, see the information on the program's Help Product

Information screen.

NEW FEATURES & DOCUMENTATION

NEW IN PMFAX LITE

Some of the new features added in version 3:

* Class 2.0 fax modems are now supported.

* Voice answering machine functions are added for use with supported

voice/fax modems (Rockwell chipset). When enabled, you can use the

same line to record voice messages and receive faxes. This is fully

integrated with OS/2 Multimedia Audio, so you use your OS/2 Multimedia

system to play voice messages on your speakers and record your

greeting message with your microphone.

* Enhanced cover sheets are supported. In addition to the standard cover

sheets where you can select a bitmap image, you can also select from

some enhanced cover sheets which are provided for your use. The retail

version can create and edit these enhanced cover sheets.

* The FMD.SYS driver is no longer required. The Lite version now uses

your COM devices.

* The ability to accept "hot handoff" calls from data programs for

data/fax line sharing is now supported. Data programs which use this

feature can pass the call to the fax program after they have answered

the phone and determined that it is a fax (rather than data) call. Ask

your data communications software supplier if they support this

feature.

* Various keyboard improvements are included for using the fax program

and its settings notebook.

NEW IN PMFAX PRO

Some of the new features added in version 3:

* Enhanced cover sheets are now supported, including full editing so you

can position the fields anywhere on your cover sheet, place your

signature after the comment field, and put logos and images anywhere

on the page. You can even use borders which will automatically stretch

to accommodate long cover sheet comments. [The BonusPak Lite version

includes some new enhanced cover sheets, but does not support cover

sheet creation or editing.]

* Voice support for telephone voice/fax hardware is provided, including

integration with OS/2 Multimedia audio, REXX scripts that you can

modify and enhanced for doing voice mail, remote message and fax

retrieval, fax-on-demand, forwarding, paging and other voice/fax

answering services. Supported hardware includes many voice/fax modems

and Brooktrout TR114 boards. [The Lite version supports a basic voice

answering machine for fax modems but does not support REXX scripts.]

* Internet E-Mail support is provided. This allows you to send and

receive fax documents (and voice, text and data items) via e-mail with

various Internet e-mail products. When our fax software and

cooperating e-mail products are used for sending and retrieving the

e-mail, the document is transparently delivered to the receiver and

appears in their fax log as a received document. The new -Z start-up

parameter and some of the voice scripts can forward your fax and/or

voice messages to your Internet e-mail mailbox, so you can access your

messages and faxes by retrieving your e-mail messages and your faxes

and messages will appear as "received" items in the fax software. The

new "Relay" support also makes it easy to set up Internet "forwarding

nodes" and "Internet Relay Systems" for sending faxes through the

Internet and avoiding long-distance telephone charges.

* The new universal log supports fax, voice, text and data items in the

log. Icons in the log identify the type of the item and show if a fax

has been printed. New page numbering shows if a cover sheet is used,

how many pages were successfully sent before a failure, and so forth.

Text items can be converted for faxing with the click of a button,

including use of printer driver commands to control fonts and formats.

In the LAN version, the voice, text and data items can be routed to

other OS/2 workstations just like fax items.

* You can now drag-and-drop "anything" in the log. You can use

drag-and-drop to copy or move objects of all types (fax, voice, text

and data) from the log to the desktop or a folder. All log record

information stays with the file as extended attributes, so the object

can be fully restored to the log when you drag it back. You can also

append multiple faxes together by dragging them from the log, desktop

or folder to the workspace, play a voice message by dragging it to the

workspace, or view text and data (hexdump) items by dragging them to

the workspace.

* A full REXX API is included. REXX programs can now be used to control

receive mode, send faxes, check job status, process items from the

log, print or delete fax files, modify phone book records, generate

reports from the log or phone books, and more. The REXX interface lets

you create advanced applications which deal with voice, fax, text and

e-mail. The complete REXX API is documented and included in the retail

version. You can distribute your REXX applications without royalty to

other licensed users of the retail products. Your REXX programs will

work with all supported hardware, and will even work with our

multiline and LAN versions.

* Multi-character searching in the log and phone books makes it fast and

easy to locate the item that you need.

* Enhanced drawing tools for lines, boxes, circles, ellipses, arrows and

check marks are provided for the Edit Draw command. Now it's even

easier to fill out forms online!

* Printer driver commands can now be embedded in the documents from your

Windows or DOS programs for automatic sending and other features.

* You can use drag-and-drop from the Color Palette to change the

workspace color or log background color, and Ctrl-Drag to set colors

for different status types in the log.

* Priority sending is provided to allow an important fax to be sent

before other spooled fax jobs.

* Hold mode is provided to temporarily suspend faxing or gracefully shut

down a busy system.

* The intelligent retransmission option can be selected to resend just

the unsent pages. The ability to specify a page range is also included

in the Resend command.

* You can do automatic retry just for BUSY failures or for all failures.

Some users prefer to check the fax number before retrying other types

of failures.

* The Utility Maintain log command now supports automatic daily

operation and the ability to archive the documents and log entries to

an archive directory.

* A new Cut command will separate pages that were "stuck together" when

faxed to you.

* You can now modify a previously sent cover sheet and resend the

document (by using Ctrl-double-click on a log item) or modify a

previously sent cover sheet and apply it to your current document (by

using Alt-double-click on a log item).

* New recipient editing let's you review, edit or delete items in your

list of recipients before sending. Double-click on a recipient in the

Send dialog to get a recipient edit/delete dialog.

* The retail version now allows you to use COM devices (from COM.SYS,

SIO.SYS or other third-party device drivers) or the FMD devices (from

our FMD.SYS device driver). [The BonusPak Lite version now uses only

COM ports, while the retail version also provides the FMD device

driver for port sharing and other features.]

* The Shared option for fax modems waits for the line to become

available for sending if your data communications program is using it,

and (with FMD ports) allows your data communications programs to use

the fax modem without needing to take PMfax out of receive mode.

* The ability to accept "hot handoff" calls from data programs for

data/fax line sharing is now supported. Data programs which use this

feature can pass the call to the fax program after they have answered

the phone and determined that it is a fax (rather than data) call. Ask

your data communications software supplier if they support this

feature.

* You can start current call receiving from the command line or with a

program icon using a new start-up parameter, thereby making it easier

for you to share a line for voice and fax calls.

* Prompting for client data is supported so you can generate charge-back

reports or do other tracking on all your fax usage.

* T.30/T.33 Subaddress Routing is supported with Class 1 fax modems and

Brooktrout cards. This can be used to automatically route faxes to the

recipient when faxing to our LAN products or other LAN fax systems

which support this standard. To use outbound Subaddress Routing,

simply put a ' (single quote) character after the regular fax number

followed by the route digits.

* The LPT49 utility is included to add LPT ports so you can install the

fax printer driver on LPT4 - LPT9 if you are already using LPT1, LPT2

and LPT3 for other printers.

* The fax printer driver is now faster and takes advantage of new Warp

features. It also supports word wrapping in >>FONT emulation mode.

* Hardware support is enhanced and now includes Mwave, SatisFAXtion,

Brooktrout, GammaLink, Class 2.0 modems, modems with Caller ID and

other fax hardware. See the User's Guide or on-line Help for

instructions on using Caller ID.

* The new LAN and Multiline versions now support multiple fax servers,

enhanced automatic routing (by DID, DTMF, OCR, T.30 Subaddress,

TSI/CSID, or line number), multiuser voice messaging and remote

fax/voice retrieval, routing groups, "first page only" viewing option

for manual routing, customizable notification options, improved

support for Notes e-mail, new interface for the fax administrator,

automatic busy call blocking for optimized throughput between

multiline systems, and more.

* The OS/2 programs are now translated into more than 25 languages

including Czech, Danish, Dutch, Finnish, French, German, Hungarian,

Italian, Norwegian, Polish, Portuguese, Russian, Spanish, Swedish, UK

English, Catalan, Bulgarian, Turkish, Greek, Brazilian Portuguese,

Slovenian, Japanese, Korean, Traditional Chinese, and Simplified

Chinese.

* If you are using the Lite version, contact Keller Group

(www.kellergroup.com) or your software reseller to purchase the Pro

version today.

NEW IN PMFAX LAN

Some of the LAN features added in version 3:

* LAN/MULTILINE - multiple fax server facility (multiple cooperating

CPUs)

* LAN - Voice support, including true multiuser routing & remote

retrieval

* LAN - OCR Routing

* LAN - T.30/T.33 Subaddress Routing

* LAN - TSI/Called Subscriber ID Routing

* LAN - Line-based Routing

* LAN - selecting and routing multiple items with one route command

* LAN - routing groups for routing to distribution groups

* LAN - routing all log items, not just received faxes (voice/text/data)

* LAN - "view first page only" security for manual routing

* LAN - fax "Stop" command from OS/2 workstations

* LAN - public phone book creation from OS/2 workstation (add '(p)' to

name)

* LAN - flexible options to customize notification upon

send/receive/error

* LAN - immediate use of any WS.INI changes that you make from your

scripts

* LAN - Lan User Administration utility (container-based, run on

workstation)

* LAN - Options Maintain logs command to clean workstation logs from

server

DOCUMENTATION FOR SOME NEWER FEATURES

Version 3 features:

PRIORITY SENDING - If you check the "Priority" checkbox on the Fax Send

dialog, your job will be sent before other "normal priority" jobs. This is

useful if there are other jobs already waiting to send (such as during a

broadcast), but you want your job to send right away. This works for both

stand-alone and LAN versions. API users can use the new priority field in

the FXINFO structure, and enhanced printer driver users can use the new

priority argument to the >>AT printer driver command (>>AT=,,1 for high

priority).

PRIVATE/SHARED PORT - Fax modem configuration includes a "Private"

checkbox. When not checked (the default), we use the port in a new "shared"

mode which provides two useful features:

1. SHARED PORT - For receive mode, the port is not acquired until the

phone actually rings, so you can leave the fax software in receive

mode while using the fax modem for outbound data program activity. The

port is only "in use" when we are actually receiving or sending a fax.

No need to use the FxRcv utility or commands to take the program out

of receive mode anymore! (This uses our FMD.SYS device driver, and

operates with virtually no system overhead.)

2. WAIT FOR LINE - For sending, if the port is in use by another program

when we want to send a fax, we will wait for up to one hour for it to

become available before reporting an error. When you release the port

(such as by closing your data program), we will send any spooled faxes

that are scheduled for transmission.

When checked, we use the port in "private" mode where we acquire the

port when waiting for incoming calls and expect the port to be

immediately available when we send a fax (like past operation).

IMPORTANT NOTE: The shared port feature depends upon the hardware ring

indicator in your com port chip. If you are using an external fax

modem and your cable does not provide the necessary signals, the fax

software will not know that the line is ringing and will therefore not

answer the incoming calls. IF THE SOFTWARE DOES NOT ANSWER INCOMING

CALLS WHEN SET IN RECEIVE MODE, CHECK THE "PRIVATE" CHECKBOX ON THE

MODEM PAGE OF THE SETTINGS NOTEBOOK.

ALT COM - The fax modem ports list includes both COM and FMD ports. The

software uses our FMD.SYS device driver to access the fax modem serial

ports. When selected, the software uses the COM ports (via COM.SYS,

SIO.SYS, or other replacement COM drivers) for accessing the fax modem

serial ports. This is most useful when you need to use special COM device

drivers for intelligent serial port cards such as Digiboard, Artic, or

Hayes ESP (via SIO). When COM ports are used, the "shared port" feature of

FMD.SYS is not available, but the "wait for line" feature still works.

LAN - ENHANCED LAN ROUTING AND ROUTING GROUPS - For LAN systems running in

private mode, the routing capabilities have been substantially enhanced:

1. You can now route any type of fax... not just received faxes.

2. You can select multiple faxes in the log and route the entire set of

faxes with a single Route command in the log pop-up menu.

3. Routing supports "groups". Just as in phone books, you can now specify

"group names" for workstation users in the FxRdr program, and the

Route command adds a Group button for selecting a routing group.

CLIENT NUMBER DATA FOR CHARGE-BACK REPORTS - You can now make the software

prompt the user for "client information" when sending a fax, and even

prevent them from sending a fax without selecting a valid client number.

This information is saved in the "notes" field of the fax log and the

activity report, from which you can easily generate charge-back reports and

usage reports. This is controlled by the presence of the FxNotes.INI file

in the log directory (for stand-alone systems or shared mode LAN systems)

or the public directory (for private mode LAN systems).

* The file may contain:

o A "=Prompt Text" line to specify the your prompt.

o A second line with just "=" if you wish to prevent the user from

sending a fax without selecting a customer number.

o Additional lines which specify the values which will be displayed

in a pull-down list for user selection. Each value can be up to

40 alphanumeric characters.

* For example:

o If the file exists but is empty, the user gets a prompt box but

no pull-down list, and the user can enter a value or leave it

blank.

o If the file contains only "=Enter Customer Number", the user gets

a retitled prompt box but no pull-down list, and the user can

enter a value or leave it blank.

o If the file contains:

=Enter Customer Number

Cust 1, Matter 1

Cust 1, Matter 2

Cust 2

Cust 3

then the user gets a retitled prompt box and a pull-down list,

and the user can select a value from the list, enter a different

value, or leave it blank.

o If the file contains:

=Enter Customer Number

=

Cust 1, Matter 1

Cust 1, Matter 2

Cust 2

Cust 3

then the user gets a retitled prompt box and a pull-down list,

and the user MUST select a value from the list in order to send a

fax.

"HOT CALL" RECEIVING - There's now an easier way to receive a "current

call" fax (when you've answered a call and then realize that someone is

sending you a fax), and even a way for a data program to pass "hot" calls

to our program. ("Hot" means that the COM port is already open and the

modem has already picked up the call, so we need to receive on the attached

port without resetting the modem or dropping the call.) Unlike the previous

way of doing current call receiving with program commands or the FxRcv

utility program, it doesn't even matter whether or not our program is

running! You tell us what to do by using the new "-n" parameter followed by

an appropriate value. The value is a number composed of the following bits.

For example: -n0 does a current call receive, then displays the fax, and

-n7,10 does a hot receive on open COM handle 10, then exits.

bit 0

0 means reset the modem and take it offhook

1 means assume modem is already connected

bit 1

0 means we open our configured port

1 means you are providing an open COM handle

bit 2

0 means we will stay running & display the fax

1 means we will log the received fax and exit

COLOR DRAG/DROP IN LOG - You can drag/drop colors from the OS/2 Color

Palettes to the workspace background and the log display using OS/2

drag/drop standards (drop to change background color, Ctrl-drop to change

foreground color). All the different status types (sent, received, error,

edit) in the log are independently changable by using Ctrl-drop on the

different status line types. As before, you can drag/drop fonts from the

OS/2 Font Palette, too.

HOLD MODE - The program now includes Fax/Hold commands, a -H startup

parameter, and FxRcv utility program parameters for "hold mode". When in

hold mode, no new send or receive jobs will be started, but current

activity will complete gracefully. This is useful for laptop users when no

phone line is available or for shutting down LAN/multiline systems.

T.30/T.33 SUBADDRESS ROUTING - We now support IS-141 T.30 / T.33 Subaddress

Routing with Class 1 fax modem and Brooktrout cards. To use outbound

Subaddress Routing, simply put a ' (single quote) character after the

regular fax number followed by the route digits. Inbound, the route digits

are treated just like digits from DTMF or DID for automatic LAN routing.

NEW FOR API USERS - FxKill function for killing fax jobs which are sending

or spooled Priority Sending and "priority" field in FxSend's FXINFO

structure. Also the ">>AT" command in the enhanced printer driver has been

extended to include a priority argument. The format of the command is now

">>AT=time,date,priority". For example ">>AT=,,1" sets the priority flag.

NEW DRAWING TOOLS - Added to the edit menu are five new drawing tools!

Line, arrow, box, ellipse, and check have been added for better mark up

capability. The draw command still does free-hand line drawing... but for

straight lines, the new line command allows dragging of a line from a first

point to a second. The arrow, box, and ellipse all work with the same

dragging style. Try it!

PHONEBOOK AND LOG SEARCH - A new field has been added to the phonebook and

log screens to allow an incremental search. The arrow keys end can be used

to find the next and previous entries. Home and end keys will go to the

first and last matches.

AUTOMATIC BUSY CALL BLOCKING (MULTILINE SYSTEMS) - If sending with multiple

lines to the same fax number (such as to several people in a company), the

system automatically determines how many fax lines are available at the

recipient's site and the additional spooled jobs to that number are done

sequentially after the currently connected jobs complete. This avoids

unnecessary BUSY attempts, yet continues to send jobs in parallel (and

thereby get them delivered much sooner) if the recipient has multiple fax

rollover lines available.

ADDITIONAL LPT PORTS - If all your normal LPT ports (LPT1, LPT2 and LPT3)

are in use, now you can create additional LPT ports for use by the fax

printer driver. The LPT49 utility will create LPT4 through LPT9 on your

system. You can install the fax printer object on any of these LPT ports.

You can also create multiple fax printer objects on different LPT ports and

each printer object can have different job properties (orientation,

resolution, page length, emulation, etc.).

EMBEDDED PRINTER COMMAND IN DOCUMENTS - Printer driver commands can now be

placed directly in various word processor documents that are being printed

to our LPT port using PCL or Proprinter emulation (such as from DOS or

Windows programs). For example, if

>>TO=Mark,Keller Group,555-5555

is in a WordPerfect document and sent to the printer using a printer font

(so that the characters are visible to the printer driver), the fax printer

driver will automatically send the document. The command won't appear in

the fax that is sent.

FAXCHART UTILITY - Available free via download, the FaxChart utility

program can analyze your fax usage based on "lines used", "number of calls"

or "minutes of line usage" for various date ranges and time periods. It

reads your usage data from either the fax log or activity report file, then

charts the results graphically as line, area or bar charts (with or without

3D). FaxChart helps you study your usage patterns and determine when to add

additional fax lines.

LAN - MULTIPLE FAX SERVER FACILITY - If you need extra fax capacity in your

fax server or fax broadcast system or want to spread your fax lines across

multiple processors, now it's easy with this optional feature! The Multiple

Fax Server Facility applies multiple OS/2 machines to the task of fax

sending and/or receiving (i.e., multiple fax servers). The multiple fax

servers are transparent to users of the LAN or broadcast system, and can be

used with all other features (multiline options, e-mail options, DID

routing, Multi-LAN Group Facility, etc.). Each fax server is independently

started or stopped, and can even use a completely different type of fax

hardware. Now the number of fax lines in your server or broadcast platform

is virtually unlimited!

LAN - FAX STOP FROM WORKSTATION - The workstation software, the redirector,

and the server program have been changed to allow a workstartion's

private-mode fax in Spool or Send status to be selected from the log and be

stopped. Just right click on the log entry and select 'Stop'.

LAN - CREATING SHARED PHONE BOOKS FROM WORKSTATION - New in build 232, the

phone book list now indicates which phone books are "public phone books"

(i.e., shared with other LAN users) by placing a " (p)" after the phone

book name in the list. LAN workstation users can also create a new public

phone book by including the " (p)" suffix on the phone book name. Phone

books which are created without this suffix are private phone books for the

LAN workstation user.

LAN - CUSTOMIZING NOTIFICATION PROCESSING - The FxRdr (LAN redirector)

program now accepts command line parameters to override the default

notification processing. You can control the level of notification

separately for command notification (-nc#), e-mail notification (-ne#) and

print notification (-np#) where # is 0 to notify only for receive or bad

receive events, 1 to also notify on bad send events, and 2 to also notify

on successful send events.

LAN - DYNAMIC ADMINISTRATION AND NEW INTERFACE - New in build 232f, the

FxRdr (LAN redirector) program now reads the workstation configuration file

(WS.INI) whenever it is modified and automatically creates the private fax

directories for any new users that are found in the file. This allows you

to use your own programs or REXX scripts to update the fax users from your

LAN user information without needing to restart the FxRdr program. Or

better yet, you can use the new Lan User Administration program (free via

BBS download) to administer the fax users from any OS/2 workstation (not

just on the fax server machine). The Lan User Adminstration program

displays the workstation users in an OS/2 container with multiple views

(icon, detail, flowed, etc.) and lets you add, replicate and delete users

with object selection and pop-up menus.

LAN - NEW EMAIL GATEWAY OPTIONS - New options were added to FxVIM to

control the file type used on received e-mailed faxes. FxVIM will by

default attach standard PCX image pages in ccMail and attach a Tiff class-F

file in Notes. This behaviour can be overridden with new command line

options: -PCX will force multiple PCX page attachment, -DCX will force DCX

file attachment, -TIF will force Tiff class-F file attachment, and -NOFAX

will turn off all file attachment.

ACCELERATOR KEYS AND KEYBOARD ENABLING FEATURES - New in build 223g and

later, we now provide keyboard and tab improvements, including improved tab

movement in the Settings notebook, use of the Del key for deleting

highlighted items from the "To list" or phone book or log window, support

for using Shift-F10 to get pop-up menu in the phone book or log window or

status window, and accelerator keys on the Send Fax screen so you can open

the phone book with a single keystroke. In 236c and later, you can

double-click on a item in the "To list" to view/edit/delete the item.

CALLER ID SUPPORT - If you have Caller ID services from your telephone

company and you are using a modem which supports your telephone company's

Caller ID services, you can tell your modem to capture the Caller ID

information and report it to the software. To use Caller ID: 1) Sign up for

caller id service from your telephone company. 2) You must be in "Private"

mode (on Modem page of Settings) so that we have the port open and ready at

the first ring. 3) You must set "Answer Rings" (on Modem page of Settings)

to 2 or more since Caller ID data comes between first and second ring. 4)

You must enter a "Special command" (on Modem Type page of Settings) to

enable Caller ID in your modem. This command is #CID=1 for Rockwell

chipsets, +VCID=1 for Cirrus chipsets, or S40.2=1 for Zyxel. Caller ID

information is placed in the Name field in the log record.

INTERNET E-MAIL SUPPORT - In the Pro and LAN products, the software can

send and receive fax documents via e-mail by cooperating with various

Internet E-mail products. When our fax software and cooperating e-mail

products are used for sending and retrieving the e-mail, the fax document

is transparently delivered to the receiver and appears in their fax log as

a received document.

* SEND FAXES BY E-MAIL - If you specify an Internet e-mail address in

place of a fax telephone number, the software will call the

cooperating e-mail software to "send" the fax. The e-mail software

will encode the fax document and send it to the intended recipient as

an e-mail message. You can use an Internet e-mail address in place of

a fax telephone number in phone book entries or anywhere you would

normally enter a fax telephone number.

* RECEIVE FAXES (AND VOICE MESSAGES) BY E-MAIL - When your cooperating

Internet e-mail software retrieves an e-mail message, it can place it

in the fax software's log as a received document. The retrieved

document looks like a fax document (or voice message) which was

received over the phone line. The cooperating e-mail software will

decode the contents of the e-mail message and call the FxRcv.exe

utility program (FxRcv -rcvd <file> <from> <subject>) to have the fax

software "receive" the document.

* Voice messages can also be received via e-mail, and automatic

forwarding of received fax documents and voice messages via e-mail is

useful. Some of the Answer scripts (set on the Voice page of the

Settings notebook) will forward your fax and/or voice messages to your

Internet e-mail mailbox. When you are travelling, you can access your

messages and faxes by retrieving your e-mail messages, and your faxes

and messages will appear as "received" items in your fax software so

you can view and play them just as you would back at your office.

* To enable sending via e-mail, you must enter the appropriate E-Mail

command for your e-mail software on the Program page of the Settings

notebook. This command is called to submit documents to your Internet

e-mail software.

* The E-Mail command line should include the tokens %FILE%, %ADDRESS%

and %SUBJECT%. The software will substitute the appropriate items for

these tokens when executing the command line. For example, if you are

using Post Road Mailer version 2.0, the E-Mail command may be:

c:\dir\prmfax %FILE% %ADDRESS% %SUBJECT%

* Version 3 can now send/receive fax documents and voice, text and data

items via e-mail with various Internet e-mail products including Post

Road Mailer and PMMail. The new -Z start-up parameter makes it even

easier to forward items via e-mail or fax. The new "Relay" support

also makes it easy to set up Internet "forwarding nodes" and "Internet

Relay Systems" for sending faxes through the Internet and avoiding

long-distance telephone charges.

DISTINCTIVE RING SUPPORT (build 3.01.00) - If you have distinctive ring

service from your telephone company and you are using a modem which

supports it, you can tell your modem to answer only on specific ring

patterns. Also, for those creating their own answer scripts, a new Rexx

call: FxLnLine( 'DRING' ) will return the ring type. To use distinctive

ring:

1. Sign up for distinctive ring service from your telephone company.

2. You must enter a "Special command" (on Modem Type page of Settings) to

enable distinctive ring in your modem. This command is -SDR=n for

Rockwell chipsets (where n has bits set for the type of ring to

answer: bit 1 = ring type 1, bit 2 = ring type 2, bit 4 = ring type

3). So -SDR=3 will answer on both ring type 1 and 2. Please check your

modem commands manual for more details.

<end of readme>