SBU BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
Hillsboro, OR  97214-6497

DATE:	11/95

SUBJECT:	Release Notes for AMI System Configuration Utility 3.21 and Overlay 1.01

## About This Release:

1.	This is the release of Overlay 1.01 and the AMI System Configuration Utility  3.21 for use on the following platforms:

(ALTServer) -- int31A0

2.	NOTE:	AMI System Configuration Utility 3.21 and Overlay 1.01 should only be used with the following BIOS versions (or more recent BIOS versions):

Platform	       First compatible BIOS        Latest Compatible Release

(ALTServer)	Release 1.00.12			R1.00.15

AMISCU 3.11 and OVL 00.39 should be used for BIOS 1.00.11.BI0 version.

AMISCU 3.09 and OVL 00.35 should be used for BIOS 1.00.07.BI0 version.

## Issues Resolved in this Release:

- When changing the status of the 1M Hole on an ALTServer  system, the SCU will only need to be saved once (previously required two saves.)  DCS #2318

- If all A/D Channels are set to disable and saved, the SCU will correctly save the settings and will reload settings correctly on the next run (previously would reset to defaults.) DCS #2219

- The Help provided for the memory caching options on the ALTServer platform is correct (previously did not include “Disable” and erroneously included “Write-Through.”)

- CMOS bit definitions for location and disposition of Console Redirection is correct (previously, bit settings were incorrect.)  DCS #2125 & DCS #2280

- OVLs supporting multiple add-in cards are handled correctly (previously would cause .CFG error message and undetermined SCU operation.)  DCS #2273

- Help text was added to the A/D Channel Switch Menu and the ISA ROM Memory Shadow Options Menu to indicate ‘SPACE’ would toggle the selection.  DCS #2357

- Help for conversion between Kelvin and Celsius was corrected (‘add’ changed to ‘subtract’.) DCS #2210

- Text for ‘Lock Toggle” button changed to ‘ISA/EISA Lock Toggle’ (DCS #2226)

## Features Added or Changed in this Release:

- This is the last release of the AMI SCU that will support Overlay Lite, the issues forcing its use have been resolved.  Future releases of the AMI SCU will not include or necessarily be compatible with Overlay Lite.

- Additional help is provided for the 1M ISA hole at 15M.

## Outstanding issues:

- The SCSI BIOS version information is not available in the System Information Log. (DCS #2183)

- PCI video devices are not special cased resulting in a blank screen DCS #2198.  If an EISA or ISA non-video device requests memory at C0000 it will receive this resource and a PCI video device will be relocated because it is scanned by the AMISCU last.  Video should always be given precedence for this memory location.

- IRQ 12 is always assigned to the Mouse and can not be freed for other resources.

- If himem.sys does not load, viewing help when editing functions in an adapter card or baseboard may result in an “Out of Memory” error.

- On some versions of the BIOS, systems with 16M utilizing the 1M Hole display an incorrect amount of extended memory.

- The AMI SCU must be run with DOS loaded high.  Failure to load DOS high will result in an error message indicating an “XMS paging fault” has occurred and the configuration will not be saved correctly.  If himem.sys is not loaded (thus preventing DOS from loading high) this problem will not occur but other problems relating to low memory and higher wait times will appear.

- BIOS serial redirection does not support all keys used by the SCU.  Some keys (F1-F12, arrows) may need to be programmed into the terminal emulation package being used by the user.  Some keys (ctrl-char, backtab, alt-char, PgUp, PgDn, insert) are not supported at all and alternate keys must be used (i.e. multiple down arrows or tabs in place of PgDn, ‘wrapping’ through the choices instead of using backtab or PgUp, using tab to highlight a choice instead of using the control character hotkey.)

Overlay Lite User’s Guide

AMI System Configuration Utility 3.21, Overlay 01.01 and “Overlay Lite”

Note: The following instructions shall also apply to the “Overlay Lite” utility supplied with subsequent versions of the SCU and OVL.

The AMI SCU operates in Real Mode and is limited to using the 640KB memory space associated with DOS.  In some configurations, users of the PCI/EISA server systems may experience “Out of Memory” errors when attempting to configure certain combinations of adapter cards.

In the unlikely event of this error being seen , “Overlay Lite” has been developed.  By implementing a paging scheme in the AMI SCU 3.08 overlay 00.31, the available amount of memory for add-in card OVL/CFG files dramatically doubled.  However, even with 115K-130KB available for the add-in card’s OVL/CFG files it is mathematically possible to produce system configurations to exceed this limit. The “Overlay Lite” program frees approximately 130KB of additional memory for use by ISA/EISA/PCI adapter card OVL/CFG files.  This additional memory is useable by the elimination of the very large server baseboard overlay file.  The consequences of eliminating the baseboard overlay is the effective “locking” of the baseboard’s resources.  Locking the baseboard’s resources does not produce any immediate problems, but it does prevent the user from changing any of the choices provided by the standard AMISCU overlay.  The use of “Overlay Lite” can reduce memory requirements for system configuration to an extent that virtually no EISA/ISA/PCI configuration will be affected by the 640KB memory limitation.

The AMI System Configuration Utility  3.21 Overlay 1.01should be used on the following platforms:

ALTServer -- int31A0

AMI System Configuration Utility Overlay 01.01 should only be used with the following BIOS versions (or more recent BIOS versions):

Platform				Compatible BIOS*

ALTServer				1.00.04 BI0, 1.00.07 BI0, 1.00.11.BI0,

1.00.15.BI0

*AMISCU 3.07 and OVL 00.26 should be used for earlier BIOS versions.

This document provides easy-to-perform instructions, functional considerations, and some special features for the use with “Overlay Lite.”  The special features related to the AMISCU 3.21 Overlay 1.01are implemented to assist sophisticated users to monitor available memory and determine when “Overlay Lite” should be used.

Main Menu for SCU 3.21, Overlay 1.01

Perform the following steps to obtain a working version of Overlay Lite:

- Format and ‘sys’ a new diskette with DOS 6.22.

- Copy the contents of the provided System Configuration Diskette onto the newly created bootable diskette.

- Boot the server system using the diskette.

- When the system boots, the user will be presented a main menu with a number of choices:

1. Execute AMISCU

2. Execute AMISCU (baseboard configuration only)

3. Execute AMISCU using the Overlay Lite

4. Execute SCSISelect (TM) Utility  (int31A0/ALTServer baseboard only)

The user has ten seconds to make a selection or menu item (1) will be automatically selected.

In order to help our customers determine when memory problems might occur, the AMISCU 3.09 includes two new features:

- Pressing <CTRL-M> at the main menu will display the largest free memory block (this value is effectively the amount of free memory in the system.)

- Pressing <CTRL-F> at the main menu will toggle-on (or toggle-off) a dynamic memory meter that appears in the upper left-hand corner of the screen.  This “gas gauge” meter will be erased at every screen update by the AMISCU and is sometimes covered up.  Asking for “Help (F1)” will usually provide a menu that displays this meter without being “covered up” right away.

- The <CTRL-M>  and <CTRL-F> features are only available at the main menu.

Execute AMISCU Selection

The majority of system configurations should work without incident.  By selecting this option, the AMISCU will automatically locate all EISA/PCI devices and configure them into the system per the Plug-N-Play and PCI specifications. If this is the first time the system has been configured after shipment or after a “clear NVRAM” the user should choose “Execute AMISCU.”  The vast majority of OEMs are not expected to ever have a problem during the configuration of ISA/EISA/PCI add-in cards when executing this selection.  However, if “Out of Memory” errors occur, the user should reboot using the System Configuration Utility and choose either “Execute AMISCU (baseboard configuration only)” if this is the first time the system has been configured since shipment or a “clear NVRAM” or “Execute AMISCU Using the Overlay Lite” if the system has been successfully configured since the last “clear NVRAM” or initial receipt of the system.

Execute AMISCU (baseboard configuration only) Selection

The user should configure the baseboard as desired, save the configuration, exit the AMISCU, and reboot the system using the AMISCU diskette and choose Execute AMISCU Using the Overlay Lite.  Note: choosing “Execute AMISCU (baseboard configuration only)” will result in the configuration of the baseboard only; the previous settings of any adapter cards will be lost.

Execute AMISCU Using the Overlay Lite Selection

If configuring the system through the “Execute AMISCU” selection is unsuccessful (i.e. “Out of Memory” errors), the user may with to execute “Overlay Lite”.  This option locks the system baseboard configuration and increases the available  memory by  eliminating the very large server baseboard overlay file.  Locking the baseboard’s resources does not produce any immediate problems, but it does prevent the user from changing any of the choices provided by the standard AMISCU baseboard overlay.  Any  future system configuration changes/additions/deletions must be performed using this selection.  The total time to configure the system will be approximately 5 minutes in some cases.  Note: “Execute AMISCU Using the Overlay Lite” will only provide a reliable configuration if either “Execute AMISCU” or “Execute AMISCU (baseboard configuration only)” has been  run successfully since the last “clear NVRAM” or initial receipt of the system.

Execute SCSISelect(TM) Utility Selection

This option is only for use on the ALTServer system platform.  This option was installed to select the onboard AIC-7870 fast/wide SCSI controller.  The SCSISelect utility enables and configures the AIC-7870 SCSI controller on the ALTServer platform.  By selecting this option on either Xtended Xpress system platforms, this action will either lock up the system or cause unpredictable results.  The SCSISelect configuration menu can be enabled on either Xtended Xpress system platforms by pressing <CTRL-A> during system initialization.