SSPD BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
Hillsboro, OR  97214-6497

DATE:	4/2/96

NOTICE:	These release notes are intended for Intel Internal Use Only.

SUBJECT:	Release Notes for AMI System Configuration Utility 3.40 and Overlay 1.30

## About This Release:

1.	This is the release of Overlay 1.30 and the AMI System Configuration Utility  3.40 for use on the following platforms:

Altair (Altair) -- int31a0 (P/N: 696994-001)

2.	NOTE:	AMI System Configuration Utility 3.40 and Overlay 1.30 should only be used with the following BIOS versions:

Platform	       First Compatible BIOS        Latest Compatible Release

Altair (Altair)		R1.00.18			R1.00.18

## Issues Resolved in this Release:

- Switch and jumper settings are displayed correctly after saving, when user modifies interrupt settings of PCI devices sharing interrupts.  (DCS #3309)

- PCI devices using new class codes defined in the PCI Specification V2.1 are no longer displayed as “undefined adapter”.  (DCS #2568)

- SCU no longer reports missing !PNPFFFF.CFG when reinvoked after running Windows ‘95.  (DCS #2145)

- Pressing <ESC> when editing resources for a PCI device resets the current field back to its original setting.  (DCS #2492)

## Features Added or Changed in this Release:

- User can specify name of .cms, .inf, and .set files created during a save operation.  (DCS #2489)

- When invoking the AMI SCU, a message indicating that user should manually verify configuration settings before saving if adapters are added to, removed from, or moved within the system has been removed.

- OVL code now detects BIOS version information and will warn user if  OVL is known to be incompatible with BIOS.

- Startup menu provided in autoexec.bat no longer includes references to OVL Lite.

- Additional support added to configure EISA adapters on system running Windows ‘95.

- Support for multifunction PCI devices enhanced.

- User no longer prompted for a configuration file when not necessary for device type.

- Informational message provided when multiple PCI devices must be configured to the same interrupt.

- All system processors now referred as Pentium ® or Pentium ® Pro.

- Several minor spelling mistakes updated.

- If a restore from CMS file is detected, the user will be prompted to restore the baseboard settings also.

- Added the word “BIOS” to the ‘Language Options’ menu on the baseboard OVL.

## Functional Considerations:

- The SCSI BIOS version information is not available in the System Information Log (record 12h). (DCS #2183)

- The AMI SCU requires exclusive access to ISA PnP adapters during configuration.  Exclusive access can be insured by booting the system with the autoexec.bat and config.sys file included on the distribution diskette.

- Running a PnP OS with either a PCI on  an ISA PnP video adapter will cause the AMI SCU to report that no video resources have been reserved.  This message is expected since the resources for the video device is managed by the PnP OS.  Help text is available for more information.

- IRQ 12 is always assigned to the Mouse and can not be freed for other resources.

- The AMI SCU may not used with extended memory managers such as “EMM386.”

- If himem.sys does not load, viewing help when editing functions in an adapter card or baseboard may result in an “Out of Memory” error.  If less than 600KB of conventional memory is available to the AMI SCU, viewing help when editing functions in an adapter card or baseboard may result in an “Out of Memory” error.

In order to help track memory usage, the AMI SCU includes two memory features:

- Pressing <CTRL-M> at the main menu will display the largest free memory block (this value is effectively the amount of free memory in the system.)

- Pressing <CTRL-F> at the main menu will toggle-on (or toggle-off) a dynamic memory meter that appears in the upper left-hand corner of the screen.  This “gas gauge” meter will be erased at every screen update by the AMI SCU and is sometimes covered up.  Asking for “Help (F1)” will usually provide a menu that displays this meter without being “covered up” right away.

- The <CTRL-M>  and <CTRL-F> features are only available at the main menu.

AMI SCU Usage

Main Boot Menu for AMI SCU

Perform the following steps to obtain a working version of the AMI SCU:

- Format and ‘sys’ a new diskette with MS-DOS.

- Copy the contents of the provided System Configuration Diskette onto the newly created bootable diskette.

- Copy the file “himem.sys” from the “DOS” directory onto the bootable diskette.

- Boot the server system using the diskette.

- When the system boots, the user will be presented a main menu with a number of choices:

1. Execute AMI SCU

2. Execute AMI SCU For System With PnP OS

3. Execute SCSISelect (TM) Utility (int31A0/ALTServer baseboard only)

The user must choose one of the options presented.

Execute AMI SCU Selection

By selecting this option, the AMI SCU will automatically locate all EISA/ISA PnP/PCI devices and configure them into the system per the Plug-N-Play and PCI specifications.

Execute AMI SCU For System With PnP OS Selection

This option should be selected when a PnP OS is used on the server system.  In this mode, the user will be able to configure all devices except for ISA PnP and PCI devices whose resources are managed by the PnP OS.

Execute SCSISelect(TM) Utility Selection

This option is only for use on the ALTServer system platform.  This option was installed to select the onboard AIC-7870 fast/wide SCSI controller.  The SCSISelect utility enables and configures the AIC-7870 SCSI controller on the ALTServer platform.  The SCSISelect configuration menu can be enabled on either Xtended Xpress platforms or the Alder platform by pressing <CTRL-A> during system initialization.